Title 14--Aeronautics and Space

CHAPTER I--FEDERAL AVIATION ADMINISTRATION, DEPARTMENT OF TRANSPORTATION

SUBCHAPTER G--AIR CARRIERS, AIR TRAVEL CLUBS, AND OPERATORS FOR

COMPENSATION OR HIRE: CERTIFICATION AND OPERATIONS

PART 125--CERTIFICATION AND OPERATIONS: AIRPLANES HAVING A SEATING

CAPACITY OF 20 OR MORE PASSENGERS OR A MAXIMUM PAYLOAD CAPACITY OF

6,000 POUNDS OR MORE

Special Federal Aviation Regulation No. 38-2

Subpart A--General

Sec. 125.1 Applicability.

Sec. 125.3 Deviation authority.

Sec. 125.5 Operating certificate and operations specifications

required.

Sec. 125.7 Display of certificate.

Sec. 125.9 Definitions.

Sec. 125.11 Certificate eligibility and prohibited operations.

Subpart B--Certification Rules and Miscellaneous Requirements

Sec. 125.21 Application for operating certificate.

Sec. 125.23 Rules applicable to operations subject to this

part.

Sec. 125.25 Management personnel required.

Sec. 125.27 Issue of certificate.

Sec. 125.29 Duration of certificate.

Sec. 125.31 Contents of certificate and operations

specifications.

Sec. 125.33 Operations specifications not a part of

certificate.

Sec. 125.35 Amendment of operations specifications.

Sec. 125.37 Duty time limitations.

Sec. 125.39 Carriage of narcotic drugs, marihuana, and

depressant or stimulant drugs or substances.

Sec. 125.41 Availability of certificate and operations

specifications.

Sec. 125.43 Use of operations specifications.

Sec. 125.45 Inspection authority.

Sec. 125.47 Change of address.

Sec. 125.49 Airport requirements.

Sec. 125.51 En route navigational facilities.

Sec. 125.53 Flight locating requirements.

Subpart C--Manual Requirements

Sec. 125.71 Preparation.

Sec. 125.73 Contents.

Sec. 125.75 Airplane flight manual.

Subpart D--Airplane Requirements

Sec. 125.91 Airplane requirements: General.

Sec. 125.93 Airplane limitations.

Subpart E--Special Airworthiness Requirements

Sec. 125.111 General.

Sec. 125.113 Cabin interiors.

Sec. 125.115 Internal doors.

Sec. 125.117 Ventilation.

Sec. 125.119 Fire precautions.

Sec. 125.121 Proof of compliance with Sec. 125.119.

Sec. 125.123 Propeller deicing fluid.

Sec. 125.125 Pressure cross-feed arrangements.

Sec. 125.127 Location of fuel tanks.

Sec. 125.129 Fuel system lines and fittings.

Sec. 125.131 Fuel lines and fittings in designated fire zones.

Sec. 125.133 Fuel valves.

Sec. 125.135 Oil lines and fittings in designated fire zones.

Sec. 125.137 Oil valves.

Sec. 125.139 Oil system drains.

Sec. 125.141 Engine breather lines.

Sec. 125.143 Firewalls.

Sec. 125.145 Firewall construction.

Sec. 125.147 Cowling.

Sec. 125.149 Engine accessory section diaphragm.

Sec. 125.151 Powerplant fire protection.

Sec. 125.153 Flammable fluids.

Sec. 125.155 Shutoff means.

Sec. 125.157 Lines and fittings.

Sec. 125.159 Vent and drain lines.

Sec. 125.161 Fire-extinguishing systems.

Sec. 125.163 Fire-extinguishing agents.

Sec. 125.165 Extinguishing agent container pressure relief.

Sec. 125.167 Extinguishing agent container compartment

temperature.

Sec. 125.169 Fire-extinguishing system materials.

Sec. 125.171 Fire-detector systems.

Sec. 125.173 Fire detectors.

Sec. 125.175 Protection of other airplane components against

fire.

Sec. 125.177 Control of engine rotation.

Sec. 125.179 Fuel system independence.

Sec. 125.181 Induction system ice prevention.

Sec. 125.183 Carriage of cargo in passenger compartments.

Sec. 125.185 Carriage of cargo in cargo compartments.

Sec. 125.187 Landing gear: Aural warning device.

Sec. 125.189 Demonstration of emergency evacuation procedures.

Subpart F--Instrument and Equipment Requirements

Sec. 125.201 Inoperable instruments and equipment.

Sec. 125.203 Radio and navigational equipment.

Sec. 125.205 Equipment requirements: Airplanes under IFR.

Sec. 125.206 Pitot heat indication systems.

Sec. 125.207 Emergency equipment requirements.

Sec. 125.209 Emergency equipment: Extended overwater

operations.

Sec. 125.211 Seat and safety belts.

Sec. 125.213 Miscellaneous equipment.

Sec. 125.215 Operating information required.

Sec. 125.217 Passenger information.

Sec. 125.219 Oxygen for medical use by passengers.

Sec. 125.221 Icing conditions: Operating limitations.

Sec. 125.223 Airborne weather radar equipment requirements.

Sec. 125.224 Traffic Alert and Collision Avoidance System.

Sec. 125.225 Flight recorders.

Sec. 125.227 Cockpit voice recorders.

Subpart G--Maintenance

Sec. 125.241 Applicability.

Sec. 125.243 Certificate holder's responsibilities.

Sec. 125.245 Organization required to perform maintenance,

preventive maintenance, and alteration.

Sec. 125.247 Inspection programs and maintenance.

Sec. 125.249 Maintenance manual requirements.

Sec. 125.251 Required inspection personnel.

Subpart H--Airman and Crewmember Requirements

Sec. 125.261 Airman: Limitations on use of services.

Sec. 125.263 Composition of flightcrew.

Sec. 125.265 Flight engineer requirements.

Sec. 125.267 Flight navigator and long-range navigation

equipment.

Sec. 125.269 Flight attendants.

Sec. 125.271 Emergency and emergency evacuation duties.

Subpart I--Flight Crewmember Requirements

Sec. 125.281 Pilot-in-command qualifications.

Sec. 125.283 Second-in-command qualifications.

Sec. 125.285 Pilot qualifications: Recent experience.

Sec. 125.287 Initial and recurrent pilot testing requirements.

Sec. 125.289 Initial and recurrent flight attendant crewmember

testing requirements.

Sec. 125.291 Pilot in command: Instrument proficiency check

requirements.

Sec. 125.293 Crewmember: Tests and checks, grace provisions,

accepted standards.

Sec. 125.295 Check airman authorization: Application and issue.

Sec. 125.297 Approval of airplane simulators and other training

devices.

Subpart J--Flight Operations

Sec. 125.311 Flight crewmembers at controls.

Sec. 125.313 Manipulation of controls when carrying passengers.

Sec. 125.315 Admission to flight deck.

Sec. 125.317 Inspector's credentials: Admission to pilots'

compartment: Forward observer's seat.

Sec. 125.319 Emergencies.

Sec. 125.321 Reporting potentially hazardous meteorological

conditions and irregularities of ground and navigation

facilities.

Sec. 125.323 Reporting mechanical irregularities.

Sec. 125.325 Instrument approach procedures and IFR landing

minimums.

Sec. 125.327 Briefing of passengers before flight.

Sec. 125.329 Minimum altitudes for use of autopilot.

Sec. 125.331 Carriage of persons without compliance with the

passenger- carrying provisions of this part.

Sec. 125.333 Stowage of food, beverage, and passenger service

equipment during airplane movement on the surface, takeoff,

and landing.

Subpart K--Flight Release Rules

Sec. 125.351 Flight release authority.

Sec. 125.353 Facilities and services.

Sec. 125.355 Airplane equipment.

Sec. 125.357 Communication and navigation facilities.

Sec. 125.359 Flight release under VFR.

Sec. 125.361 Flight release under IFR or over-the-top.

Sec. 125.363 Flight release over water.

Sec. 125.365 Alternate airport for departure.

Sec. 125.367 Alternate airport for destination: IFR or over-

the-top.

Sec. 125.369 Alternate airport weather minimums.

Sec. 125.371 Continuing flight in unsafe conditions.

Sec. 125.373 Original flight release or amendment of flight

release.

Sec. 125.375 Fuel supply: Nonturbine and turbopropeller-powered

airplanes.

Sec. 125.377 Fuel supply: Turbine-engine-powered airplanes

other than turbo- propeller.

Sec. 125.379 Landing weather minimums: IFR.

Sec. 125.381 Takeoff and landing weather minimums: IFR.

Sec. 125.383 Load manifest.

Subpart L--Records and Reports

Sec. 125.401 Crewmember record.

Sec. 125.403 Flight release form.

Sec. 125.405 Disposition of load manifest, flight release, and

flight plans.

Sec. 125.407 Maintenance log: Airplanes.

Sec. 125.409 Reports of defects or unairworthy conditions.

Sec. 125.411 Airworthiness release or maintenance record entry.

Appendix A to Part 125--Additional Emergency Equipment

Appendix B to Part 125--Criteria for Demonstration of Emergency

Evacuation Procedures Under Sec. 125.189

Appendix C to Part 125--Ice Protection

Appendix D to Part 125--Airplane Flight Recorder Specification

Special Federal Aviation Regulation No. 38-2

Editorial Note: For the text of SFAR No. 38-2, see Part 121 of this

chapter.

Subpart A--General

Sec. 125.1   Applicability.

(a) Except as provided in paragraphs (b) and (c) of this section, this part

prescribes rules governing the operations of U.S.-registered civil airplanes

which have a seating configuration of 20 or more passengers, or a maximum

payload capacity of 6,000 pounds or more when common carriage is not

involved.

(b) The rules of this part do not apply to the operations of airplanes

specified in paragraph (a) of this section, when--

(1) They are required to be operated under Part 121, 129, 135, or 137 of

this chapter;

(2) They have been issued restricted, limited, or provisional airworthiness

certificates, special flight permits, or experimental certificates;

(3) They are being operated by a Part 125 certificate holder without

carrying passengers or cargo under Part 91 for training, ferrying,

positioning, or maintenance purposes;

(4) They are being operated under Part 91 by an operator certificated to

operate those airplanes under Part 121, 135, or 137 of this chapter or are

being operated by a foreign air carrier or a foreign person in common

carriage solely outside the United States under Part 91 of this chapter; or

(5) They are being operated under a deviation authority issued under Sec.

125.3 of this chapter.

(c) The rules of this part, except Sec. 125.247, do not apply to the

operation of airplanes specified in paragraph (a) when they are operated

outside the United States by a person who is not a citizen of the United

States.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-4, 47 FR

44719, Oct. 12, 1982; Amdt. 125-5, 49 FR 34816, Sept. 4, 1984; Amdt. 125-6,

51 FR 873, Jan. 8, 1986; Amdt. 125-9, 52 FR 20028, May 28, 1987]

Sec. 125.3   Deviation authority.

(a) The Administrator may, upon consideration of the circumstances of a

particular operation, issue deviation authority providing relief from

specified sections of Part 125. This deviation authority will be issued as a

Letter of Deviation Authority.

(b) A Letter of Deviation Authority may be terminated or amended at any

time by the Administrator.

(c) A request for deviation authority must be submitted to the nearest

Flight Standards District Office, not less than 60 days prior to the date of

intended operations. A request for deviation authority must contain a

complete statement of the circumstances and justification for the deviation

requested.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-13, 54 FR

39294, Sept. 25, 1989]

Sec. 125.5  Operating certificate and operations specifications required.

(a) After February 3, 1981, no person may engage in operations governed by

this part unless that person holds a certificate and operations specification

or appropriate deviation authority.

(b) Applicants who file an application before June 1, 1981 shall continue

to operate under the rules applicable to their operations on February 2, 1981

until the application for an operating certificate required by this part has

been denied or the operating certificate and operations specifications

required by this part have been issued.

(c) The rules of this part which apply to a certificate holder also apply

to any person who engages in any operation governed by this part without an

appropriate certificate and operations specifications required by this part

or a Letter of Deviation Authority issued under Sec. 125.3.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-1A, 46 FR

10903, Feb. 5, 1981]

Sec. 125.7  Display of certificate.

(a) The certificate holder must display a true copy of the certificate in

each of its aircraft.

(b) Each operator holding a Letter of Deviation Authority issued under this

part must carry a true copy in each of its airplanes.

Sec. 125.9  Definitions.

(a) For the purposes of this part, "maximum payload capacity" means:

(1) For an airplane for which a maximum zero fuel weight is prescribed in

FAA technical specifications, the maximum zero fuel weight, less empty

weight, less all justifiable airplane equipment, and less the operating load

(consisting of minimum flightcrew, foods and beverages and supplies and

equipment related to foods and beverages, but not including disposable fuel

or oil):

(2) For all other airplanes, the maximum certificated takeoff weight of an

airplane, less the empty weight, less all justifiable airplane equipment, and

less the operating load (consisting of minimum fuel load, oil, and

flightcrew). The allowance for the weight of the crew, oil, and fuel is as

follows:

(i) Crew--200 pounds for each crewmember required under this chapter

(ii) Oil--350 pounds.

(iii) Fuel--the minimum weight of fuel required under this chapter for a

flight between domestic points 174 nautical miles apart under VFR weather

conditions that does not involve extended overwater operations.

(b) For the purposes of this part, "empty weight" means the weight of the

airframe, engines, propellers, and fixed equipment. Empty weight excludes the

weight of the crew and payload, but includes the weight of all fixed ballast,

unusable fuel supply, undrainable oil, total quantity of engine coolant, and

total quantity of hydraulic fluid.

(c) For the purposes of this part, "maximum zero fuel weight" means the

maximum permissible weight of an airplane with no disposable fuel or oil. The

zero fuel weight figure may be found in either the airplane type certificate

data sheet or the approved Airplane Flight Manual, or both.

(d) For the purposes of this section, "justifiable airplane equipment"

means any equipment necessary for the operation of the airplane. It does not

include equipment or ballast specifically installed, permanently or

otherwise, for the purpose of altering the empty weight of an airplane to

meet the maximum payload capacity.

Sec. 125.11  Certificate eligibility and prohibited operations.

(a) No person is eligible for a certificate or operations specifications

under this part if the person holds the appropriate operating certificate

and/or operations specifications necessary to conduct operations under Part

121, 129 or 135 of this chapter.

(b) No certificate holder may conduct any operation which results directly

or indirectly from any person's holding out to the public to furnish

transportation.

(c) No person holding operations specifications under this part may operate

or list on its operations specifications any aircraft listed on any

operations specifications or other required aircraft listing under Part 121,

129, or 135 of this chapter.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980 as amended by Amdt. 125-9, 52 FR

20028, May 28, 1987]

Subpart B--Certification Rules and Miscellaneous Requirements

Sec. 125.21  Application for operating certificate.

(a) Each applicant for the issuance of an operating certificate must submit

an application in a form and manner prescribed by the Administrator to the

FAA Flight Standards district office in whose area the applicant proposes to

establish or has established its principal operations base. The application

must be submitted at least 60 days before the date of intended operations.

(b) Each application submitted under paragraph (a) of this section must

contain a signed statement showing the following:

(1) The name and address of each director and each officer or person

employed or who will be employed in a management position described in Sec.

125.25.

(2) A list of flight crewmembers with the type of airman certificate held,

including ratings and certificate numbers.

Sec. 125.23  Rules applicable to operations subject to this part.

Each person operating an airplane in operations under this part shall--

(a) While operating inside the United States, comply with the applicable

rules in Part 91 of this chapter; and

(b) While operating outside the United States, comply with Annex 2, Rules

of the Air, to the Convention on International Civil Aviation or the

regulations of any foreign country, whichever applies, and with any rules of

Parts 61 and 91 of this chapter and this part that are more restrictive than

that Annex or those regulations and that can be complied with without

violating that Annex or those regulations. Annex 2 is incorporated by

reference in Sec. 91.703(b) of this chapter.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-12, 54 FR

34331, Aug. 18, 1989]

Effective Date Note: At 54 FR 34331, August 18, 1989, Sec. 125.23(b) was

amended by changing the cross reference "Sec. 91.1(c)" to read "Sec.

91.703(b)", effective August 18, 1990.

Sec. 125.25  Management personnel required.

(a) Each applicant for a certificate under this part must show that it has

enough management personnel, including at least a director of operations, to

assure that its operations are conducted in accordance with the requirements

of this part.

(b) Each applicant shall--

(1) Set forth the duties, responsibilities, and authority of each of its

management personnel in the general policy section of its manual;

(2) List in the manual the names and addresses of each of its management

personnel;

(3) Designate a person as responsible for the scheduling of inspections

required by the manual and for the updating of the approved weight and

balance system on all airplanes.

(c) Each certificate holder shall notify the FAA Flight Standards district

office charged with the overall inspection of the certificate holder of any

change made in the assignment of persons to the listed positions within 10

days, excluding Saturdays, Sundays, and Federal holidays, of such change.

Sec. 125.27  Issue of certificate.

(a) An applicant for a certificate under this subpart is entitled to a

certificate if the Administrator finds that the applicant is properly and

adequately equipped and able to conduct a safe operation in accordance with

the requirements of this part and the operations specifications provided for

in this part.

(b) The Administrator may deny an application for a certificate under this

subpart if the Administrator finds--

(1) That an operating certificate required under this part or Part 121,

123, or 135 of this chapter previously issued to the applicant was revoked;

or

(2) That a person who was employed in a management position under Sec.

125.25 of this part with (or has exercised control with respect to) any

certificate holder under Part 121, 123, 125, or 135 of this chapter whose

operating certificate has been revoked, will be employed in any of those

positions or a similar position with the applicant and that the person's

employment or control contributed materially to the reasons for revoking that

certificate.

Sec. 125.29   Duration of certificate.

(a) A certificate issued under this part is effective until surrendered,

suspended, or revoked.

(b) The Administrator may suspend or revoke a certificate under section 609

of the Federal Aviation Act of 1958 and the applicable procedures of Part 13

of this chapter for any cause that, at the time of suspension or revocation,

would have been grounds for denying an application for a certificate.

(c) If the Administrator suspends or revokes a certificate or it is

otherwise terminated, the holder of that certificate shall return it to the

Administrator.

Sec. 125.31   Contents of certificate and operations specifications.

(a) Each certificate issued under this part contains the following:

(1) The holder's name.

(2) A description of the operations authorized.

(3) The date it is issued.

(b) The operations specifications issued under this part contain the

following:

(1) The kinds of operations authorized.

(2) The types and registration numbers of airplanes authorized for use.

(3) Approval of the provisions of the operator's manual relating to

airplane inspections, together with necessary conditions and limitations.

(4) Registration numbers of airplanes that are to be inspected under an

approved airplane inspection program under Sec. 125.247.

(5) Procedures for control of weight and balance of airplanes.

(6) Any other item that the Administrator determines is necessary to cover

a particular situation.

Sec. 125.33   Operations specifications not a part of certificate.

Operations specifications are not a part of an operating certificate.

Sec. 125.35   Amendment of operations specifications.

(a) The FAA Flight Standards district office charged with the overall

inspection of the certificate holder may amend any operations specifications

issued under this part if--

(1) It determines that safety in air commerce requires that amendment; or

(2) Upon application by the holder, that district office determines that

safety in air commerce allows that amendment.

(b) The certificate holder must file an application to amend operations

specifications at least 15 days before the date proposed by the applicant for

the amendment to become effective, unless a shorter filing period is

approved. The application must be on a form and in a manner prescribed by the

Administrator and be submitted to the FAA Flight Standards district office

charged with the overall inspection of the certificate holder.

(c) Within 30 days after a notice of refusal to approve a holder's

application for amendment is received, the holder may petition the Director,

Flight Standards Service, to reconsider the refusal to amend.

(d) When the FAA Flight Standards district office charged with the overall

inspection of the certificate holder amends operations specifications, that

district office gives notice in writing to the holder of a proposed amendment

to the operations specifications, fixing a period of not less than 7 days

within which the holder may submit written information, views, and arguments

concerning the proposed amendment. After consideration of all relevant matter

presented, that district office notifies the holder of any amendment adopted,

or a rescission of the notice. That amendment becomes effective not less than

30 days after the holder receives notice of the adoption of the amendment,

unless the holder petitions the Director, Flight Standards Service, for

reconsideration of the amendment. In that case, the effective date of the

amendment is stayed pending a decision by the Director. If the Director finds

there is an emergency requiring immediate action as to safety in air commerce

that makes the provisions of this paragraph impracticable or contrary to the

public interest, the Director notifies the certificate holder that the

amendment is effective on the date of receipt, without previous notice.

[Doc. NO. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-13, 54 FR

39294, Sept. 25, 1989]

Sec. 125.37   Duty time limitations.

(a) Each flight crewmember must be relieved from all duty for at least 8

consecutive hours during any 24-hour period.

(b) The Administrator may specify rest, flight time, and duty time

limitations in the operations specifications that are other than those

specified in paragraph (a) of this section.

Sec. 125.39   Carriage of narcotic drugs, marihuana, and depressant or

stimulant drugs or substances.

If the holder of a certificate issued under this part permits any airplane

owned or leased by that holder to be engaged in any operation that the

certificate holder knows to be in violation of Sec. 91.19(a) of this chapter,

that operation is a basis for suspending or revoking the certificate.

[Docket No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-12, 54

FR 34331, Aug. 18, 1989]

Effective Date Note: At 54 FR 34331, August 18, 1989, Sec. 125.39 was

amended by changing the cross reference "Sec. 91.12(a)" to read "Sec.

91.19(a)", effective August 18, 1990.

Sec. 125.41   Availability of certificate and operations specifications.

Each certificate holder shall make its operating certificate and operations

specifications available for inspection by the Administrator at its principal

operations base.

Sec. 125.43   Use of operations specifications.

(a) Each certificate holder shall keep each of its employees informed of

the provisions of its operations specifications that apply to the employee's

duties and responsibilities.

(b) Each certificate holder shall maintain a complete and separate set of

its operations specifications. In addition, each certificate holder shall

insert pertinent excerpts of its operations specifications, or reference

thereto, in its manual in such a manner that they retain their identity as

operations specifications.

Sec. 125.45   Inspection authority.

Each certificate holder shall allow the Administrator, at any time or

place, to make any inspections or tests to determine its compliance with the

Federal Aviation Act of 1958, the Federal Aviation Regulations, its operating

certificate and operations specifications, its letter of deviation authority,

or its eligibililty to continue to hold its certificate or its letter of

deviation authority.

Sec. 125.47   Change of address.

Each certificate holder shall notify the FAA Flight Standards district

office charged with the overall inspection of its operations, in writing, at

least 30 days in advance, of any change in the address of its principal

business office, its principal operations base, or its principal maintenance

base.

Sec. 125.49   Airport requirements.

(a) No certificate holder may use any airport unless it is adequate for the

proposed operation, considering such items as size, surface, obstructions,

and lighting.

(b) No pilot of an airplane carrying passengers at night may take off from,

or land on, an airport unless--

(1) That pilot has determined the wind direction from an illuminated wind

direction indicator or local ground communications, or, in the case of

takeoff, that pilot's personal observations; and

(2) The limits of the area to be used for landing or takeoff are clearly

shown by boundary or runway marker lights.

(c) For the purposes of paragraph (b) of this section, if the area to be

used for takeoff or landing is marked by flare pots or lanterns, their use

must be approved by the Administrator.

Sec. 125.51  En route navigational facilities.

(a) Except as provided in paragraph (b) of this section, no certificate

holder may conduct any operation over a route unless nonvisual ground aids

are--

(1) Available over the route for navigating airplanes within the degree of

accuracy required for ATC; and

(2) Located to allow navigation to any airport of destination, or alternate

airport, within the degree of accuracy necessary for the operation involved.

(b) Nonvisual ground aids are not required for--

(1) Day VFR operations that can be conducted safely by pilotage because of

the characteristics of the terrain;

(2) Night VFR operations on routes that the Administrator determines have

reliable landmarks adequate for safe operation; or

(3) Operations where the use of celestial or other specialized means of

navigation, such as an inertial navigation system, is approved.

Sec. 125.53  Flight locating requirements.

(a) Each certificate holder must have procedures established for locating

each flight for which an FAA flight plan is not filed that--

(1) Provide the certificate holder with at least the information required

to be included in a VFR flight plan;

(2) Provide for timely notification of an FAA facility or search and rescue

facility, if an airplane is overdue or missing; and

(3) Provide the certificate holder with the location, date, and estimated

time for reestablishing radio or telephone communications, if the flight will

operate in an area where communications cannot be maintained.

(b) Flight locating information shall be retained at the certificate

holder's principal operations base, or at other places designated by the

certificate holder in the flight locating procedures, until the completion of

the flight.

(c) Each certificate holder shall furnish the representative of the

Administrator assigned to it with a copy of its flight locating procedures

and any changes or additions, unless those procedures are included in a

manual required under this part.

Subpart C--Manual Requirements

Sec. 125.71  Preparation.

(a) Each certificate holder shall prepare and keep current a manual setting

forth the certificate holder's procedures and policies acceptable to the

Administrator. This manual must be used by the certificate holder's flight,

ground, and maintenance personnel in conducting its operations. However, the

Administrator may authorize a deviation from this paragraph if the

Administrator finds that, because of the limited size of the operation, all

or part of the manual is not necessary for guidance of flight, ground, or

maintenance personnel.

(b) Each certificate holder shall maintain at least one copy of the manual

at its principal operations base.

(c) The manual must not be contrary to any applicable Federal regulations,

foreign regulation applicable to the certificate holder's operations in

foreign countries, or the certificate holder's operating certificate or

operations specifications.

(d) A copy of the manual, or appropriate portions of the manual (and

changes and additions) shall be made available to maintenance and ground

operations personnel by the certificate holder and furnished to--

(1) Its flight crewmembers; and

(2) The FAA Flight Standards district office charged with the overall

inspection of its operations.

(e) Each employee of the certificate holder to whom a manual or appropriate

portions of it are furnished under paragraph (d)(1) of this section shall

keep it up to date with the changes and additions furnished to them.

(f) Except as provided in paragraph (g) of this section, each certificate

holder shall carry appropriate parts of the manual in each airplane when away

from the principal operations base. The appropriate parts must be available

for use by ground or flight personnel. If a certificate holder carries aboard

an airplane all or any portion of the maintenance part of its manual in

microfilm, it must also carry a reading device that provides a legible

facsimile image of the microfilmed maintenance information and instructions.

(g) If a certificate holder conducts airplane inspections or maintenance at

specified stations where it keeps the approved inspection program manual, it

is not required to carry the manual aboard the airplane en route to those

stations.

Sec. 125.73  Contents.

Each manual shall have the date of the last revision and revision number on

each revised page. The manual must include--

(a) The name of each management person who is authorized to act for the

certificate holder, the person's assigned area of responsibility, and the

person's duties, responsibilities, and authority;

(b) Procedures for ensuring compliance with airplane weight and balance

limitations;

(c) Copies of the certificate holder's operations specifications or

appropriate extracted information, including area of operations authorized,

category and class of airplane authorized, crew complements, and types of

operations authorized;

(d) Procedures for complying with accident notification requirements;

(e) Procedures for ensuring that the pilot in command knows that required

airworthiness inspections have been made and that the airplane has been

approved for return to service in compliance with applicable maintenance

requirements;

(f) Procedures for reporting and recording mechanical irregularities that

come to the attention of the pilot in command before, during, and after

completion of a flight;

(g) Procedures to be followed by the pilot in command for determining that

mechanical irregularities or defects reported for previous flights have been

corrected or that correction has been deferred;

(h) Procedures to be followed by the pilot in command to obtain

maintenance, preventive maintenance, and servicing of the airplane at a place

where previous arrangements have not been made by the operator, when the

pilot is authorized to so act for the operator;

(i) Procedures for the release for, or continuation of, flight if any item

of equipment required for the particular type of operation becomes

inoperative or unserviceable en route;

(j) Procedures for refueling airplanes, eliminating fuel contamination,

protecting from fire (including electrostatic protection), and supervising

and protecting passengers during refueling;

(k) Procedures to be followed by the pilot in command in the briefing under

Sec. 125.327;

(l) Flight locating procedures, when applicable;

(m) Procedures for ensuring compliance with emergency procedures, including

a list of the functions assigned each category of required crewmembers in

connection with an emergency and emergency evacuation;

(n) The approved airplane inspection program;

(o) Procedures and instructions to enable personnel to recognize hazardous

materials, as defined in Title 49 CFR, and if these materials are to be

carried, stored, or handled, procedures and instructions for--

(1) Accepting shipment of hazardous material required by Title 49 CFR, to

assure proper packaging, marking, labeling, shipping documents, compatibility

of articles, and instructions on their loading, storage, and handling;

(2) Notification and reporting hazardous material incidents as required by

Title 49 CFR; and

(3) Notification of the pilot in command when there are hazardous materials

aboard, as required by Title 49 CFR;

(p) Procedures for the evacuation of persons who may need the assistance of

another person to move expeditiously to an exit if an emergency occurs;

(q) The identity of each person who will administer tests required by this

part, including the designation of the tests authorized to be given by the

person; and

(r) Other procedures and policy instructions regarding the certificate

holder's operations that are issued by the certificate holder.

Sec. 125.75  Airplane flight manual.

(a) Each certificate holder shall keep a current approved Airplane Flight

Manual or approved equivalent for each type airplane that it operates.

(b) Each certificate holder shall carry the approved Airplane Flight Manual

or the approved equivalent aboard each airplane it operates. A certificate

holder may elect to carry a combination of the manuals required by this

section and Sec. 125.71. If it so elects, the certificate holder may revise

the operating procedures sections and modify the presentation of performance

from the applicable Airplane Flight Manual if the revised operating

procedures and modified performance data presentation are approved by the

Administrator.

Subpart D--Airplane Requirements

Sec. 125.91  Airplane requirements: General.

(a) No certificate holder may operate an airplane governed by this part

unless it--

(1) Carries an appropriate current airworthiness certificate issued under

this chapter; and

(2) Is in an airworthy condition and meets the applicable airworthiness

requirements of this chapter, including those relating to identification and

equipment.

(b) No person may operate an airplane unless the current empty weight and

center of gravity are calculated from the values established by actual

weighing of the airplane within the preceding 36 calendar months.

(c) Paragraph (b) of this section does not apply to airplanes issued an

original airworthiness certificate within the preceding 36 calendar months.

Sec. 125.93  Airplane limitations.

No certificate holder may operate a land airplane (other than a DC-3, C-46,

CV-240, CV-340, CV-440, CV-580, CV-600, CV-640, or Martin 404) in an extended

overwater operation unless it is certificated or approved as adequate for

ditching under the ditching provisions of Part 25 of this chapter.

Subpart E--Special Airworthiness Requirements

Sec. 125.111  General.

(a) Except as provided in paragraph (b) of this section, no certificate

holder may use an airplane powered by airplane engines rated at more than 600

horsepower each for maximum continuous operation unless that airplane meets

the requirements of Secs. 125.113 through 125.181.

(b) If the Administrator determines that, for a particular model of

airplane used in cargo service, literal compliance with any requirement under

paragraph (a) of this section would be extremely difficult and that

compliance would not contribute materially to the objective sought, the

Administrator may require compliance with only those requirements that are

necessary to accomplish the basic objectives of this part.

(c) This section does not apply to any airplane certificated under--

(1) Part 4b of the Civil Air Regulations in effect after October 31, 1946;

(2) Part 25 of this chapter; or

(3) Special Civil Air Regulation 422, 422A, or 422B.

Sec. 125.113  Cabin interiors.

(a) Upon the first major overhaul of an airplane cabin or refurbishing of

the cabin interior, all materials in each compartment used by the crew or

passengers that do not meet the following requirements must be replaced with

materials that meet these requirements:

(1) For an airplane for which the application for the type certificate was

filed prior to May 1, 1972, Sec. 25.853 in effect on April 30, 1972.

(2) For an airplane for which the application for the type certificate was

filed on or after May 1, 1972, the materials requirement under which the

airplane was type certificated.

(b) Except as provided in paragraph (a) of this section, each compartment

used by the crew or passengers must meet the following requirements:

(1) Materials must be at least flash resistant.

(2) The wall and ceiling linings and the covering of upholstering, floors,

and furnishings must be flame resistant.

(3) Each compartment where smoking is to be allowed must be equipped with

self-contained ash trays that are completely removable and other compartments

must be placarded against smoking.

(4) Each receptacle for used towels, papers, and wastes must be of fire-

resistant material and must have a cover or other means of containing

possible fires started in the receptacles.

Sec. 125.115  Internal doors.

In any case where internal doors are equipped with louvres or other

ventilating means, there must be a means convenient to the crew for closing

the flow of air through the door when necessary.

Sec. 125.117  Ventilation.

Each passenger or crew compartment must be suitably ventilated. Carbon

monoxide concentration may not be more than one part in 20,000 parts of air,

and fuel fumes may not be present. In any case where partitions between

compartments have louvres or other means allowing air to flow between

compartments, there must be a means convenient to the crew for closing the

flow of air through the partitions when necessary.

Sec. 125.119  Fire precautions.

(a) Each compartment must be designed so that, when used for storing cargo

or baggage, it meets the following requirements:

(1) No compartment may include controls, wiring, lines, equipment, or

accessories that would upon damage or failure, affect the safe operation of

the airplane unless the item is adequately shielded, isolated, or otherwise

protected so that it cannot be damaged by movement of cargo in the

compartment and so that damage to or failure of the item would not create a

fire hazard in the compartment.

(2) Cargo or baggage may not interfere with the functioning of the fire-

protective features of the compartment.

(3) Materials used in the construction of the compartments, including tie-

down equipment, must be at least flame resistant.

(4) Each compartment must include provisions for safeguarding against fires

according to the classifications set forth in paragraphs (b) through (f) of

this section.

(b) Class A. Cargo and baggage compartments are classified in the "A"

category if a fire therein would be readily discernible to a member of the

crew while at that crewmember's station, and all parts of the compartment are

easily accessible in flight. There must be a hand fire extinguisher available

for each Class A compartment.

(c) Class B. Cargo and baggage compartments are classified in the "B"

category if enough access is provided while in flight to enable a member of

the crew to effectively reach all of the compartment and its contents with a

hand fire extinguisher and the compartment is so designed that, when the

access provisions are being used, no hazardous amount of smoke, flames, or

extinguishing agent enters any compartment occupied by the crew or

passengers. Each Class B compartment must comply with the following:

(1) It must have a separate approved smoke or fire detector system to give

warning at the pilot or flight engineer station.

(2) There must be a hand-held fire extinguisher available for the

compartment.

(3) It must be lined with fire-resistant material, except that additional

service lining of flame-resistant material may be used.

(d) Class C. Cargo and baggage compartments are classified in the "C"

category if they do not conform with the requirements for the "A", "B", "D",

or "E" categories. Each Class C compartment must comply with the following:

(1) It must have a separate approved smoke or fire detector system to give

warning at the pilot or flight engineer station.

(2) It must have an approved built-in fire-extinguishing system controlled

from the pilot or flight engineer station.

(3) It must be designed to exclude hazardous quantities of smoke, flames,

or extinguishing agents from entering into any compartment occupied by the

crew or passengers.

(4) It must have ventilation and draft control so that the extinguishing

agent provided can control any fire that may start in the compartment.

(5) It must be lined with fire-resistant material, except that additional

service lining of flame-resistant material may be used.

(e) Class D. Cargo and baggage compartments are classified in the "D"

category if they are so designed and constructed that a fire occurring

therein will be completely confined without endangering the safety of the

airplane or the occupants. Each Class D compartment must comply with the

following:

(1) It must have a means to exclude hazardous quantities of smoke, flames,

or noxious gases from entering any compartment occupied by the crew or

passengers.

(2) Ventilation and drafts must be controlled within each compartment so

that any fire likely to occur in the compartment will not progress beyond

safe limits.

(3) It must be completely lined with fire-resistant material.

(4) Consideration must be given to the effect of heat within the

compartment on adjacent critical parts of the airplane.

(f) Class E. On airplanes used for the carriage of cargo only, the cabin

area may be classified as a Class "E" compartment. Each Class E compartment

must comply with the following:

(1) It must be completely lined with fire-resistant material.

(2) It must have a separate system of an approved type smoke or fire

detector to give warning at the pilot or flight engineer station.

(3) It must have a means to shut off the ventilating air flow to or within

the compartment and the controls for that means must be accessible to the

flightcrew in the crew compartment.

(4) It must have a means to exclude hazardous quantities of smoke, flames,

or noxious gases from entering the flightcrew compartment.

(5) Required crew emergency exits must be accessible under all cargo

loading conditions.

Sec. 125.121   Proof of compliance with Sec. 125.119.

Compliance with those provisions of Sec. 125.119 that refer to compartment

accessibility, the entry of hazardous quantities of smoke or extinguishing

agent into compartment occupied by the crew or passengers, and the

dissipation of the extinguishing agent in Class "C" compartments must be

shown by tests in flight. During these tests it must be shown that no

inadvertent operation of smoke or fire detectors in other compartments within

the airplane would occur as a result of fire contained in any one

compartment, either during the time it is being extinguished, or thereafter,

unless the extinguishing system floods those compartments simultaneously.

Sec. 125.123   Propeller deicing fluid.

If combustible fluid is used for propeller deicing, the certificate holder

must comply with Sec. 125.153.

Sec. 125.125   Pressure cross-feed arrangements.

(a) Pressure cross-feed lines may not pass through parts of the airplane

used for carrying persons or cargo unless there is a means to allow

crewmembers to shut off the supply of fuel to these lines or the lines are

enclosed in a fuel and fume-proof enclosure that is ventilated and drained to

the exterior of the airplane. However, such an enclosure need not be used if

those lines incorporate no fittings on or within the personnel or cargo areas

and are suitably routed or protected to prevent accidental damage.

(b) Lines that can be isolated from the rest of the fuel system by valves

at each end must incorporate provisions for relieving excessive pressures

that may result from exposure of the isolated line to high temperatures.

Sec. 125.127   Location of fuel tanks.

(a) Fuel tanks must be located in accordance with Sec. 125.153.

(b) No part of the engine nacelle skin that lies immediately behind a major

air outlet from the engine compartment may be used as the wall of an integral

tank.

(c) Fuel tanks must be isolated from personnel compartments by means of

fume- and fuel-proof enclosures.

Sec. 125.129   Fuel system lines and fittings.

(a) Fuel lines must be installed and supported so as to prevent excessive

vibration and so as to be adequate to withstand loads due to fuel pressure

and accelerated flight conditions.

(b) Lines connected to components of the airplane between which there may

be relative motion must incorporate provisions for flexibility.

(c) Flexible connections in lines that may be under pressure and subject to

axial loading must use flexible hose assemblies rather than hose clamp

connections.

(d) Flexible hoses must be of an acceptable type or proven suitable for the

particular application.

Sec. 125.131   Fuel lines and fittings in designated fire zones.

Fuel lines and fittings in each designated fire zone must comply with Sec.

125.157.

Sec. 125.133   Fuel valves.

Each fuel valve must--

(a) Comply with Sec. 125.155;

(b) Have positive stops or suitable index provisions in the "on" and "off"

positions; and

(c) Be supported so that loads resulting from its operation or from

accelerated flight conditions are not transmitted to the lines connected to

the valve.

Sec. 125.135   Oil lines and fittings in designated fire zones.

Oil lines and fittings in each designated fire zone must comply with Sec.

125.157.

Sec. 125.137   Oil valves.

(a) Each oil valve must--

(1) Comply with Sec. 125.155;

(2) Have positive stops or suitable index provisions in the "on" and "off"

positions; and

(3) Be supported so that loads resulting from its operation or from

accelerated flight conditions are not transmitted to the lines attached to

the valve.

(b) The closing of an oil shutoff means must not prevent feathering the

propeller, unless equivalent safety provisions are incorporated.

Sec. 125.139   Oil system drains.

Accessible drains incorporating either a manual or automatic means for

positive locking in the closed position must be provided to allow safe

drainage of the entire oil system.

Sec. 125.141   Engine breather lines.

(a) Engine breather lines must be so arranged that condensed water vapor

that may freeze and obstruct the line cannot accumulate at any point.

(b) Engine breathers must discharge in a location that does not constitute

a fire hazard in case foaming occurs and so that oil emitted from the line

does not impinge upon the pilots' windshield.

(c) Engine breathers may not discharge into the engine air induction

system.

Sec. 125.143   Firewalls.

Each engine, auxiliary power unit, fuel-burning heater, or other item of

combusting equipment that is intended for operation in flight must be

isolated from the rest of the airplane by means of firewalls or shrouds, or

by other equivalent means.

Sec. 125.145   Firewall construction.

Each firewall and shroud must--

(a) Be so made that no hazardous quantity of air, fluids, or flame can pass

from the engine compartment to other parts of the airplane;

(b) Have all openings in the firewall or shroud sealed with close-fitting

fireproof grommets, bushings, or firewall fittings;

(c) Be made of fireproof material; and

(d) Be protected against corrosion.

Sec. 125.147   Cowling.

(a) Cowling must be made and supported so as to resist the vibration,

inertia, and air loads to which it may be normally subjected.

(b) Provisions must be made to allow rapid and complete drainage of the

cowling in normal ground and flight attitudes. Drains must not discharge in

locations constituting a fire hazard. Parts of the cowling that are subjected

to high temperatures because they are near exhaust system parts or because of

exhaust gas impingement must be made of fireproof material. Unless otherwise

specified in these regulations, all other parts of the cowling must be made

of material that is at least fire resistant.

Sec. 125.149   Engine accessory section diaphragm.

Unless equivalent protection can be shown by other means, a diaphragm that

complies with Sec. 125.145 must be provided on air-cooled engines to isolate

the engine power section and all parts of the exhaust system from the engine

accessory compartment.

Sec. 125.151   Powerplant fire protection.

(a) Designated fire zones must be protected from fire by compliance with

Secs. 125.153 through 125.159.

(b) Designated fire zones are--

(1) Engine accessory sections;

(2) Installations where no isolation is provided between the engine and

accessory compartment; and

(3) Areas that contain auxiliary power units, fuel-burning heaters, and

other combustion equipment.

Sec. 125.153   Flammable fluids.

(a) No tanks or reservoirs that are a part of a system containing flammable

fluids or gases may be located in designated fire zones, except where the

fluid contained, the design of the system, the materials used in the tank,

the shutoff means, and the connections, lines, and controls provide

equivalent safety.

(b) At least one-half inch of clear airspace must be provided between any

tank or reservior and a firewall or shroud isolating a designated fire zone.

Sec. 125.155  Shutoff means.

(a) Each engine must have a means for shutting off or otherwise preventing

hazardous amounts of fuel, oil, deicer, and other flammable fluids from

flowing into, within, or through any designated fire zone. However, means

need not be provided to shut off flow in lines that are an integral part of

an engine.

(b) The shutoff means must allow an emergency operating sequence that is

compatible with the emergency operation of other equipment, such as

feathering the propeller, to facilitate rapid and effective control of fires.

(c) Shutoff means must be located outside of designated fire zones, unless

equivalent safety is provided, and it must be shown that no hazardous amount

of flammable fluid will drain into any designated fire zone after a shutoff.

(d) Adequate provisions must be made to guard against inadvertent operation

of the shutoff means and to make it possible for the crew to reopen the

shutoff means after it has been closed.

Sec. 125.157  Lines and fittings.

(a) Each line, and its fittings, that is located in a designated fire zone,

if it carries flammable fluids or gases under pressure, or is attached

directly to the engine, or is subject to relative motion between components

(except lines and fittings forming an integral part of the engine), must be

flexible and fire-resistant with fire-resistant, factory-fixed, detachable,

or other approved fire-resistant ends.

(b) Lines and fittings that are not subject to pressure or to relative

motion between components must be of fire-resistant materials.

Sec. 125.159  Vent and drain lines.

All vent and drain lines, and their fittings, that are located in a

designated fire zone must, if they carry flammable fluids or gases, comply

with Sec. 125.157, if the Administrator finds that the rupture or breakage of

any vent or drain line may result in a fire hazard.

Sec. 125.161  Fire-extinguishing systems.

(a) Unless the certificate holder shows that equivalent protection against

destruction of the airplane in case of fire is provided by the use of

fireproof materials in the nacelle and other components that would be

subjected to flame, fire-extinguishing systems must be provided to serve all

designated fire zones.

(b) Materials in the fire-extinguishing system must not react chemically

with the extinguishing agent so as to be a hazard.

Sec. 125.163  Fire-extinguishing agents.

Only methyl bromide, carbon dioxide, or another agent that has been shown

to provide equivalent extinguishing action may be used as a fire-

extinguishing agent. If methyl bromide or any other toxic extinguishing agent

is used, provisions must be made to prevent harmful concentrations of fluid

or fluid vapors from entering any personnel compartment either because of

leakage during normal operation of the airplane or because of discharging the

fire extinguisher on the ground or in flight when there is a defect in the

extinguishing system. If a methyl bromide system is used, the containers must

be charged with dry agent and sealed by the fire-extinguisher manufacturer or

some other person using satisfactory recharging equipment. If carbon dioxide

is used, it must not be possible to discharge enough gas into the personnel

compartments to create a danger of suffocating the occupants.

Sec. 125.165  Extinguishing agent container pressure relief.

Extinguishing agent containers must be provided with a pressure relief to

prevent bursting of the container because of excessive internal pressures.

The discharge line from the relief connection must terminate outside the

airplane in a place convenient for inspection on the ground. An indicator

must be provided at the discharge end of the line to provide a visual

indication when the container has discharged.

Sec. 125.167  Extinguishing agent container compartment temperature.

Precautions must be taken to ensure that the extinguishing agent containers

are installed in places where reasonable temperatures can be maintained for

effective use of the extinguishing system.

Sec. 125.169  Fire-extinguishing system materials.

(a) Except as provided in paragraph (b) of this section, each component of

a fire-extinguishing system that is in a designated fire zone must be made of

fireproof materials.

(b) Connections that are subject to relative motion between components of

the airplane must be made of flexible materials that are at least fire-

resistant and be located so as to minimize the probability of failure.

Sec. 125.171  Fire-detector systems.

Enough quick-acting fire detectors must be provided in each designated fire

zone to assure the detection of any fire that may occur in that zone.

Sec. 125.173  Fire detectors.

Fire detectors must be made and installed in a manner that assures their

ability to resist, without failure, all vibration, inertia, and other loads

to which they may be normally subjected. Fire detectors must be unaffected by

exposure to fumes, oil, water, or other fluids that may be present.

Sec. 125.175  Protection of other airplane components against fire.

(a) Except as provided in paragraph (b) of this section, all airplane

surfaces aft of the nacelles in the area of one nacelle diameter on both

sides of the nacelle centerline must be made of material that is at least

fire resistant.

(b) Paragraph (a) of this section does not apply to tail surfaces lying

behind nacelles unless the dimensional configuration of the airplane is such

that the tail surfaces could be affected readily by heat, flames, or sparks

emanating from a designated fire zone or from the engine from a designated

fire zone or from the engine compartment of any nacelle.

Sec. 125.177  Control of engine rotation.

(a) Except as provided in paragraph (b) of this section, each airplane must

have a means of individually stopping and restarting the rotation of any

engine in flight.

(b) In the case of turbine engine installations, a means of stopping

rotation need be provided only if the Administrator finds that rotation could

jeopardize the safety of the airplane.

Sec. 125.179  Fuel system independence.

(a) Each airplane fuel system must be arranged so that the failure of any

one component does not result in the irrecoverable loss of power of more than

one engine.

(b) A separate fuel tank need not be provided for each engine if the

certificate holder shows that the fuel system incorporates features that

provide equivalent safety.

Sec. 125.181  Induction system ice prevention.

A means for preventing the malfunctioning of each engine due to ice

accumulation in the engine air induction system must be provided for each

airplane.

Sec. 125.183  Carriage of cargo in passenger compartments.

(a) Except as provided in paragraph (b) or (c) of this section, no

certificate holder may carry cargo in the passenger compartment of an

airplane.

(b) Cargo may be carried aft of the foremost seated passengers if it is

carried in an approved cargo bin that meets the following requirements:

(1) The bin must withstand the load factors and emergency landing

conditions applicable to the passenger seats of the airplane in which the bin

is installed, multiplied by a factor of 1.15, using the combined weight of

the bin and the maximum weight of cargo that may be carried in the bin.

(2) The maximum weight of cargo that the bin is approved to carry and any

instructions necessary to ensure proper weight distribution within the bin

must be conspicuously marked on the bin.

(3) The bin may not impose any load on the floor or other structure of the

airplane that exceeds the load limitations of that structure.

(4) The bin must be attached to the seat tracks or to the floor structure

of the airplane, and its attachment must withstand the load factors and

emergency landing conditions applicable to the passenger seats of the

airplane in which the bin is installed, multiplied by either the factor 1.15

or the seat attachment factor specified for the airplane, whichever is

greater, using the combined weight of the bin and the maximum weight of cargo

that may be carried in the bin.

(5) The bin may not be installed in a position that restricts access to or

use of any required emergency exit, or of the aisle in the passenger

compartment.

(6) The bin must be fully enclosed and made of material that is at least

flame-resistant.

(7) Suitable safeguards must be provided within the bin to prevent the

cargo from shifting under emergency landing conditions.

(8) The bin may not be installed in a position that obscures any

passenger's view of the "seat belt" sign, "no smoking" sign, or any required

exit sign, unless an auxiliary sign or other approved means for proper

notification of the passenger is provided.

(c) All cargo may be carried forward of the foremost seated passengers and

carry-on baggage may be carried alongside the foremost seated passengers if

the cargo (including carry-on baggage) is carried either in approved bins as

specified in paragraph (b) of this section or in accordance with the

following:

(1) It is properly secured by a safety belt or other tie down having enough

strength to eliminate the possibility of shifting under all normally

anticipated flight and ground conditions.

(2) It is packaged or covered in a manner to avoid possible injury to

passengers.

(3) It does not impose any load on seats or the floor structure that

exceeds the load limitation for those components.

(4) Its location does not restrict access to or use of any required

emergency or regular exit, or of the aisle in the passenger compartment.

(5) Its location does not obscure any passenger's view of the "seat belt"

sign, "no smoking" sign, or required exit sign, unless an auxiliary sign or

other approved means for proper notification of the passenger is provided.

Sec. 125.185  Carriage of cargo in cargo compartments.

When cargo is carried in cargo compartments that are designed to require

the physical entry of a crewmember to extinguish any fire that may occur

during flight, the cargo must be loaded so as to allow a crewmember to

effectively reach all parts of the compartment with the contents of a hand-

held fire extinguisher.

Sec. 125.187  Landing gear: Aural warning device.

(a) Except for airplanes that comply with the requirements of Sec. 25.729

of this chapter on or after January 6, 1992, each airplane must have a

landing gear aural warning device that functions continuously under the

following conditions:

(1) For airplanes with an established approach wing-flap position, whenever

the wing flaps are extended beyond the maximum certificated approach climb

configuration position in the Airplane Flight Manual and the landing gear is

not fully extended and locked.

(2) For airplanes without an established approach climb wing-flap position,

whenever the wing flaps are extended beyond the position at which landing

gear extension is normally performed and the landing gear is not fully

extended and locked.

(b) The warning system required by paragraph (a) of this section--

(1) May not have a manual shutoff;

(2) Must be in addition to the throttle-actuated device installed under the

type certification airworthiness requirements; and

(3) May utilize any part of the throttle-actuated system including the

aural warning device.

(c) The flap position sensing unit may be installed at any suitable place

in the airplane.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-16, 56

FR 63762, Dec. 5, 1991]

*****************************************************************************

56 FR 63760, No. 234, Dec. 5, 1991

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the airworthiness standards for landing gear aural warning systems in

transport category airplanes to reflect current design practices. They

require that if a landing is attempted when the landing gear is not locked

down, the flightcrew must be given an aural warning in sufficient time to

allow the landing gear to be locked down or a go-around to be made. These

amendments state the intent of the current regulations in more objective

terms to eliminate nuisance warnings and to simplify the certification

process.

EFFECTIVE DATE: January 6, 1992.

*****************************************************************************

Sec. 125.189   Demonstration of emergency evacuation procedures.

(a) Each certificate holder must show, by actual demonstration conducted in

accordance with paragraph (a) of Appendix B of this part, that the emergency

evacuation procedures for each type and model of airplane with a seating of

more than 44 passengers, that is used in its passenger-carrying operations,

allow the evacuation of the full seating capacity, including crewmembers, in

90 seconds or less, in each of the following circumstances:

(1) A demonstration must be conducted by the certificate holder upon the

initial introduction of a type and model of airplane into passenger-carrying

operations. However, the demonstration need not be repeated for any airplane

type or model that has the same number and type of exits, the same cabin

configuration, and the same emergency equipment as any other airplane used by

the certificate holder in successfully demonstrating emergency evacuation in

compliance with this paragraph.

(2) A demonstration must be conducted--

(i) Upon increasing by more than 5 percent the passenger seating capacity

for which successful demonstration has been conducted; or

(ii) Upon a major change in the passenger cabin interior configuration that

will affect the emergency evacuation of passengers.

(b) If a certificate holder has conducted a successful demonstration

required by Sec. 121.291(a) in the same type airplane as a Part 121 or Part

123 certificate holder, it need not conduct a demonstration under this

paragraph in that type airplane to achieve certification under Part 125.

(c) Each certificate holder operating or proposing to operate one or more

landplanes in extended overwater operations, or otherwise required to have

certain equipment under Sec. 125.209, must show, by a simulated ditching

conducted in accordance with paragraph (b) of Appendix B of this part, that

it has the ability to efficiently carry out its ditching procedures.

(d) If a certificate holder has conducted a successful demonstration

required by Sec. 121.291(b) in the same type airplane as a Part 121 or Part

123 certificate holder, it need not conduct a demonstration under this

paragraph in that type airplane to achieve certification under Part 125.

Subpart F--Instrument and Equipment Requirements

Sec. 125.201  Inoperable instruments and equipment.

(a) No person may take off an airplane with inoperable instruments or

equipment installed unless the following conditions are met:

(1) An approved Minimum Equipment List exists for that airplane.

(2) The Flight Standards District Office having certification

responsibility has issued the certificate holder operations specifications

authorizing operations in accordance with an approved Minimum Equipment List.

The flight crew shall have direct access at all times prior to flight to all

of the information contained in the approved Minimum Equipment List through

printed or other means approved by the Administrator in the certificate

holders operations specifications. An approved Minimum Equipment List, as

authorized by the operations specifications, constitutes an approved change

to the type design without requiring recertification.

(3) The approved Minimum Equipment List must:

(i) Be prepared in accordance with the limitations specified in paragraph

(b) of this section.

(ii) Provide for the operation of the airplane with certain instruments and

equipment in an inoperable condition.

(4) Records identifying the inoperable instruments and equipment and the

information required by paragraph (a)(3)(ii) of this section must be

available to the pilot.

(5) The airplane is operated under all applicable conditions and

limitations contained in the Minimum Equipment List and the operations

specifications authorizing use of the Minimum Equipment List.

(b) The following instruments and equipment may not be included in the

Minimum Equipment List:

(1) Instruments and equipment that are either specifically or otherwise

required by the airworthiness requirements under which the airplane is type

certificated and which are essential for safe operations under all operating

conditions.

(2) Instruments and equipment required by an airworthiness directive to be

in operable condition unless the airworthiness directive provides otherwise.

(3) Instruments and equipment required for specific operations by this

part.

(c) Notwithstanding paragraphs (b)(1) and (b)(3) of this section, an

airplane with inoperable instruments or equipment may be operated under a

special flight permit under Secs. 21.197 and 21.199 of this chapter.

[Amdt. 125-15, 56 FR 12310, Mar. 22, 1991]

*****************************************************************************

56 FR 12306, No. 56, Mar. 22, 1991

SUMMARY: This amendment provides for the development and use of Minimum

Equipment Lists (MEL) for certain single-engine air carrier aircraft. In

addition, this amendment revises the requirements for the use of an MEL to

make them consistent throughout the regulations. This action is needed to

provide for the implementation of MEL authorizations through the issuance of

operations specifications. The changes streamline administrative procedures

and provide greater consistency in the MEL authorization process.

EFFECTIVE DATE: June 20, 1991.

*****************************************************************************

Sec. 125.203   Radio and navigational equipment.

(a) No person may operate an airplane unless it has two-way radio

communications equipment able, at least in flight, to transmit to, and

receive from, ground facilities 25 miles away.

(b) No person may operate an airplane over-the-top unless it has radio

navigational equipment able to receive radio signals from the ground

facilities to be used.

(c) No person may operate an airplane carrying passengers under IFR or in

extended overwater operations unless it has at least the following radio

communication and navigational equipment appropriate to the facilities to be

used which are capable of transmitting to, and receiving from, at any place

on the route to be flown, at least one ground facility:

(1) Two transmitters, (2) two microphones, (3) two headsets or one headset

and one speaker (4) a marker beacon receiver, (5) two independent receivers

for navigation, and (6) two independent receivers for communications.

(d) For the purposes of paragraphs (c)(5) and (c)(6) of this section, a

receiver is independent if the function of any part of it does not depend on

the functioning of any part of another receiver. However, a receiver that can

receive both communications and navigational signals may be used in place of

a separate communications receiver and a separate navigational signal

receiver.

Sec. 125.205  Equipment requirements: Airplanes under IFR.

No person may operate an airplane under IFR unless it has--

(a) A vertical speed indicator;

(b) A free-air temperature indicator;

(c) A heated pitot tube for each airspeed indicator;

(d) A power failure warning device or vacuum indicator to show the power

available for gyroscopic instruments from each power source;

(e) An alternate source of static pressure for the altimeter and the

airspeed and vertical speed indicators;

(f) At least two generators each of which is on a separate engine, or which

any combination of one-half of the total number are rated sufficiently to

supply the electrical loads of all required instruments and equipment

necessary for safe emergency operation of the airplane; and

(g) Two independent sources of energy (with means of selecting either), of

which at least one is an engine-driven pump or generator, each of which is

able to drive all gyroscopic instruments and installed so that failure of one

instrument or source does not interfere with the energy supply to the

remaining instruments or the other energy source. For the purposes of this

paragraph, each engine-driven source of energy must be on a different engine.

(h) For the purposes of paragraph (f) of this section, a continuous

inflight electrical load includes one that draws current continuously during

flight, such as radio equipment, electrically driven instruments, and lights,

but does not include occasional intermittent loads.

(i) An airspeed indicating system with heated pitot tube or equivalent

means for preventing malfunctioning due to icing.

(j) A sensitive altimeter.

(k) Instrument lights providing enough light to make each required

instrument, switch, or similar instrument easily readable and installed so

that the direct rays are shielded from the flight crewmembers' eyes and that

no objectionable reflections are visible to them. There must be a means of

controlling the intensity of illumination unless it is shown that nondimming

instrument lights are satisfactory.

Sec. 125.206  Pitot heat indication systems.

(a) Except as provided in paragraph (b) of this section, after April 12,

1981, no person may operate a transport category airplane equipped with a

flight instrument pitot heating system unless the airplane is equipped with

an operable pitot heat indication system that complies with Sec. 25.1326 of

this chapter in effect on April 12, 1978.

(b) A certificate holder may obtain an extension of the April 12, 1981,

compliance date specified in paragraph (a) of this section, but not beyond

April 12, 1983, from the Director, Flight Standards Service if the

certificate holder--

(1) Shows that due to circumstances beyond its control it cannot comply by

the specified compliance date; and

(2) Submits by the specified compliance date a schedule for compliance

acceptable to the Director, indicating that compliance will be achieved at

the earliest practicable date.

[Amdt. 125-3, 46 FR 43806, Aug. 31, 1981, as amended by Amdt. 125-13, 54 FR

39294, Sept. 25, 1989]

Sec. 125.207  Emergency equipment requirements.

(a) No person may operate an airplane having a seating capacity of 20 or

more passengers unless it is equipped with the following emergency equipment:

(1) One approved first aid kit for treatment of injuries likely to occur in

flight or in a minor accident, which meets the following specifications and

requirements:

(i) Each first aid kit must be dust and moisture proof and contain only

materials that either meet Federal Specifications GGK-391a, as revised, or as

approved by the Administrator.

(ii) Required first aid kits must be readily accessible to the cabin flight

attendants.

(iii) At time of takeoff, each first aid kit must contain at least the

following or other contents approved by the Administrator:

Quantity

Contents:

Adhesive bandage compressors, 1 in                                  16

Antiseptic swabs                                                    20

Ammonia inhalents                                                   10

Bandage compressors, 4 in                                            8

Triangular bandage compressors, 40 in                                5

Arm splint, noninflatable                                            1

Leg splint, noninflatable                                            1

Roller bandage, 4 in                                                 4

Adhesive tape, 1-in standard roll                                    2

Bandage scissors                                                     1

(2) A crash axe carried so as to be accessible to the crew but inaccessible

to passengers during normal operations.

(3) Signs that are visible to all occupants to notify them when smoking is

prohibited and when safety belts should be fastened. The signs must be so

constructed that they can be turned on and off by a crewmember. They must be

turned on for each takeoff and each landing and when otherwise considered to

be necessary by the pilot in command.

(4) The additional emergency equipment specified in Appendix A of this

part.

(b) Megaphones. Each passenger-carrying airplane must have a portable

battery-powered megaphone or megaphones readily accessible to the crewmembers

assigned to direct emergency evacuation, installed as follows:

(1) One megaphone on each airplane with a seating capacity of more than 60

and less than 100 passengers, at the most rearward location in the passenger

cabin where it would be readily accessible to a normal flight attendant seat.

However, the Administrator may grant a deviation from the requirements of

this paragraph if the Administrator finds that a different location would be

more useful for evacuation of persons during an emergency.

(2) Two megaphones in the passenger cabin on each airplane with a seating

capacity of more than 99 and less than 200 passengers, one installed at the

forward end and the other at the most rearward location where it would be

readily accessible to a normal flight attendant seat.

(3) Three megaphones in the passenger cabin on each airplane with a seating

capacity of more than 199 passengers, one installed at the forward end, one

installed at the most rearward location where it would be readily accessible

to a normal flight attendant seat, and one installed in a readily accessible

location in the mid-section of the airplane.

[Dkt. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-19, 59

FR 1781, Jan. 12, 1994]

*****************************************************************************

59 FR 1780, No. 8, Jan. 12, 1994

SUMMARY: This amendment revises the regulations concerning first aid kits

required on board air carrier, air taxi, and commercial aircraft to remove

the burn compound from the list of items required for the kits. This

amendment responds to a petition from Air Transport Association, supported by

the American Red Cross, that the burn compound be removed from the kits since

the use of ice or cold water is the preferred treatment for minor burns. This

amendment will relieve affected operators from the expense of having to

periodically replace an item in the first aid kits that is not needed.

DATES: Effective date: January 12, 1994.

*****************************************************************************

Sec. 125.209  Emergency equipment: Extended overwater operations.

(a) No person may operate an airplane in extended overwater operations

unless it carries, installed in conspicuously marked locations easily

accessible to the occupants if a ditching occurs, the following equipment:

(1) An approved life preserver equipped with an approved survivor locator

light, or an approved flotation means, for each occupant of the aircraft. The

life preserver or other flotation means must be easily accessible to each

seated occupant. If a flotation means other than a life preserver is used, it

must be readily removable from the airplane.

(2) Enough approved life rafts (with proper buoyancy) to carry all

occupants of the airplane, and at least the following equipment for each raft

clearly marked for easy identification--

(i) One canopy (for sail, sunshade, or rain catcher);

(ii) One radar reflector (or similar device);

(iii) One life raft repair kit;

(iv) One bailing bucket;

(v) One signaling mirror;

(vi) One police whistle;

(vii) One raft knife;

(viii) One CO 2, bottle for emergency inflation;

(ix) One inflation pump;

(x) Two oars;

(xi) One 75-foot retaining line;

(xii) One magnetic compass;

(xiii) One dye marker;

(xiv) One flashlight having at least two size "D" cells or equivalent;

(xv) At least one approved pyrotechnic signaling device;

(xvi) A 2-day supply of emergency food rations supplying at least 1,000

calories a day for each person;

(xvii) One sea water desalting kit for each two persons that raft is rated

to carry, or two pints of water for each person the raft is rated to carry;

(xviii) One fishing kit; and

(xix) One book on survival appropriate for the area in which the airplane

is operated.

(b) No person may operate an airplane in extended overwater operations

unless there is attached to one of the life rafts required by paragraph (a)

of this section, a survival type emergency locator transmitter that meets

TSO-C91. Batteries used in this transmitter must be replaced (or recharged,

if the batteries are rechargeable) when the transmitter has been in use for

more than 1 cumulative hour, and also when 50 percent of their useful life

(or for rechargeable batteries, 50 percent of their useful life of charge),

as established by the transmitter manufacturer under TSO-C91 has expired. The

new expiration date for the replacement or recharged batteries must be

legibly marked on the outside of the transmitter. The battery useful life or

useful life of charge requirements of this paragraph do not apply to

batteries (such as water-activated batteries) that are essentially unaffected

during probably storage intervals.

Sec. 125.211  Seat and safety belts.

(a) No person may operate an airplane unless there are available during the

takeoff, en route flight, and landing--

(1) An approved seat or berth for each person on board the airplane who is

at least 2 years old; and

(2) An approved safety belt for separate use by each person on board the

airplane who is at least 2 years old, except that two persons occupying a

berth may share one approved safety belt and two persons occupying a multiple

lounge or divan seat may share one approved safety belt during en route

flight only.

(b) Except as provided in paragraphs (b)(1) and (b)(2) of this section,

each person on board an airplane operated under this part shall occupy an

approved seat or berth with a separate safety belt properly secured about him

or her during movement on the surface, takeoff, and landing. A safety belt

provided for the occupant of a seat may not be used for more than one person

who has reached his or her second birthday. Notwithstanding the preceding

requirements, a child may:

(1) Be held by an adult who is occupying a seat or berth if that child has

not reached his or her second birthday; or

(2) Notwithstanding any other requirement of this chapter, occupy an

approved child restraint system furnished by the certificate holder or one of

the persons described in paragraph (b)(2)(i) of this section, provided:

(i) The child is accompanied by a parent, guardian, or attendant designated

by the child's parent or guardian to attend to the safety of the child during

the flight;

(ii) The approved child restraint system bears one or more labels as

follows:

(A) Seats manufactured to U.S. standards between January 1, 1981, and

February 25, 1985, must bear the label: "This child restraint system conforms

to all applicable Federal motor vehicle safety standards." Vest- and harness-

type child restraint systems manufactured before February 26, 1985, bearing

such a label are not approved for the purposes of this section;

(B) Seats manufactured to U.S. standards on or after February 26, 1985,

must bear two labels:

(1) "This child restraint system conforms to all applicable Federal motor

vehicle safety standards"; and

(2) "THIS RESTRAINT IS CERTIFIED FOR USE IN MOTOR VEHICLES AND AIRCRAFT" in

red lettering;

(C) Seats that do not qualify under paragraphs (b)(2)(ii)(A) and

(b)(2)(ii)(B) of this section must bear either a label showing approval of a

foreign government or a label showing that the seat was manufactured under

the standards of the United Nations; and

(iii) The certificate holder complies with the following requirements:

(A) The restraint system must be properly secured to an approved forward-

facing seat or berth;

(B) The child must be properly secured in the restraint system and must not

exceed the specified weight limit for the restraint system; and

(C) The restraint system must bear the appropriate label(s).

(c) No certificate holder may prohibit a child, if requested by the child's

parent, guardian, or designated attendant from occupying a child restraint

system furnished by the child's parent, guardian, or designated attendant,

provided the child holds an authorization for an approved seat or berth and

the requirements contained in paragraphs (b)(2)(i) through (b)(2)(iii) of

this section are met. This section does not prohibit the certificate holder

from providing child restraint systems or, consistent with safe operating

practices, determining the most appropriate passenger seat location for the

child restraint system.

(d) Each sideward facing seat must comply with the applicable requirements

of Sec. 25.785(c) of this chapter.

(e) No certificate holder may take off or land an airplane unless each

passenger seat back is in the upright position. Each passenger shall comply

with instructions given by a crewmember in compliance with this paragraph.

This paragraph does not apply to seats on which cargo or persons who are

unable to sit erect for a medical reason are carried in accordance with

procedures in the certificate holder's manual if the seat back does not

obstruct any passenger's access to the aisle or to any emergency exit.

(f) Each occupant of a seat equipped with a shoulder harness must fasten

the shoulder harness during takeoff and landing, except that, in the case of

crewmembers, the shoulder harness need not be fastened if the crewmember

cannot perform his required duties with the shoulder harness fastened.

[Dkt. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-17, 57

FR 42674, Sept. 15, 1992]

*****************************************************************************

57 FR 42662, No. 179, Sept. 15, 1992

SUMMARY: This final rule amends the Federal Aviation Regulations by requiring

operators and certificate holders to allow the use of approved child

restraint systems and by updating certain regulations concerning passenger

and crewmember safety, attitude indicators, and check airmen. This action is

in response to requests from the public, consumer groups, and Congress;

reports from FAA inspectors; and investigations and recommendations by the

National Transportation Safety Board. The rule is intended to increase the

safety of crewmembers and passengers on board aircraft and to update other

operational amendments.

EFFECTIVE DATE: October 15, 1992.

*****************************************************************************

Sec. 125.213  Miscellaneous equipment.

No person may conduct any operation unless the following equipment is

installed in the airplane:

(a) If protective fuses are installed on an airplane, the number of spare

fuses approved for the airplane and appropriately described in the

certificate holder's manual.

(b) A windshield wiper or equivalent for each pilot station.

(c) A power supply and distribution system that meets the requirements of

Secs. 25.1309, 25.1331, 25.1351 (a) and (b) (1) through (4), 25.1353,

25.1355, and 25.1431(b) or that is able to produce and distribute the load

for the required instruments and equipment, with use of an external power

supply if any one power source or component of the power distribution system

fails. The use of common elements in the system may be approved if the

Administrator finds that they are designed to be reasonably protected against

malfunctioning. Engine-driven sources of energy, when used, must be on

separate engines.

(d) A means for indicating the adequacy of the power being supplied to

required flight instruments.

(e) Two independent static pressure systems, vented to the outside

atmospheric pressure so that they will be least affected by air flow

variation or moisture or other foreign matter, and installed so as to be

airtight except for the vent. When a means is provided for transferring an

instrument from its primary operating system to an alternative system, the

means must include a positive positioning control and must be marked to

indicate clearly which system is being used.

(f) A placard on each door that is the means of access to a required

passenger emergency exit to indicate that it must be open during takeoff and

landing.

(g) A means for the crew, in an emergency, to unlock each door that leads

to a compartment that is normally accessible to passengers and that can be

locked by passengers.

Sec. 125.215   Operating information required.

(a) The operator of an airplane must provide the following materials, in

current and appropriate form, accessible to the pilot at the pilot station,

and the pilot shall use them:

(1) A cockpit checklist.

(2) An emergency cockpit checklist containing the procedures required by

paragraph (c) of this section, as appropriate.

(3) Pertinent aeronautical charts.

(4) For IFR operations, each pertinent navigational en route, terminal

area, and approach and letdown chart;

(5) One-engine-inoperative climb performance data and, if the airplane is

approved for use in IFR or over-the-top operations, that data must be

sufficient to enable the pilot to determine that the airplane is capable of

carrying passengers over-the-top or in IFR conditions at a weight that will

allow it to climb, with the critical engine inoperative, at least 50 feet a

minute when operating at the MEA's of the route to be flown or 5,000 feet

MSL, whichever is higher.

(b) Each cockpit checklist required by paragraph (a)(1) of this section

must contain the following procedures:

(1) Before starting engines;

(2) Before take-off;

(3) Cruise;

(4) Before landing;

(5) After landing;

(6) Stopping engines.

(c) Each emergency cockpit checklist required by paragraph (a)(2) of this

section must contain the following procedures, as appropriate:

(1) Emergency operation of fuel, hydraulic, electrical, and mechanical

systems.

(2) Emergency operation of instruments and controls.

(3) Engine inoperative procedures.

(4) Any other emergency procedures necessary for safety.

Sec. 125.217  Passenger information.

(a) Except as provided in paragraph (b) of this section, no person may

operate an airplane carrying passengers unless it is equipped with signs that

meet the requirements of Sec. 25.791 of this chapter and that are visible to

passengers and flight attendants to notify them when smoking is prohibited

and when safety belts must be fastened. The signs must be so constructed that

the crew can turn them on and off. They must be turned on during airplane

movement on the surface, for each takeoff, for each landing, and when

otherwise considered to be necessary by the pilot in command.

(b) No passenger or crewmember may smoke while any "No Smoking" sign is

lighted nor may any passenger or crewmember smoke in any lavatory.

(c) Each passenger required by Sec. 125.211(b) to occupy a seat or berth

shall fasten his or her safety belt about him or her and keep it fastened

while any "Fasten Seat Belt" sign is lighted.

(d) Each passenger shall comply with instructions given him or her by

crewmembers regarding compliance with paragraphs (b) and (c) of this section.

[Amdt. 125-17, 57 FR 42675, Sept. 15, 1992]

*****************************************************************************

57 FR 42662, No. 179, Sept. 15, 1992

SUMMARY: This final rule amends the Federal Aviation Regulations by requiring

operators and certificate holders to allow the use of approved child

restraint systems and by updating certain regulations concerning passenger

and crewmember safety, attitude indicators, and check airmen. This action is

in response to requests from the public, consumer groups, and Congress;

reports from FAA inspectors; and investigations and recommendations by the

National Transportation Safety Board. The rule is intended to increase the

safety of crewmembers and passengers on board aircraft and to update other

operational amendments.

EFFECTIVE DATE: October 15, 1992.

*****************************************************************************

Sec. 125.219   Oxygen for medical use by passengers.

(a) Except as provided in paragraphs (d) and (e) of this section, no

certificate holder may allow the carriage or operation of equipment for the

storage, generation or dispensing of medical oxygen unless the unit to be

carried is constructed so that all valves, fittings, and gauges are protected

from damage during that carriage or operation and unless the following

conditions are met:

(1) The equipment must be--

(i) Of an approved type or in conformity with the manufacturing, packaging,

marking, labeling, and maintenance requirements of Title 49 CFR Parts 171,

172, and 173, except Sec. 173.24(a)(1);

(ii) When owned by the certificate holder, maintained under the certificate

holder's approved maintenance program;

(iii) Free of flammable contaminants on all exterior surfaces; and

(iv) Appropriately secured.

(2) When the oxygen is stored in the form of a liquid, the equipment must

have been under the certificate holder's approved maintenance program since

its purchase new or since the storage container was last purged.

(3) When the oxygen is stored in the form of a compressed gas as defined in

Title 49 CFR 173.300(a)--

(i) When owned by the certificate holder, it must be maintained under its

approved maintenance program; and

(ii) The pressure in any oxygen cylinder must not exceed the rated cylinder

pressure.

(4) The pilot in command must be advised when the equipment is on board and

when it is intended to be used.

(5) The equipment must be stowed, and each person using the equipment must

be seated so as not to restrict access to or use of any required emergency or

regular exit or of the aisle in the passenger compartment.

(b) When oxygen is being used, no person may smoke and no certificate

holder may allow any person to smoke within 10 feet of oxygen storage and

dispensing equipment carried under paragraph (a) of this section.

(c) No certificate holder may allow any person other than a person trained

in the use of medical oxygen equipment to connect or disconnect oxygen

bottles or any other ancillary component while any passenger is aboard the

airplane.

(d) Paragraph (a)(1)(i) of this section does not apply when that equipment

is furnished by a professional or medical emergency service for use on board

an airplane in a medical emergency when no other practical means of

transportation (including any other properly equipped certificate holder) is

reasonably available and the person carried under the medical emergency is

accompanied by a person trained in the use of medical oxygen.

(e) Each certificate holder who, under the authority of paragraph (d) of

this section, deviates from paragraph (a)(1)(i) of this section under a

medical emergency shall, within 10 days, excluding Saturdays, Sundays, and

Federal holidays, after the deviation, send to the FAA Flight Standards

district office charged with the overall inspection of the certificate holder

a complete report of the operation involved, including a description of the

deviation and the reasons for it.

Sec. 125.221   Icing conditions: Operating limitations.

(a) No pilot may take off an airplane that has frost, ice, or snow adhering

to any propeller, windshield, wing, stabilizing or control surface, to a

powerplant installation, or to an airspeed, altimeter, rate of climb, or

flight attitude instrument system, except under the follow conditions:

(1) Takeoffs may be made with frost adhering to the wings, or stabilizing

or control surfaces, if the frost has been polished to make it smooth.

(2) Takeoffs may be made with frost under the wing in the area of the fuel

tanks if authorized by the Administrator.

(b) No certificate holder may authorize an airplane to take off and no

pilot may take off an airplane any time conditions are such that frost, ice,

or snow may reasonably be expected to adhere to the airplane unless the pilot

has completed the testing required under Sec. 125.287(a)(9) and unless one of

the following requirements is met:

(1) A pretakeoff contamination check, that has been established by the

certificate holder and approved by the Administrator for the specific

airplane type, has been completed within 5 minutes prior to beginning

takeoff. A pretakeoff contamination check is a check to make sure the wings

and control surfaces are free of frost, ice, or snow.

(2) The certificate holder has an approved alternative procedure and under

that procedure the airplane is determined to be free of frost, ice, or snow.

(3) The certificate holder has an approved deicing/anti-icing program that

complies with Sec. 121.629(c) of this chapter and the takeoff complies with

that program.

(c) Except for an airplane that has ice protection provisions that meet

Appendix C of this part or those for transport category airplane type

certification, no pilot may fly--

(1) Under IFR into known or forecast light or moderate icing conditions; or

(2) Under VFR into known light or moderate icing conditions, unless the

airplane has functioning deicing or anti-icing equipment protecting each

propeller, windshield, wing, stabilizing or control surface, and each

airspeed, altimeter, rate of climb, or flight attitude instrument system.

(d) Except for an airplane that has ice protection provisions that meet

Appendix C of this part or those for transport category airplane type

certification, no pilot may fly an airplane into known or forecast severe

icing conditions.

(e) If current weather reports and briefing information relied upon by the

pilot in command indicate that the forecast icing condition that would

otherwise prohibit the flight will not be encountered during the flight

because of changed weather conditions since the forecast, the restrictions in

paragraphs (b) and (c) of this section based on forecast conditions do not

apply.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-18, 58 FR

69629, Dec. 30, 1993]

*****************************************************************************

58 FR 69620, No. 249, Dec. 30, 1993

SUMMARY: This interim final rule requires part 125 certificate holders to

provide pilot testing on conducting operations in ground icing conditions,

part 135 certificate holders to provide pilot training on conducting

operations in ground icing conditions, and part 125 and 135 certificate

holders to check airplanes for contamination (i.e., frost, ice, or snow)

prior to takeoff when ground icing conditions exist. This rule is necessary

because accident statistics and experience indicate the importance of

effectively determining whether the airplane's wings and control surfaces are

free of all frost, ice, or snow prior to beginning a takeoff. The rule is

intended to provide an added level of safety to flight operations in ground

icing conditions under parts 125 and 135.

DATES: This interim final rule is effective January 31, 1994.

*****************************************************************************

Sec. 125.223   Airborne weather radar equipment requirements.

(a) No person may operate an airplane governed by this part in passenger-

carrying operations unless approved airborne weather radar equipment is

installed in the airplane.

(b) No person may begin a flight under IFR or night VFR conditions when

current weather reports indicate that thunderstorms, or other potentially

hazardous weather conditions that can be detected with airborne weather radar

equipment, may reasonably be expected along the route to be flown, unless the

airborne weather radar equipment required by paragraph (a) of this section is

in satisfactory operating condition.

(c) If the airborne weather radar equipment becomes inoperative en route,

the airplane must be operated under the instructions and procedures specified

for that event in the manual required by Sec. 125.71.

(d) This section does not apply to airplanes used solely within the State

of Hawaii, within the State of Alaska, within that part of Canada west of

longitude 130 degrees W, between latitude 70 degrees N, and latitude 53

degrees N, or during any training, test, or ferry flight.

(e) Without regard to any other provision of this part, an alternate

electrical power supply is not required for airborne weather radar equipment.

Sec. 125.224   Traffic Alert and Collision Avoidance System.

(a) After December 30, 1993, no person may operate a large airplane that

has a passenger seating configuration, excluding any pilot seat, of more than

30 seats unless it is equipped with an approved TCAS II traffic alert and

collision avoidance system and the appropriate class of Mode S transponder.

(b) The manual required by Sec. 125.71 of this part shall contain the

following information on the TCAS II system required by this section.

(1) Appropriate procedures for--

(i) The operation of the equipment; and

(ii) Proper flightcrew action with respect to the equipment.

(2) An outline of all input sources that must be operating for the TCAS II

to function properly.

[Doc. No. 25355, Amdt. No. 125-11, 54 FR 951, Jan. 10, 1989, as amended at

Amdt. No. 125-14, 55 FR 13247, Apr. 9, 1990]

*****************************************************************************

55 FR 13242, No. 68, Apr. 9, 1990

SUMMARY: This rule revises the schedule for installing Traffic Alert and

Collision Avoidance Systems (TCAS II) on airplanes with more than 30

passenger seats. The TCAS II system will provide a collision avoidance

capability that operates independently of the ground-based Air Traffic

Control (ATC) system and in areas where there is no ATC coverage. Congress

recently passed legislation permitting an extension of the schedule. This

action implements the legislation, reduces the prospect that carriers will

divert critical maintenance and modification resources away from other safety

programs to meet the TCAS II schedule, and allows the FAA to evaluate the

operation of TCAS II in the total ATC environment.

EFFECTIVE DATE: May 9, 1990.

*****************************************************************************

Sec. 125.225  Flight recorders.

(a) Except as provided in paragraph (d) of this section, after October 11,

1991, no person may operate a large airplane type certificated before October

1, 1969, for operations above 25,000 feet altitude, nor a multiengine,

turbine powered airplane type certificated before October 1, 1969, unless it

is equipped with one or more approved flight recorders that utilize a digital

method of recording and storing data and a method of readily retrieving that

data from the storage medium. The following information must be able to be

determined within the ranges, accuracies, resolution, and recording intervals

specified in Appendix D of this part:

(1) Time;

(2) Altitude;

(3) Airspeed;

(4) Vertical acceleration;

(5) Heading;

(6) Time of each radio transmission to or from air traffic control;

(7) Pitch attitude;

(8) Roll attitude;

(9) Longitudinal acceleration;

(10) Control column or pitch control surface position; and

(11) Thrust of each engine.

(b) Except as provided in paragraph (d) of this section, after October 11,

1991, no person may operate a large airplane type certificated after

September 30, 1969, for operations above 25,000 feet altitude, nor a

multiengine, turbine powered airplane type certificated after September 30,

1969, unless it is equipped with one or more approved flight recorders that

utilize a digital method of recording and storing data and a method of

readily retrieving that data from the storage medium. The following

information must be able to be determined within the ranges, accuracies,

resolutions, and recording intervals specified in Appendix D of this part:

(1) Time;

(2) Altitude;

(3) Airspeed;

(4) Vertical acceleration;

(5) Heading;

(6) Time of each radio transmission either to or from air traffic control;

(7) Pitch attitude;

(8) Roll attitude;

(9) Longitudinal acceleration;

(10) Pitch trim position;

(11) Control column or pitch control surface position;

(12) Control wheel or lateral control surface position;

(13) Rudder pedal or yaw control surface position;

(14) Thrust of each engine;

(15) Position of each trust reverser;

(16) Trailing edge flap or cockpit flap control position; and

(17) Leading edge flap or cockpit flap control position.

(c) After October 11, 1991, no person may operate a large airplane equipped

with a digital data bus and ARINC 717 digital flight data acquisition unit

(DFDAU) or equivalent unless it is equipped with one or more approved flight

recorders that utilize a digital method of recording and storing data and a

method of readily retrieving that data from the storage medium. Any

parameters specified in Appendix D of this part that are available on the

digital data bus must be recorded within the ranges, accuracies, resolutions,

and sampling intervals specified.

(d) No person may operate under this part an airplane that is manufactured

after October 11, 1991, unless it is equipped with one or more approved

flight recorders that utilize a digital method of recording and storing data

and a method of readily retrieving that data from the storage medium. The

parameters specified in Appendix D of this part must be recorded within the

ranges, accuracies, resolutions and sampling intervals specified. For the

purpose of this section, "manufactured" means the point in time at which the

airplane inspection acceptance records reflect that the airplane is complete

and meets the FAA-approved type design data.

(e) Whenever a flight recorder required by this section is installed, it

must be operated continuously from the instant the airplane begins the

takeoff roll until it has completed the landing roll at an airport.

(f) Except as provided in paragraph (g) of this section, and except for

recorded data erased as authorized in this paragraph, each certificate holder

shall keep the recorded data prescribed in paragraph (a), (b), (c), or (d) of

this section, as applicable, until the airplane has been operated for at

least 25 hours of the operating time specified in Sec. 125.227(a) of this

chapter. A total of 1 hour of recorded data may be erased for the purpose of

testing the flight recorder or the flight recorder system. Any erasure made

in accordance with this paragraph must be of the oldest recorded data

accumulated at the time of testing. Except as provided in paragraph (g) of

this section, no record need be kept more than 60 days.

(g) In the event of an accident or occurrence that requires immediate

notification of the National Transportation Safety Board under 49 CFR Part

830 and that results in termination of the flight, the certificate holder

shall remove the recording media from the airplane and keep the recorded data

required by paragraph (a), (b), (c), or (d) of this section, as applicable,

for at least 60 days or for a longer period upon the request of the Board or

the Administrator.

(h) Each flight recorder required by this section must be installed in

accordance with the requirements of Sec. 25.1459 of this chapter in effect on

August 31, 1977. The correlation required by Sec. 25.1459(c) of this chapter

need be established only on one airplane of any group of airplanes.

(1) That are of the same type;

(2) On which the flight recorder models and their installations are the

same; and

(3) On which there are no differences in the type design with respect to

the installation of the first pilot's instruments associated with the flight

recorder. The most recent instrument calibration, including the recording

medium from which this calibration is derived, and the recorder correlation

must be retained by the certificate holder.

(i) Each flight recorder required by this section that records the data

specified in paragraphs (a), (b), (c), or (d) of this section must have an

approved device to assist in locating that recorder under water.

[Doc. No. 25530, Amdt. 125-10, 53 FR 26148, July 11, 1988; 53 FR 30906, Aug.

16, 1988]

Sec. 125.227  Cockpit voice recorders.

(a) No certificate holder may operate a large turbine engine powered

airplane or a large pressurized airplane with four reciprocating engines

unless an approved cockpit voice recorder is installed in that airplane and

is operated continuously from the start of the use of the checklist (before

starting engines for the purpose of flight) to completion of the final

checklist at the termination of the flight.

(b) Each certificate holder shall establish a schedule for completion,

before the prescribed dates, of the cockpit voice recorder installations

required by paragraph (a) of this section. In addition, the certificate

holder shall identify any airplane specified in paragraph (a) of this section

he intends to discontinue using before the prescribed dates.

(c) The cockpit voice recorder required by this section must also meet the

following standards:

(1) The requirements of Part 25 of this chapter in effect after October 11,

1991.

(2) After September 1, 1980, each recorder container must--

(i) Be either bright orange or bright yellow;

(ii) Have reflective tape affixed to the external surface to facilitate its

location under water; and

(iii) Have an approved underwater locating device on or adjacent to the

container which is secured in such a manner that it is not likely to be

separated during crash impact, unless the cockpit voice recorder and the

flight recorder, required by Sec. 125.225 of this chapter, are installed

adjacent to each other in such a manner that they are not likely to be

separated during crash impact.

(d) In complying with this section, an approved cockpit voice recorder

having an erasure feature may be used so that, at any time during the

operation of the recorder, information recorded more than 30 minutes earlier

may be erased or otherwise obliterated.

(e) For those aircraft equipped to record the uninterrupted audio signals

received by a boom or a mask microphone the flight crewmembers are required

to use the boom microphone below 18,000 feet mean sea level. No person may

operate a large turbine engine powered airplane or a large pressurized

airplane with four reciprocating engines manufactured after October 11, 1991,

or on which a cockpit voice recorder has been installed after October 11,

1991, unless it is equipped to record the uninterrupted audio signal received

by a boom or mask microphone in accordance with Sec. 25.1457(c)(5) of this

chapter.

(f) In the event of an accident or occurrence requiring immediate

notification of the National Transportation Safety Board under 49 CFR Part

830 of its regulations, which results in the termination of the flight, the

certificate holder shall keep the recorded information for at least 60 days

or, if requested by the Administrator or the Board, for a longer period.

Information obtained from the record is used to assist in determining the

cause of accidents or occurrences in connection with investigations under 49

CFR Part 830. The Administrator does not use the record in any civil penalty

or certificate action.

[Doc. No. 25530, Amdt. 125-10, 53 FR 26149, July 11, 1988]

Subpart G--Maintenance

Sec. 125.241   Applicability.

This subpart prescribes rules, in addition to those prescribed in other

parts of this chapter, for the maintenance of airplanes, airframes, aircraft

engines, propellers, appliances, each item of survival and emergency

equipment, and their component parts operated under this part.

Sec. 125.243   Certificate holder's responsibilities.

(a) With regard to airplanes, including airframes, aircraft engines,

propellers, appliances, and survival and emergency equipment, operated by a

certificate holder, that certificate holder is primarily responsible for--

(1) Airworthiness;

(2) The performance of maintenance, preventive maintenance, and alteration

in accordance with applicable regulations and the certificate holder's

manual;

(3) The scheduling and performance of inspections required by this part;

and

(4) Ensuring that maintenance personnel make entries in the airplane

maintenance log and maintenance records which meet the requirements of Part

43 of this chapter and the certificate holder's manual, and which indicate

that the airplane has been approved for return to service after maintenance,

preventive maintenance, or alteration has been performed.

Sec. 125.245   Organization required to perform maintenance, preventive

maintenance, and alteration.

The certificate holder must ensure that each person with whom it arranges

for the performance of maintenance, preventive maintenance, alteration, or

required inspection items identified in the certificate holder's manual in

accordance with Sec. 125.249(a)(3)(ii) must have an organization adequate to

perform that work.

Sec. 125.247   Inspection programs and maintenance.

(a) No person may operate an airplane subject to this part unless

(1) The replacement times for life-limited parts specified in the aircraft

type certificate data sheets, or other documents approved by the

Administrator, are complied with;

(2) Defects disclosed between inspections, or as a result of inspection,

have been corrected in accordance with Part 43 of this chapter; and

(3) The airplane, including airframe, aircraft engines, propellers,

appliances, and survival and emergency equipment, and their component parts,

is inspected in accordance with an inspection program approved by the

Administrator.

(b) The inspection program specified in paragraph (a)(3) of this section

must include at least the following:

(1) Instructions, procedures, and standards for the conduct of inspections

for the particular make and model of airplane, including necessary tests and

checks. The instructions and procedures must set forth in detail the parts

and areas of the airframe, aircraft engines, propellers, appliances, and

survival and emergency equipment required to be inspected.

(2) A schedule for the performance of inspections that must be performed

under the program, expressed in terms of the time in service, calendar time,

number of system operations, or any combination of these.

(c) No person may be used to perform the inspections required by this part

unless that person is authorized to perform maintenance under Part 43 of this

chapter.

(d) No person may operate an airplane subject to this part unless--

(1) The installed engines have been maintained in accordance with the

overhaul periods recommended by the manufacturer or a program approved by the

Administrator; and

(2) The engine overhaul periods are specified in the inspection programs

required by Sec. 125.247(a)(3).

(e) Inspection programs which may be approved for use under this part

include, but are not limited to--

(1) A continuous inspection program which is a part of a current continuous

airworthiness program approved for use by a certificate holder under Part 121

or Part 135 of this chapter;

(2) Inspection programs currently recommended by the manufacturer of the

airplane, aircraft engines, propellers, appliances, or survival and emergency

equipment; or

(3) An inspection program developed by a certificate holder under this

part.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-2, 46 FR

24409, Apr. 30, 1981]

Sec. 125.249  Maintenance manual requirements.

(a) Each certificate holder's manual required by Sec. 125.71 of this part

shall contain, in addition to the items required by Sec. 125.73 of this part,

at least the following:

(1) A description of the certificate holders maintenance organization, when

the certificate holder has such an organization.

(2) A list of those persons with whom the certificate holder has arranged

for performance of inspections under this part. The list shall include the

persons' names and addresses.

(3) The inspection programs required by Sec. 125.247 of this part to be

followed in the performance of inspections under this part including--

(i) The method of performing routine and nonroutine inspections (other than

required inspections);

(ii) The designation of the items that must be inspected (required

inspections), including at least those which if improperly accomplished could

result in a failure, malfunction, or defect endangering the safe operation of

the airplane;

(iii) The method of performing required inspections;

(iv) Procedures for the inspection of work performed under previously

required inspection findings ("buy-back procedures");

(v) Procedures, standards, and limits necessary for required inspections

and acceptance or rejection of the items required to be inspected;

(vi) Instructions to prevent any person who performs any item of work from

performing any required inspection of that work; and

(vii) Procedures to ensure that work interruptions do not adversely affect

required inspections and to ensure required inspections are properly

completed before the airplane is released to service.

(b) In addition, each certificate holder's manual shall contain a suitable

system which may include a coded system that provides for the retention of

the following:

(1) A description (or reference to data acceptable to the Administrator) of

the work performed.

(2) The name of the person performing the work and the person's certificate

type and number.

(3) The name of the person approving the work and the person's certificate

type and number.

Sec. 125.251  Required inspection personnel.

(a) No person may use any person to perform required inspections unless the

person performing the inspection is appropriately certificated, properly

trained, qualified, and authorized to do so.

(b) No person may perform a required inspection if that person performed

the item of work required to be inspected.

Subpart H--Airman and Crewmember Requirements

Sec. 125.261  Airman: Limitations on use of services.

(a) No certificate holder may use any person as an airman nor may any

person serve as an airman unless that person--

(1) Holds an appropriate current airman certificate issued by the FAA;

(2) Has any required appropriate current airman and medical certificates in

that person's possession while engaged in operations under this part; and

(3) Is otherwise qualified for the operation for which that person is to be

used.

(b) Each airman covered by paragraph (a) of this section shall present the

certificates for inspection upon the request of the Administrator.

Sec. 125.263  Composition of flightcrew.

(a) No certificate holder may operate an airplane with less than the

minimum flightcrew specified in the type certificate and the Airplane Flight

Manual approved for that type airplane and required by this part for the kind

of operation being conducted.

(b) In any case in which this part requires the performance of two or more

functions for which an airman certificate is necessary, that requirement is

not satisfied by the performance of multiple functions at the same time by

one airman.

(c) On each flight requiring a flight engineer, at least one flight

crewmember, other than the flight engineer, must be qualified to provide

emergency performance of the flight engineer's functions for the safe

completion of the flight if the flight engineer becomes ill or is otherwise

incapacitated. A pilot need not hold a flight engineer's certificate to

perform the flight engineer's functions in such a situation.

Sec. 125.265  Flight engineer requirements.

(a) No person may operate an airplane for which a flight engineer is

required by the type certification requirements without a flight crewmember

holding a current flight engineer certificate.

(b) No person may serve as a required flight engineer on an airplane

unless, within the preceding 6 calendar months, that person has had at least

50 hours of flight time as a flight engineer on that type airplane, or the

Administrator has checked that person on that type airplane and determined

that person is familiar and competent with all essential current information

and operating procedures.

Sec. 125.267  Flight navigator and long-range navigation equipment.

(a) No certificate holder may operate an airplane outside the 48

conterminous States and the District of Columbia when its position cannot be

reliably fixed for a period of more than 1 hour, without--

(1) A flight crewmember who holds a current flight navigator certificate;

or

(2) Two independent, properly functioning, and approved long-range means of

navigation which enable a reliable determination to be made of the position

of the airplane by each pilot seated at that person's duty station.

(b) Operations where a flight navigator or long-range navigation equipment,

or both, are required are specified in the operations specifications of the

operator.

Sec. 125.269  Flight attendants.

(a) Each certificate holder shall provide at least the following flight

attendants on each passenger-carrying airplane used:

(1) For airplanes having more than 19 but less than 51 passengers--one

flight attendant.

(2) For airplanes having more than 50 but less than 101 passengers--two

flight attendants.

(3) For airplanes having more than 100 passengers--two flight attendants

plus one additional flight attendant for each unit (or part of a unit) of 50

passengers above 100 passengers.

(b) The number of flight attendants approved under paragraphs (a) and (b)

of this section are set forth in the certificate holder's operations

specifications.

(c) During takeoff and landing, flight attendants required by this section

shall be located as near as practicable to required floor level exits and

shall be uniformly distributed throughout the airplane to provide the most

effective egress of passengers in event of an emergency evacuation.

Sec. 125.271  Emergency and emergency evacuation duties.

(a) Each certificate holder shall, for each type and model of airplane,

assign to each category of required crewmember, as appropriate, the necessary

functions to be performed in an emergency or a situation requiring emergency

evacuation. The certificate holder shall show those functions are realistic,

can be practically accomplished, and will meet any reasonably anticipated

emergency, including the possible incapacitation of individual crewmembers or

their inability to reach the passenger cabin because of shifting cargo in

combination cargo-passenger airplanes.

(b) The certificate holder shall describe in its manual the functions of

each category of required crewmembers under paragraph (a) of this section.

Subpart I--Flight Crewmember Requirements

Sec. 125.281   Pilot-in-command qualifications.

No certificate holder may use any person, nor may any person serve, as

pilot in command of an airplane unless that person--

(a) Holds at least a commercial pilot certificate, an appropriate category,

class, and type rating, and an instrument rating; and

(b) Has had at least 1,200 hours of flight time as a pilot, including 500

hours of cross-country flight time, 100 hours of night flight time, including

at least 10 night takeoffs and landings, and 75 hours of actual or simulated

instrument flight time, at least 50 hours of which were actual flight.

Sec. 125.283   Second-in-command qualifications.

No certificate holder may use any person, nor may any person serve, as

second in command of an airplane unless that person--

(a) Holds at least a commercial pilot certificate with appropriate category

and class ratings, and an instrument rating; and

(b) For flight under IFR, meets the recent instrument experience

requirements prescribed for a pilot in command in Part 61 of this chapter.

Sec. 125.285  Pilot qualifications: Recent experience.

(a) No certificate holder may use any person, nor may any person serve, as

a required pilot flight crewmember unless within the preceding 90 days that

person has made at least three takeoffs and landings in the type airplane in

which that person is to serve. The takeoffs and landings required by this

paragraph may be performed in a visual simulator approved under Sec. 125.297

to include takeoff and landing maneuvers. However, any person who fails to

qualify for a 90-consecutive-day period following the date of that person's

last qualification under this paragraph must reestablish recency of

experience as provided in paragraph (b) of this section.

(b) A required pilot flight crewmember who has not met the requirements of

paragraph (a) of this section may reestablish recency of experience by making

at least three takeoffs and landings under the supervision of an authorized

check airman, in accordance with the following:

(1) At least one takeoff must be made with a simulated failure of the most

critical powerplant.

(2) At least one landing must be made from an ILS approach to the lowest

ILS minimums authorized for the certificate holder.

(3) At least one landing must be made to a complete stop.

(c) A required pilot flight crewmember who performs the maneuvers

prescribed in paragraph (b) of this section in a visual simulator must--

(1) Have previously logged 100 hours of flight time in the same type

airplane in which the pilot is to serve; and

(2) Be observed on the first two landings made in operations under this

part by an authorized check airman who acts as pilot in command and occupies

a pilot seat. The landings must be made in weather minimums that are not less

than those contained in the certificate holder's operations specifications

for Category I operations and must be made within 45 days following

completion of simulator testing.

(d) An authorized check airman who observes the takeoffs and landings

prescribed in paragraphs (b) and (c)(3) of this section shall certify that

the person being observed is proficient and qualified to perform flight duty

in operations under this part, and may require any additional maneuvers that

are determined necessary to make this certifying statement.

Sec. 125.287  Initial and recurrent pilot testing requirements.

(a) No certificate holder may use any person, nor may any person serve as a

pilot, unless, since the beginning of the 12th calendar month before that

service, that person has passed a written or oral test, given by the

Administrator or an authorized check airman on that person's knowledge in the

following areas--

(1) The appropriate provisions of Parts 61, 91, and 125 of this chapter and

the operations specifications and the manual of the certificate holder;

(2) For each type of airplane to be flown by the pilot, the airplane

powerplant, major components and systems, major appliances, performance and

operating limitations, standard and emergency operating procedures, and the

contents of the approved Airplane Flight Manual or approved equivalent, as

applicable;

(3) For each type of airplane to be flown by the pilot, the method of

determining compliance with weight and balance limitations for takeoff,

landing, and en route operations;

(4) Navigation and use of air navigation aids appropriate to the operation

of pilot authorization, including, when applicable, instrument approach

facilities and procedures;

(5) Air traffic control procedures, including IFR procedures when

applicable;

(6) Meteorology in general, including the principles of frontal systems,

icing, fog, thunderstorms, and windshear, and, if appropriate for the

operation of the certificate holder, high altitude weather;

(7) Procedures for avoiding operations in thunderstorms and hail, and for

operating in turbulent air or in icing conditions;

(8) New equipment, procedures, or techniques, as appropriate;

(9) Knowledge and procedures for operating during ground icing conditions,

(i.e., any time conditions are such that frost, ice, or snow may reasonably

be expected to adhere to the airplane), if the certificate holder expects to

authorize takeoffs in ground icing conditions, including:

(i) The use of holdover times when using deicing/anti-icing fluids.

(ii) Airplane deicing/anti-icing procedures, including inspection and check

procedures and responsibilities.

(iii) Communications.

(iv) Airplane surface contamination (i.e., adherence of frost, ice, or

snow) and critical area identification, and knowledge of how contamination

adversely affects airplane performance and flight characteristics.

(v) Types and characteristics of deicing/anti-icing fluids, if used by the

certificate holder.

(vi) Cold weather preflight inspection procedures.

(vii) Techniques for recognizing contamination on the airplane.

(b) No certificate holder may use any person, nor may any person serve, as

a pilot in any airplane unless, since the beginning of the 12th calendar

month before that service, that person has passed a competency check given by

the Administrator or an authorized check airman in that type of airplane to

determine that person's competence in practical skills and techniques in that

airplane or type of airplane. The extent of the competency check shall be

determined by the Administrator or authorized check airman conducting the

competency check. The competency check may include any of the maneuvers and

procedures currently required for the original issuance of the particular

pilot certificate required for the operations authorized and appropriate to

the category, class, and type of airplane involved. For the purposes of this

paragraph, type, as to an airplane, means any one of a group of airplanes

determined by the Administrator to have a similar means of propulsion, the

same manufacturer, and no significantly different handling or flight

characteristics.

(c) The instrument proficiency check required by Sec. 125.291 may be

substituted for the competency check required by this section for the type of

airplane used in the check.

(d) For the purposes of this part, competent performance of a procedure or

maneuver by a person to be used as a pilot requires that the pilot be the

obvious master of the airplane with the successful outcome of the maneuver

never in doubt.

(e) The Administrator or authorized check airman certifies the competency

of each pilot who passes the knowledge or flight check in the certificate

holder's pilot records.

(f) Portions of a required competency check may be given in an airplane

simulator or other appropriate training device, if approved by the

Administrator.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-18, 58 FR

69629, Dec. 30, 1993]

*****************************************************************************

58 FR 69620, No. 249, Dec. 30, 1993

SUMMARY: This interim final rule requires part 125 certificate holders to

provide pilot testing on conducting operations in ground icing conditions,

part 135 certificate holders to provide pilot training on conducting

operations in ground icing conditions, and part 125 and 135 certificate

holders to check airplanes for contamination (i.e., frost, ice, or snow)

prior to takeoff when ground icing conditions exist. This rule is necessary

because accident statistics and experience indicate the importance of

effectively determining whether the airplane's wings and control surfaces are

free of all frost, ice, or snow prior to beginning a takeoff. The rule is

intended to provide an added level of safety to flight operations in ground

icing conditions under parts 125 and 135.

DATES: This interim final rule is effective January 31, 1994.

*****************************************************************************

Sec. 125.289  Initial and recurrent flight attendant crewmember testing

requirements.

No certificate holder may use any person, nor may any person serve, as a

flight attendant crewmember, unless, since the beginning of the 12th calendar

month before that service, the certificate holder has determined by

appropriate initial and recurrent testing that the person is knowledgeable

and competent in the following areas as appropriate to assigned duties and

responsibilities:

(a) Authority of the pilot in command;

(b) Passenger handling, including procedures to be followed in handling

deranged persons or other persons whose conduct might jeopardize safety;

(c) Crewmember assignments, functions, and responsibilities during ditching

and evacuation of persons who may need the assistance of another person to

move expeditiously to an exit in an emergency;

(d) Briefing of passengers;

(e) Location and operation of portable fire extinguishers and other items

of emergency equipment;

(f) Proper use of cabin equipment and controls;

(g) Location and operation of passenger oxygen equipment;

(h) Location and operation of all normal and emergency exits, including

evacuation chutes and escape ropes; and

(i) Seating of persons who may need assistance of another person to move

rapidly to an exit in an emergency as prescribed by the certificate holder's

operations manual.

Sec. 125.291    Pilot in command: Instrument proficiency check requirements.

(a) No certificate holder may use any person, nor may any person serve, as

a pilot in command of an airplane under IFR unless, since the beginning of

the sixth calendar month before that service, that person has passed an

instrument proficiency check and the Administrator or an authorized check

airman has so certified in a letter of competency.

(b) No pilot may use any type of precision instrument approach procedure

under IFR unless, since the beginning of the sixth calendar month before that

use, the pilot has satisfactorily demonstrated that type of approach

procedure and has been issued a letter of competency under paragraph (g) of

this section. No pilot may use any type of nonprecision approach procedure

under IFR unless, since the beginning of the sixth calendar month before that

use, the pilot has satisfactorily demonstrated either that type of approach

procedure or any other two different types of nonprecision approach

procedures and has been issued a letter of competency under paragraph (g) of

this section. The instrument approach procedure or procedures must include at

least one straight-in approach, one circling approach, and one missed

approach. Each type of approach procedure demonstrated must be conducted to

published minimums for that procedure.

(c) The instrument proficiency check required by paragraph (a) of this

section consists of an oral or written equipment test and a flight check

under simulated or actual IFR conditions. The equipment test includes

questions on emergency procedures, engine operation, fuel and lubrication

systems, power settings, stall speeds, best engine-out speed, propeller and

supercharge operations, and hydraulic, mechanical, and electrical systems, as

appropriate. The flight check includes navigation by instruments, recovery

from simulated emergencies, and standard instrument approaches involving

navigational facilities which that pilot is to be authorized to use.

(1) For a pilot in command of an airplane, the instrument proficiency check

must include the procedures and maneuvers for a commercial pilot certificate

with an instrument rating and, if required, for the appropriate type rating.

(2) The instrument proficiency check must be given by an authorized check

airman or by the Administrator.

(d) If the pilot in command is assigned to pilot only one type of airplane,

that pilot must take the instrument proficiency check required by paragraph

(a) of this section in that type of airplane.

(e) If the pilot in command is assigned to pilot more than one type of

airplane, that pilot must take the instrument proficiency check required by

paragraph (a) of this section in each type of airplane to which that pilot is

assigned, in rotation, but not more than one flight check during each period

described in paragraph (a) of this section.

(f) Portions of a required flight check may be given in an airplane

simulator or other appropriate training device, if approved by the

Administrator.

(g) The Administrator or authorized check airman issues a letter of

competency to each pilot who passes the instrument proficiency check. The

letter of competency contains a list of the types of instrument approach

procedures and facilities authorized.

Sec. 125.293   Crewmember: Tests and checks, grace provisions, accepted

standards.

(a) If a crewmember who is required to take a test or a flight check under

this part completes the test or flight check in the calendar month before or

after the calendar month in which it is required, that crewmember is

considered to have completed the test or check in the calendar month in which

it is required.

(b) If a pilot being checked under this subpart fails any of the required

maneuvers, the person giving the check may give additional training to the

pilot during the course of the check. In addition to repeating the maneuvers

failed, the person giving the check may require the pilot being checked to

repeat any other maneuvers that are necessary to determine the pilot's

proficiency. If the pilot being checked is unable to demonstrate satisfactory

performance to the person conducting the check, the certificate holder may

not use the pilot, nor may the pilot serve, in the capacity for which the

pilot is being checked in operations under this part until the pilot has

satisfactorily completed the check.

Sec. 125.295   Check airman authorization: Application and issue.

Each certificate holder desiring FAA approval of a check airman shall

submit a request in writing to the FAA Flight Standards district office

charged with the overall inspection of the certificate holder. The

Administrator may issue a letter of authority to each check airman if that

airman passes the appropriate oral and flight test. The letter of authority

lists the tests and checks in this part that the check airman is qualified to

give, and the category, class and type airplane, where appropriate, for which

the check airman is qualified.

Sec. 125.297   Approval of airplane simulators and other training devices.

(a) Airplane simulators and other training devices approved by the

Administrator may be used in checks required in this subpart.

(b) Each airplane simulator and other training device that is used in

checks required under this subpart must meet the following requirements:

(1) It must be specifically approved for--

(i) The certificate holder;

(ii) The type airplane and, if applicable, the particular variation within

type for which the check is being conducted; and

(iii) The particular maneuver, procedure, or crewmember function involved.

(2) It must maintain the performance, functional, and other characteristics

that are required for approval.

(3) It must be modified to conform with any modification to the airplane

being simulated that changes the performance, functional, or other

characteristics required for approval.

Subpart J--Flight Operations

Sec. 125.311  Flight crewmembers at controls.

(a) Except as provided in paragraph (b) of this section, each required

flight crewmember on flight deck duty must remain at the assigned duty

station with seat belt fastened while the airplane is taking off or landing

and while it is en route.

(b) A required flight crewmember may leave the assigned duty station--

(1) If the crewmember's absence is necessary for the performance of duties

in connection with the operation of the airplane;

(2) If the crewmember's absence is in connection with physiological needs;

or

(3) If the crewmember is taking a rest period and relief is provided--

(i) In the case of the assigned pilot in command, by a pilot qualified to

act as pilot in command.

(ii) In the case of the assigned second in command, by a pilot qualified to

act as second in command of that airplane during en route operations.

However, the relief pilot need not meet the recent experience requirements of

Sec. 125.285.

Sec. 125.313  Manipulation of controls when carrying passengers.

No pilot in command may allow any person to manipulate the controls of an

airplane while carrying passengers during flight, nor may any person

manipulate the controls while carrying passengers during flight, unless that

person is a qualified pilot of the certificate holder operating that

airplane.

Sec. 125.315  Admission to flight deck.

(a) No person may admit any person to the flight deck of an airplane unless

the person being admitted is--

(1) A crewmember;

(2) An FAA inspector or an authorized representative of the National

Transportation Safety Board who is performing official duties; or

(3) Any person who has the permission of the pilot in command.

(b) No person may admit any person to the flight deck unless there is a

seat available for the use of that person in the passenger compartment,

except--

(1) An FAA inspector or an authorized representative of the Administrator

or National Transportation Safety Board who is checking or observing flight

operations; or

(2) A certificated airman employed by the certificate holder whose duties

require an airman certificate.

Sec. 125.317  Inspector's credentials: Admission to pilots' compartment:

Forward observer's seat.

(a) Whenever, in performing the duties of conducting an inspection, an FAA

inspector presents an Aviation Safety Inspector credential, FAA Form 110A, to

the pilot in command of an airplane operated by the certificate holder, the

inspector must be given free and uninterrupted access to the pilot

compartment of that airplane. However, this paragraph does not limit the

emergency authority of the pilot in command to exclude any person from the

pilot compartment in the interest of safety.

(b) A forward observer's seat on the flight deck, or forward passenger seat

with headset or speaker, must be provided for use by the Administrator while

conducting en route inspections. The suitability of the location of the seat

and the headset or speaker for use in conducting en route inspections is

determined by the Administrator.

Sec. 125.319  Emergencies.

(a) In an emergency situation that requires immediate decision and action,

the pilot in command may take any action considered necessary under the

circumstances. In such a case, the pilot in command may deviate from

prescribed operations, procedures and methods, weather minimums, and this

chapter, to the extent required in the interests of safety.

(b) In an emergency situation arising during flight that requires immediate

decision and action by appropriate management personnel in the case of

operations conducted with a flight following service and which is known to

them, those personnel shall advise the pilot in command of the emergency,

shall ascertain the decision of the pilot in command, and shall have the

decision recorded. If they cannot communicate with the pilot, they shall

declare an emergency and take any action that they consider necessary under

the circumstances.

(c) Whenever emergency authority is exercised, the pilot in command or the

appropriate management personnel shall keep the appropriate ground radio

station fully informed of the progress of the flight. The person declaring

the emergency shall send a written report of any deviation, through the

operator's director of operations, to the Administrator within 10 days,

exclusive of Saturdays, Sundays, and Federal holidays, after the flight is

completed or, in the case of operations outside the United States, upon

return to the home base.

Sec. 125.321  Reporting potentially hazardous meteorological conditions and

irregularities of ground and navigation facilities.

Whenever the pilot in command encounters a meteorological condition or an

irregularity in a ground or navigational facility in flight, the knowledge of

which the pilot in command considers essential to the safety of other

flights, the pilot in command shall notify an appropriate ground station as

soon as practicable.

Sec. 125.323  Reporting mechanical irregularities.

The pilot in command shall ensure that all mechanical irregularities

occurring during flight are entered in the maintenance log of the airplane at

the next place of landing. Before each flight, the pilot in command shall

ascertain the status of each irregularity entered in the log at the end of

the preceding flight.

Sec. 125.325  Instrument approach procedures and IFR landing minimums.

No person may make an instrument approach at an airport except in

accordance with IFR weather minimums and unless the type of instrument

approach procedure to be used is listed in the certificate holder's

operations specifications.

Sec. 125.327  Briefing of passengers before flight.

(a) Before each takeoff, each pilot in command of an airplane carrying

passengers shall ensure that all passengers have been orally briefed on--

(1) Smoking. Each passenger shall be briefed on when, where, and under what

conditions smoking is prohibited. This briefing shall include a statement

that the Federal Aviation Regulations require passenger compliance with the

lighted passenger information signs, posted placards, areas designated for

safety purposes as no smoking areas, and crewmember instructions with regard

to these items.

(2) The use of safety belts, including instructions on how to fasten and

unfasten the safety belts. Each passenger shall be briefed on when, where,

and under what conditions the safety belt must be fastened about him or her.

This briefing shall include a statement that the Federal Aviation Regulations

require passenger compliance with lighted passenger information signs and

crewmember instructions concerning the use of safety belts.

(3) The placement of seat backs in an upright position before takeoff and

landing;

(4) Location and means for opening the passenger entry door and emergency

exits;

(5) Location of survival equipment;

(6) If the flight involves extended overwater operation, ditching

procedures and the use of required flotation equipment;

(7) If the flight involves operations above 12,000 feet MSL, the normal and

emergency use of oxygen; and

(8) Location and operation of fire extinguishers.

(b) Before each takeoff, the pilot in command shall ensure that each person

who may need the assistance of another person to move expeditiously to an

exit if an emergency occurs and that person's attendant, if any, has received

a briefing as to the procedures to be followed if an evacuation occurs. This

paragraph does not apply to a person who has been given a briefing before a

previous leg of a flight in the same airplane.

(c) The oral briefing required by paragraph (a) of this section shall be

given by the pilot in command or a member of the crew. It shall be

supplemented by printed cards for the use of each passenger containing--

(1) A diagram and method of operating the emergency exits; and

(2) Other instructions necessary for the use of emergency equipment on

board the airplane.

Each card used under this paragraph must be carried in the airplane in

locations convenient for the use of each passenger and must contain

information that is appropriate to the airplane on which it is to be used.

(d) The certificate holder shall describe in its manual the procedure to be

followed in the briefing required by paragraph (a) of this section.

(e) If the airplane does not proceed directly over water after takeoff, no

part of the briefing required by paragraph (a)(6) of this section has to be

given before takeoff but the briefing required by paragraph (a)(6) must be

given before reaching the overwater part of the flight.

[Dkt. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-17, 57

FR 42675, Sept. 15, 1992]

*****************************************************************************

57 FR 42662, No. 179, Sept. 15, 1992

SUMMARY: This final rule amends the Federal Aviation Regulations by requiring

operators and certificate holders to allow the use of approved child

restraint systems and by updating certain regulations concerning passenger

and crewmember safety, attitude indicators, and check airmen. This action is

in response to requests from the public, consumer groups, and Congress;

reports from FAA inspectors; and investigations and recommendations by the

National Transportation Safety Board. The rule is intended to increase the

safety of crewmembers and passengers on board aircraft and to update other

operational amendments.

EFFECTIVE DATE: October 15, 1992.

*****************************************************************************

Sec. 125.329   Minimum altitudes for use of autopilot.

(a) Except as provided in paragraphs (b), (c), and (d) of this section, no

person may use an autopilot at an altitude above the terrain which is less

than 500 feet or less than twice the maximum altitude loss specified in the

approved Airplane Flight Manual or equivalent for a malfunction of the

autopilot, whichever is higher.

(b) When using an instrument approach facility other than ILS, no person

may use an autopilot at an altitude above the terrain that is less than 50

feet below the approved minimum descent altitude for that procedure, or less

than twice the maximum loss specified in the approved Airplane Flight Manual

or equivalent for a malfunction of the autopilot under approach conditions,

whichever is higher.

(c) For ILS approaches when reported weather conditions are less than the

basic weather conditions in Sec. 91.155 of this chapter, no person may use an

autopilot with an approach coupler at an altitude above the terrain that is

less than 50 feet above the terrain, or the maximum altitude loss specified

in the approved Airplane Flight Manual or equivalent for the malfunction of

the autopilot with approach coupler, whichever is higher.

(d) Without regard to paragraph (a), (b), or (c) of this section, the

Administrator may issue operations specifications to allow the use, to

touchdown, of an approved flight control guidance system with automatic

capability, if--

(1) The system does not contain any altitude loss (above zero) specified in

the approved Airplane Flight Manual or equivalent for malfunction of the

autopilot with approach coupler; and

(2) The Administrator finds that the use of the system to touchdown will

not otherwise adversely affect the safety standards of this section.

[Doc. No. 19779, 45 FR 67325, Oct. 9, 1980, as amended by Amdt. 125-12, 54 FR

34332, Aug. 18, 1989]

Effective Date Note: At 54 FR 34332, August 18, 1989, Sec. 125.329(c) was

amended by changing the cross reference "Sec. 91.105" to read "Sec. 91.155",

effective August 18, 1990.

Sec. 125.331   Carriage of persons without compliance with the passenger-

carrying provisions of this part.

The following persons may be carried aboard an airplane without complying

with the passenger-carrying requirements of this part:

(a) A crewmember.

(b) A person necessary for the safe handling of animals on the airplane.

(c) A person necessary for the safe handling of hazardous materials (as

defined in Subchapter C of Title 49 CFR).

(d) A person performing duty as a security or honor guard accompanying a

shipment made by or under the authority of the U.S. Government.

(e) A military courier or a military route supervisor carried by a military

cargo contract operator if that carriage is specifically authorized by the

appropriate military service.

(f) An authorized representative of the Administrator conducting an en

route inspection.

(g) A person authorized by the Administrator.

Sec. 125.333  Stowage of food, beverage, and passenger service equipment

during airplane movement on the surface, takeoff, and landing.

(a) No certificate holder may move an airplane on the surface, take off, or

land when any food, beverage, or tableware furnished by the certificate

holder is located at any passenger seat.

(b) No certificate holder may move an airplane on the surface, take off, or

land unless each food and beverage tray and seat back tray table is secured

in its stowed position.

(c) No certificate holder may permit an airplane to move on the surface,

take off, or land unless each passenger serving cart is secured in its stowed

position.

(d) Each passenger shall comply with instructions given by a crewmember

with regard to compliance with this section.

[Amdt. 125-17, 57 FR 42675, Sept. 15, 1992]

*****************************************************************************

57 FR 42662, No. 179, Sept. 15, 1992

SUMMARY: This final rule amends the Federal Aviation Regulations by requiring

operators and certificate holders to allow the use of approved child

restraint systems and by updating certain regulations concerning passenger

and crewmember safety, attitude indicators, and check airmen. This action is

in response to requests from the public, consumer groups, and Congress;

reports from FAA inspectors; and investigations and recommendations by the

National Transportation Safety Board. The rule is intended to increase the

safety of crewmembers and passengers on board aircraft and to update other

operational amendments.

EFFECTIVE DATE: October 15, 1992.

*****************************************************************************

Subpart K--Flight Release Rules

Sec. 125.351  Flight release authority.

(a) No person may start a flight without authority from the person

authorized by the certificate holder to exercise operational control over the

flight.

(b) No person may start a flight unless the pilot in command or the person

authorized by the cetificate holder to exercise operational control over the

flight has executed a flight release setting forth the conditions under which

the flight will be conducted. The pilot in command may sign the flight

release only when both the pilot in command and the person authorized to

exercise operational control believe the flight can be made safely, unless

the pilot in command is authorized by the certificate holder to exercise

operational control and execute the flight release without the approval of

any other person.

(c) No person may continue a flight from an intermediate airport without a

new flight release if the airplane has been on the ground more than 6 hours.

Sec. 125.353  Facilities and services.

During a flight, the pilot in command shall obtain any additional available

information of meteorological conditions and irregularities of facilities and

services that may affect the safety of the flight.

Sec. 125.355  Airplane equipment.

No person may release an airplane unless it is airworthy and is equipped as

prescribed.

Sec. 125.357  Communication and navigation facilities.

No person may release an airplane over any route or route segment unless

communication and navigation facilities equal to those required by Sec.

125.51 are in satisfactory operating condition.

Sec. 125.359   Flight release under VFR.

No person may release an airplane for VFR operation unless the ceiling and

visibility en route, as indicated by available weather reports or forecasts,

or any combination thereof, are and will remain at or above applicable VFR

minimums until the airplane arrives at the airport or airports specified in

the flight release.

Sec. 125.361  Flight release under IFR or over-the-top.

Except as provided in Sec. 125.363, no person may release an airplane for

operations under IFR or over-the-top unless appropriate weather reports or

forecasts, or any combination thereof, indicate that the weather conditions

will be at or above the authorized minimums at the estimated time of arrival

at the airport or airports to which released.

Sec. 125.363  Flight release over water.

(a) No person may release an airplane for a flight that involves extended

overwater operation unless appropriate weather reports or forecasts, or any

combination thereof, indicate that the weather conditions will be at or above

the authorized minimums at the estimated time of arrival at any airport to

which released or to any required alternate airport.

(b) Each certificate holder shall conduct extended overwater operations

under IFR unless it shows that operating under IFR is not necessary for

safety.

(c) Each certificate holder shall conduct other overwater operations under

IFR if the Administrator determines that operation under IFR is necessary for

safety.

(d) Each authorization to conduct extended overwater operations under VFR

and each requirement to conduct other overwater operations under IFR will be

specified in the operations specifications.

Sec. 125.365  Alternate airport for departure.

(a) If the weather conditions at the airport of takeoff are below the

landing minimums in the certificate holder's operations specifications for

that airport, no person may release an airplane from that airport unless the

flight release specifies an alternate airport located within the following

distances from the airport of takeoff:

(1) Airplanes having two engines. Not more than 1 hour from the departure

airport at normal cruising speed in still air with one engine inoperative.

(2) Airplanes having three or more engines. Not more than 2 hours from the

departure airport at normal cruising speed in still air with one engine

inoperative.

(b) For the purposes of paragraph (a) of this section, the alternate

airport weather conditions must meet the requirements of the certificate

holder's operations specifications.

(c) No person may release an airplane from an airport unless that person

lists each required alternate airport in the flight release.

Sec. 125.367  Alternate airport for destination: IFR or over-the-top.

(a) Except as provided in paragraph (b) of this section, each person

releasing an airplane for operation under IFR or over-the-top shall list at

least one alternate airport for each destination airport in the flight

release.

(b) An alternate airport need not be designated for IFR or over-the-top

operations where the airplane carries enough fuel to meet the requirements of

Secs. 125.375 and 125.377 for flights outside the 48 conterminous States and

the District of Columbia over routes without an available alternate airport

for a particular airport of destination.

(c) For the purposes of paragraph (a) of this section, the weather

requirements at the alternate airport must meet the requirements of the

operator's operations specifications.

(d) No person may release a flight unless that person lists each required

alternate airport in the flight release.

Sec. 125.369  Alternate airport weather minimums.

No person may list an airport as an alternate airport in the flight release

unless the appropriate weather reports or forecasts, or any combination

thereof, indicate that the weather conditions will be at or above the

alternate weather minimums specified in the certificate holder's operations

specifications for that airport when the flight arrives.

Sec. 125.371  Continuing flight in unsafe conditions.

(a) No pilot in command may allow a flight to continue toward any airport

to which it has been released if, in the opinion of the pilot in command, the

flight cannot be completed safely, unless, in the opinion of the pilot in

command, there is no safer procedure. In that event, continuation toward that

airport is an emergency situation.

Sec. 125.373   Original flight release or amendment of flight release.

(a) A certificate holder may specify any airport authorized for the type of

airplane as a destination for the purpose of original release.

(b) No person may allow a flight to continue to an airport to which it has

been released unless the weather conditions at an alternate airport that was

specified in the flight release are forecast to be at or above the alternate

minimums specified in the operations specifications for that airport at the

time the airplane would arrive at the alternate airport. However, the flight

release may be amended en route to include any alternate airport that is

within the fuel range of the airplane as specified in Sec. 125.375 or Sec.

125.377.

(c) No person may change an original destination or alternate airport that

is specified in the original flight release to another airport while the

airplane is en route unless the other airport is authorized for that type of

airplane.

(d) Each person who amends a flight release en route shall record that

amendment.

Sec. 125.375   Fuel supply: Nonturbine and turbopropeller-powered airplanes.

(a) Except as provided in paragraph (b) of this section, no person may

release for flight or take off a nonturbine or turbopropeller-powered

airplane unless, considering the wind and other weather conditions expected,

it has enough fuel--

(1) To fly to and land at the airport to which it is released;

(2) Thereafter, to fly to and land at the most distant alternate airport

specified in the flight release; and

(3) Thereafter, to fly for 45 minutes at normal crusing fuel consumption.

(b) If the airplane is released for any flight other than from one point in

the conterminous United States to another point in the conterminous United

States, it must carry enough fuel to meet the requirements of paragraphs (a)

(1) and (2) of this section and thereafter fly for 30 minutes plus 15 percent

of the total time required to fly at normal cruising fuel consumption to the

airports specified in paragraphs (a) (1) and (2) of this section, or fly for

90 minutes at normal cruising fuel consumption, whichever is less.

(c) No person may release a nonturbine or turbopropeller-powered airplane

to an airport for which an alternate is not specified under Sec. 125.367(b)

unless it has enough fuel, considering wind and other weather conditions

expected, to fly to that airport and thereafter to fly for 3 hours at normal

cruising fuel consumption.

Sec. 125.377  Fuel supply: Turbine-engine-powered airplanes other than turbo-

propeller.

(a) Except as provided in paragraph (b) of this section, no person may

release for flight or takeoff a turbine-powered airplane (other than a

turbopropeller-powered airplane) unless, considering the wind and other

weather conditions expected, it has enough fuel--

(1) To fly to and land at the airport to which it is released;

(2) Thereafter, to fly to and land at the most distant alternate airport

specified in the flight release; and

(3) Thereafter, to fly for 45 minutes at normal cruising fuel consumption.

(b) For any operation outside the 48 conterminous United States and the

District of Columbia, unless authorized by the Administrator in the

operations specifications, no person may release for flight or take off a

turbine-engine powered airplane (other than a turbopropeller-powered

airplane) unless, considering wind and other weather conditions expected, it

has enough fuel--

(1) To fly and land at the airport to which it is released;

(2) After that, to fly for a period of 10 percent of the total time

required to fly from the airport of departure and land at the airport to

which it was released;

(3) After that, to fly to and land at the most distant alternate airport

specified in the flight release, if an alternate is required; and

(4) After that, to fly for 30 minutes at holding speed at 1,500 feet above

the alternate airport (or the destination airport if no alternate is

required) under standard temperature conditions.

(c) No person may release a turbine-engine-powered airplane (other than a

turbopropeller airplane) to an airport for which an alternate is not

specified under Sec. 125.367(b) unless it has enough fuel, considering wind

and other weather conditions expected, to fly to that airport and thereafter

to fly for at least 2 hours at normal cruising fuel consumption.

(d) The Administrator may amend the operations specifications of a

certificate holder to require more fuel than any of the minimums stated in

paragraph (a) or (b) of this section if the Administrator finds that

additional fuel is necessary on a particular route in the interest of safety.

Sec. 125.379   Landing weather minimums: IFR.

(a) If the pilot in command of an airplane has not served 100 hours as

pilot in command in the type of airplane being operated, the MDA or DH and

visibility landing minimums in the certificate holder's operations

specification are increased by 100 feet and one-half mile (or the RVR

equivalent). The MDA or DH and visibility minimums need not be increased

above those applicable to the airport when used as an alternate airport, but

in no event may the landing minimums be less than a 300-foot ceiling and 1

mile of visibility.

(b) The 100 hours of pilot-in-command experience required by paragraph (a)

may be reduced (not to exceed 50 percent) by substituting one landing in

operations under this part in the type of airplane for 1 required hour of

pilot-in-command experience if the pilot has at least 100 hours as pilot in

command of another type airplane in operations under this part.

(c) Category II minimums, when authorized in the certificate holder's

operations specifications, do not apply until the pilot in command subject to

paragraph (a) of this section meets the requirements of that paragraph in the

type of airplane the pilot is operating.

Sec. 125.381   Takeoff and landing weather minimums: IFR.

(a) Regardless of any clearance from ATC, if the reported weather

conditions are less than that specified in the certificate holder's

operations specifications, no pilot may--

(1) Take off an airplane under IFR; or

(2) Except as provided in paragraph (c) of this section, land an airplane

under IFR.

(b) Except as provided in paragraph (c) of this section, no pilot may

execute an instrument approach procedure if the latest reported visibility is

less than the landing minimums specified in the certificate holder's

operations specifications.

(c) If a pilot initiates an instrument approach procedure when the latest

weather report indicates that the specified visibility minimums exist, and a

later weather report indicating below minimums conditions is received after

the airplane--

(1) Is on an ILS final approach and has passed the outer marker,

(2) Is on final approach segment using a nonprecision approach procedure,

or

(3) Is on PAR final approach and has been turned over to the final approach

controller, the approach may be continued and a landing may be made if the

pilot in command finds, upon reaching the authorized MAP or DH, that actual

weather conditions are at least equal to the minimums prescribed in the

operations specifications.

[Doc. No. 19779, 45 FR 67235, Oct. 9, 1980, as amended by Amdt. 125-2, 46 FR

24409, Apr. 30, 1981]

Sec. 125.383   Load manifest.

(a) Each certificate holder is responsible for the preparation and accuracy

of a load manifest in duplicate containing information concerning the loading

of the airplane. The manifest must be prepared before each takeoff and must

include--

(1) The number of passengers;

(2) The total weight of the loaded airplane;

(3) The maximum allowable takeoff and landing weights for that flight;

(4) The center of gravity limits;

(5) The center of gravity of the loaded airplane, except that the actual

center of gravity need not be computed if the airplane is loaded according to

a loading schedule or other approved method that ensures that the center of

gravity of the loaded airplane is within approved limits. In those cases, an

entry shall be made on the manifest indicating that the center of gravity is

within limits according to a loading schedule or other approved method:

(6) The registration number of the airplane;

(7) The origin and destination ; and

(8) Names of passengers.

(b) The pilot in command of an airplane for which a load manifest must be

prepared shall carry a copy of the completed load manifest in the airplane to

its destination. The certificate holder shall keep copies of completed load

manifests for at least 30 days at its principal operations base, or at

another location used by it and approved by the Administrator.

Subpart L--Records and Reports

Sec. 125.401  Crewmember record.

(a) Each certificate holder shall--

(1) Maintain current records of each crewmember that show whether or not

that crewmember complies with this chapter (e.g., proficiency checks,

airplane qualifications, any required physical examinations, and flight time

records); and

(2) Record each action taken concerning the release from employment or

physical or professional disqualification of any flight crewmember and keep

the record for at least 6 months thereafter.

(b) Each certificate holder shall maintain the records required by

paragraph (a) of this section at its principal operations base, or at another

location used by it and approved by the Administrator.

(c) Computer record systems approved by the Administrator may be used in

complying with the requirements of paragraph (a) of this section.

Sec. 125.403  Flight release form.

(a) The flight release may be in any form but must contain at least the

following information concerning each flight:

(1) Company or organization name.

(2) Make, model, and registration number of the airplane being used.

(3) Date of flight.

(4) Name and duty assignment of each crewmember.

(5) Departure airport, destination airports, alternate airports, and route.

(6) Minimum fuel supply (in gallons or pounds).

(7) A statement of the type of operation (e.g., IFR, VFR).

(b) The airplane flight release must contain, or have attached to it,

weather reports, available weather forecasts, or a combination thereof.

Sec. 125.405  Disposition of load manifest, flight release, and flight plans.

(a) The pilot in command of an airplane shall carry in the airplane to its

destination the original or a signed copy of the--

(1) Load manifest required by Sec. 125.383;

(2) Flight release;

(3) Airworthiness release; and

(4) Flight plan, including route.

(b) If a flight originates at the principal operations base of the

certificate holder, it shall retain at that base a signed copy of each

document listed in paragraph (a) of this section.

(c) Except as provided in paragraph (d) of this section, if a flight

originates at a place other than the principal operations base of the

certificate holder, the pilot in command (or another person not aboard the

airplane who is authorized by the operator) shall, before or immediately

after departure of the flight, mail signed copies of the documents listed in

paragraph (a) of this section to the principal operations base.

(d) If a flight originates at a place other than the principal operations

base of the certificate holder and there is at that place a person to manage

the flight departure for the operator who does not depart on the airplane,

signed copies of the documents listed in paragraph (a) of this section may be

retained at that place for not more than 30 days before being sent to the

principal operations base of the certificate holder. However, the documents

for a particular flight need not be further retained at that place or be sent

to the principal operations base, if the originals or other copies of them

have been previously returned to the principal operations base.

(e) The certificate holder shall:

(1) Identify in its operations manual the person having custody of the

copies of documents retained in accordance with paragraph (d) of this

section; and

(2) Retain at its principal operations base either the original or a copy

of the records required by this section for at least 30 days.

Sec. 125.407  Maintenance log: Airplanes.

(a) Each person who takes corrective action or defers action concerning a

reported or observed failure or malfunction of an airframe, aircraft engine,

propeller, or appliance shall record the action taken in the airplane

maintenance log in accordance with Part 43 of this chapter.

(b) Each certificate holder shall establish a procedure for keeping copies

of the airplane maintenance log required by this section in the airplane for

access by appropriate personnel and shall include that procedure in the

manual required by Sec. 125.249.

Sec. 125.409   Reports of defects or unairworthy conditions.

(a) Each certificate holder shall report the occurrence or detection of

each failure, malfunction, or defect, in a form and manner prescribed by the

Administrator.

(b) The report must be made within 72 hours to the FAA Flight Standards

district office in whose area the certificate holder has its principal

operations base. The procedures to be used in complying with this section

must be made a part of the manual procedures required by Sec. 125.73(f).

Sec. 125.411  Airworthiness release or maintenance record entry.

(a) No certificate holder may operate an airplane after maintenance,

preventive maintenance, or alteration is performed on the airplane unless the

person performing that maintenance, preventive maintenance, or alteration

prepares or causes to be prepared--

(1) An airworthiness release; or

(2) An entry in the aircraft maintenance records in accordance with the

certificate holder's manual.

(b) The airworthiness release or maintenance record entry required by

paragraph (a) of this section must--

(1) Be prepared in accordance with the procedures set forth in the

certificate holder's manual;

(2) Include a certification that--

(i) The work was performed in accordance with the requirements of the

certificate holder's manual;

(ii) All items required to be inspected were inspected by an authorized

person who determined that the work was satisfactorily completed;

(iii) No known condition exists that would make the airplane unairworthy;

and

(iv) So far as the work performed is concerned, the airplane is in

condition for safe operation; and

(3) Be signed by a person authorized in Part 43 of this chapter to perform

maintenance, preventive maintenance, and alteration.

(c) When an airworthiness release form is prepared, the certificate holder

must give a copy to the pilot in command and keep a record of it for at least

60 days.

(d) Instead of restating each of the conditions of the certification

required by paragraph (b) of this section, the certificate holder may state

in its manual that the signature of a person authorized in Part 43 of this

chapter constitutes that certification.