PART 147--AVIATION MAINTENANCE TECHNICIAN SCHOOLS

Subpart A--General

Sec.

147.1  Applicability.

147.3  Certificate required.

147.5  Application and issue.

147.7  Duration of certificates.

Subpart B--Certification Requirements

147.11  Ratings.

147.13  Facilities, equipment, and material requirements.

147.15  Space requirements.

147.17  Instructional equipment requirements.

147.19  Material, tool, and shop equipment requirements.

147.21  General curriculum requirements.

147.23  Instructor requirements.

Subpart C--Operating Rules

147.31  Attendance and enrollment, tests and credit for prior instruction or

experience.

147.33  Records.

147.35  Transcripts and graduation certificates.

147.36  Maintenance of instructor requirements.

147.37  Maintenance of facilities, equipment and material.

147.38  Maintenance of curriculum requirements.

147.38a  Quality of instruction.

147.39  Display of certificate.

147.41  Change of location.

147.43  Inspection.

147.45  Advertising.

Appendix A to Part 147--Curriculum Requirements

Appendix B to Part 147--General Curriculum Subjects

Appendix C to Part 147--Airframe Curriculum Subjects

Appendix D to Part 147--Powerplant Curriculum Subjects

Authority: Secs. 313(a), 314, 601, and 607, 72 Stat. 752; 49 U.S.C.

1354(a), 1355, 1421, and 1427.

Source: Docket No. 1157, 27 FR 6669 July 13, 1962, unless otherwise noted.

Subpart A--General

Sec. 147.1   Applicability.

This part prescribes the requirements for issuing aviation maintenance

technician school certificates and associated ratings and the general

operating rules for the holders of those certificates and ratings.

Sec. 147.3   Certificate required.

No person may operate as a certificated aviation maintenance technician

school without, or in violation of, an aviation maintenance technician school

certificate issued under this part.

[Doc. No. 15196, 41 FR 47230, Oct. 28, 1976]

Sec. 147.5   Application and issue.

(a) An application for a certificate and rating, or for an additional

rating, under this part is made on a form and in a manner prescribed by the

Administrator, and submitted with--

(1) A description of the proposed curriculum;

(2) A list of the facilities and materials to be used;

(3) A list of its instructors, including the kind of certificate and

ratings held and the certificate numbers; and

(4) A statement of the maximum number of students it expects to teach at

any one time.

(b) An applicant who meets the requirements of this part is entitled to an

aviation maintenance technician school certificate and associated ratings

prescribing such operations specifications and limitations as are necessary

in the interests of safety.

[Doc. No. 1157, 27 FR 6669, July 13, 1962, as amended by Amdt. 147-5, 57 FR

28959, June 29, 1992]

Sec. 147.7   Duration of certificates.

(a) An aviation maintenance technician school certificate or rating is

effective until it is surrendered, suspended, or revoked.

(b) The holder of a certificate that is surrendered, suspended, or revoked,

shall return it to the Administrator.

[Doc. No. 1157, 27 FR 6669, July 19, 1962, as amended by Amdt. 147-3, 41 FR

47230, Oct. 28, 1976]

Subpart B--Certification Requirements

Sec. 147.11   Ratings.

The following ratings are issued under this part:

(a) Airframe.

(b) Powerplant.

(c) Airframe and powerplant.

Sec. 147.13   Facilities, equipment, and material requirements.

An applicant for an aviation maintenance technician school certificate and

rating, or for an additional rating, must have at least the facilities,

equipment, and materials specified in Secs. 147.15 to 147.19 that are

appropriate to the rating he seeks.

Sec. 147.15   Space requirements.

An applicant for an aviation maintenance technician school certificate and

rating, or for an additional rating, must have such of the following properly

heated, lighted, and ventilated facilities as are appropriate to the rating

he seeks and as the Administrator determines are appropriate for the maximum

number of students expected to be taught at any time:

(a) An enclosed classroom suitable for teaching theory classes.

(b) Suitable facilities, either central or located in training areas,

arranged to assure proper separation from the working space, for parts,

tools, materials, and similar articles.

(c) Suitable area for application of finishing materials, including paint

spraying.

(d) Suitable areas equipped with washtank and degreasing equipment with air

pressure or other adequate cleaning equipment.

(e) Suitable facilities for running engines.

(f) Suitable area with adequate equipment, including benches, tables, and

test equipment, to disassemble, service, and inspect.

(1) Ignition, electrical equipment, and appliances;

(2) Carburetors and fuel systems; and

(3) Hydraulic and vacuum systems for aircraft, aircraft engines, and their

appliances.

(g) Suitable space with adequate equipment, including tables, benches,

stands, and jacks, for disassembling, inspecting, and rigging aircraft.

(h) Suitable space with adequate equipment for disassembling, inspecting,

assembling, troubleshooting, and timing engines.

[Amdt. 147-2, 35 FR 5533, Apr. 3, 1970, as amended by Amdt. 147-5, 57 FR

28959, June 29, 1992]

Sec. 147.17   Instructional equipment requirements.

(a) An applicant for a mechanic school certificate and rating, or for an

additional rating, must have such of the following instructional equipment as

is appropriate to the rating he seeks:

(1) Various kinds of airframe structures, airframe systems and components,

powerplants, and powerplant systems and components (including propellers), of

a quantity and type suitable to complete the practical projects required by

its approved curriculums.

(2) At least one aircraft of a type currently certificated by FAA for

private or commercial operation, with powerplant, propeller, instruments,

navigation and communications equipment, landing lights, and other equipment

and accessories on which a maintenance technician might be required to work

and with which the technician should be familiar.

(b) The equipment required by paragraph (a) of this section need not be in

an airworthy condition. However, if it was damaged, it must have been

repaired enough for complete assembly.

(c) Airframes, powerplants, propellers, appliances, and components thereof,

on which instruction is to be given, and from which practical working

experience is to be gained, must be so diversified as to show the different

methods of construction, assembly, inspection, and operation when installed

in an aircraft for use. There must be enough units so that not more than

eight students will work on any one unit at a time.

(d) If the aircraft used for instructional purposes does not have

retractable landing gear and wing flaps, the school must provide training

aids, or operational mock-ups of them.

[Doc. No. 1157, 27 FR 6669, July 19, 1962, as amended by Amdt. 147-5, 57 FR

28959, June 29, 1992]

Sec. 147.19  Materials, special tools, and shop equipment requirements.

An applicant for an aviation maintenance technician school certificate and

rating, or for an additional rating, must have an adequate supply of

material, special tools, and such of the shop equipment as are appropriate to

the approved curriculum of the school and are used in constructing and

maintaining aircraft, to assure that each student will be properly

instructed. The special tools and shop equipment must be in satisfactory

working condition for the purpose for which they are to be used.

[57 FR 28959, June 29, 1992]

Sec. 147.21   General curriculum requirements.

(a) An applicant for an aviation maintenance technician school certificate

and rating, or for an additional rating, must have an approved curriculum

that is designed to qualify his students to perform the duties of a mechanic

for a particular rating or ratings.

(b) The curriculum must offer at least the following number of hours of

instruction for the rating shown, and the instruction unit hour shall not be

less than 50 minutes in length--

(1) Airframe--1,150 hours (400 general plus 750 airframe).

(2) Powerplant--1,150 hours (400 general plus 750 powerplant).

(3) Combined airframe and powerplant--1,900 hours (400 general plus 750

airframe and 750 powerplant).

(c) The curriculum must cover the subjects and items prescribed in

appendixes B, C, or D, as applicable. Each item must be taught to at least

the indicated level of proficiency, as defined in appendix A.

(d) The curriculum must show--

(1) The required practical projects to be completed;

(2) For each subject, the proportions of theory and other instruction to be

given; and

(3) A list of the minimum required school tests to be given.

(e) Notwithstanding the provisions of paragraphs (a) through (d) of this

section and Sec. 147.11, the holder of a certificate issued under subpart B

of this part may apply for and receive approval of special courses in the

performance of special inspection and preventive maintenance programs for a

primary category aircraft type certificated under Sec. 21.24(b) of this

chapter. The school may also issue certificates of competency to persons

successfully completing such courses provided that all other requirements of

this part are met and the certificate of competency specifies the aircraft

make and model to which the certificate applies.

[Doc. No. 1157, 27 FR 6669, July 13, 1962 as amended by Amdt. 147-1, 32 FR

5770 Apr. 11, 1967; Amdt. 147-5, 57 FR 28959, June 29, 1992; Amdt. 147-6,

57 FR 41370, Sept. 9, 1992]

Sec. 147.23   Instructor requirements.

An applicant for an aviation maintenance technician school certificate and

rating, or for an additional rating, must provide the number of instructors

holding appropriate mechanic certificates and ratings that the Administrator

determines necessary to provide adequate instruction and supervision of the

students, including at least one such instructor for each 25 students in each

shop class. However, the applicant may provide specialized instructors, who

are not certificated mechanics, to teach mathematics, physics, basic

electricity, basic hydraulics, drawing, and similar subjects. The applicant

is required to maintain a list of the names and qualifications of specialized

instructors, and upon request, provide a copy of the list to the FAA.

[57 FR 28959, June 29, 1992]

Subpart C--Operating Rules

Sec. 147.31   Attendance and enrollment, tests, and credit for prior

instruction or experience.

(a) A certificated aviation maintenance technician school may not require

any student to attend classes of instruction more than 8 hours in any day or

more than 6 days or 40 hours in any 7-day period.

(b) Each school shall give an appropriate test to each student who

completes a unit of instruction as shown in that school's approved

curriculum.

(c) A school may not graduate a student unless he has completed all of the

appropriate curriculum requirements. However, the school may credit a student

with instruction or previous experience as follows:

(1) A school may credit a student with instruction satisfactorily completed

at--

(i) An accredited university, college, junior college;

(ii) An accredited vocational, technical, trade or high school;

(iii) A military technical school;

(iv) A certificated aviation maintenance technician school.

(2) A school may determine the amount of credit to be allowed--

(i) By an entrance test equal to one given to the students who complete a

comparable required curriculum subject at the crediting school;

(ii) By an evaluation of an authenticated transcript from the student's

former school; or

(iii) In the case of an applicant from a military school, only on the basis

of an entrance test.

(3) A school may credit a student with previous aviation maintenance

experience comparable to required curriculum subjects. It must determine the

amount of credit to be allowed by documents verifying that experience, and by

giving the student a test equal to the one given to students who complete the

comparable required curriculum subject at the school.

(4) A school may credit a student seeking an additional rating with

previous satisfactory completion of the general portion of an AMTS

curriculum.

(d) A school may not have more students enrolled than the number stated in

its application for a certificate, unless it amends its application and has

it approved.

(e) A school shall use an approved system for determining final course

grades and for recording student attendance. The system must show hours of

absence allowed and show how the missed material will be made available to

the student.

[Amdt. 147-2, 35 FR 5534, Apr. 3, 1970, as amended by Amdt. 147-4, 43 FR

22643, May 25, 1978; Amdt. 147-5, 57 FR 28959, June 29, 1992]

Sec. 147.33   Records.

(a) Each certificated aviation maintenance technician school shall keep a

current record of each student enrolled, showing--

(1) His attendance, tests, and grades received on the subjects required by

this part;

(2) The instruction credited to him under Sec. 147.31(c), if any; and

(3) The authenticated transcript of his grades from that school.

It shall retain the record for at least two years after the end of the

student's enrollment, and shall make each record available for inspection by

the Administrator during that period.

(b) Each school shall keep a current progress chart or individual progress

record for each of its students, showing the practical projects or laboratory

work completed, or to be completed, by the student in each subject.

[Doc. No. 1157, 27 FR 6669, July 13, 1962]

Sec. 147.35   Transcripts and graduation certificates.

(a) Upon request, each certificated aviation maintenance technician school

shall provide a transcript of the student's grades to each student who is

graduated from that school or who leaves it before being graduated. An

official of the school shall authenticate the transcript. The transcript must

state the curriculum in which the student was enrolled, whether the student

satisfactorily completed that curriculum, and the final grades the student

received.

(b) Each school shall give a graduation certificate or certificate of

completion to each student that it graduates. An official of the school shall

authenticate the certificate. The certificate must show the date of

graduation and the approved curriculum title.

[Doc. No. 1157, 27 FR 6669, July 13, 1962, as amended by Amdt. 147-5, 57 FR

28959, June 29, 1992]

Sec. 147.36   Maintenance of instructor requirements.

Each certificated aviation maintenance technician school shall, after

certification or addition of a rating, continue to provide the number of

instructors holding appropriate mechanic certificates and ratings that the

Administrator determines necessary to provide adequate instruction to the

students, including at least one such instructor for each 25 students in each

shop class. The school may continue to provide specialized instructors who

are not certificated mechanics to teach mathematics, physics, drawing, basic

electricity, basic hydraulics, and similar subjects.

[Amdt. 147-5, 57 FR 28959, June 29, 1992]

Sec. 147.37   Maintenance of facilities, equipment, and material.

(a) Each certificated aviation maintenance technician school shall provide

facilities, equipment, and material equal to the standards currently required

for the issue of the certificate and rating that it holds.

(b) A school may not make a substantial change in facilities, equipment, or

material that have been approved for a particular curriculum, unless that

change is approved in advance.

Sec. 147.38   Maintenance of curriculum requirements.

(a) Each certificated aviation maintenance technician school shall adhere

to its approved curriculum. With FAA approval, curriculum subjects may be

taught at levels exceeding those shown in Appendix A of this part.

(b) A school may not change its approved curriculum unless the change is

approved in advance.

[Amdt. 147-2, 35 FR 5534, Apr. 3, 1970, as amended by Amdt. 147-5, 57 FR

28960, June 29, 1992]

Sec. 147.38a    Quality of instruction.

Each certificated aviation maintenance technician school shall provide

instruction of such quality that, of its graduates of a curriculum for each

rating who apply for a mechanic certificate or additional rating within 60

days after they are graduated, the percentage of those passing the applicable

FAA written tests on their first attempt during any period of 24 calendar

months is at least the percentage figured as follows:

(a) For a school graduating fewer than 51 students during that period--the

national passing norm minus the number 20.

(b) For a school graduating at least 51, but fewer than 201, students

during that period--the national passing norm minus the number 15.

(c) For a school graduating more than 200 students during that period--the

national passing norm minus the number 10.

As used in this section, "national passing norm" is the number representing

the percentage of all graduates (of a curriculum for a particular rating) of

all certificated aviation maintenance technician schools who apply for a

mechanic certificate or additional rating within 60 days after they are

graduated and pass the applicable FAA written tests on their first attempt

during the period of 24 calendar months described in this section.

[Amdt. 147-2, 35 FR 5534, Apr. 3, 1970, as amended by Amdt. 147-3, 41 FR

47230, Oct. 28, 1976]

Sec. 147.39   Display of certificate.

Each holder of an aviation maintenance technician school certificate and

ratings shall display them at a place in the school that is normally

accessible to the public and is not obscured. The certificate must be

available for inspection by the Administrator.

Sec. 147.41   Change of location.

The holder of an aviation maintenance technician school certificate may not

make any change in the school's location unless the change is approved in

advance. If the holder desires to change the location he shall notify the

Administrator, in writing, at least 30 days before the date the change is

contemplated. If he changes its location without approval, the certificate is

revoked.

Sec. 147.43   Inspection.

The Administrator may, at any time, inspect an aviation maintenance

technician school to determine its compliance with this part. Such an

inspection is normally made once each six months to determine if the school

continues to meet the requirements under which it was originally

certificated. After such an inspection is made, the school is notified, in

writing, of any deficiencies found during the inspection. Other informal

inspections may be made from time to time.

Sec. 147.45   Advertising.

(a) A certificated aviation maintenance technician school may not make any

statement relating to itself that is false or is designed to mislead any

person considering enrollment therein.

(b) Whenever an aviation maintenance technician school indicates in

advertising that it is a certificated school, it shall clearly distinguish

between its approved courses and those that are not approved.

Appendix A to Part 147--Curriculum Requirements

This appendix defines terms used in Appendices B, C, and D of this part,

and describes the levels of proficiency at which items under each subject in

each curriculum must be taught, as outlined in Appendices B, C, and D.

(a) Definitions. As used in Appendices B, C, and D:

(1) "Inspect" means to examine by sight and touch.

(2) "Check" means to verify proper operation.

(3) "Troubleshoot" means to analyze and identify malfunctions.

(4) "Service" means to perform functions that assure continued operation.

(5) "Repair" means to correct a defective condition. Repair of an airframe

or powerplant system includes component replacement and adjustment, but not

component repair.

(6) "Overhaul" means to disassemble, inspect, repair as necessary, and

check.

(b) Teaching levels.

(1) Level 1 requires:

(i) Knowledge of general principles, but no practical application.

(ii) No development of manipulative skill.

(iii) Instruction by lecture, demonstration, and discussion.

(2) Level 2 requires:

(i) Knowledge of general principles, and limited practical application.

(ii) Development of sufficient manipulative skill to perform basic

operations.

(iii) Instruction by lecture, demonstration, discussion, and limited

practical application.

(3) Level 3 requires:

(i) Knowledge of general principles, and performance of a high degree of

practical application.

(ii) Development of sufficient manipulative skills to simulate return to

service.

(iii) Instruction by lecture, demonstration, discussion, and a high degree

of practical application.

(c) Teaching materials and equipment.

The curriculum may be presented utilizing currently accepted educational

materials and equipment, including, but not limited to: calculators,

computers, and audio-visual equipment.

[Amdt. 147-2, 35 FR 5534, Apr. 3, 1970, as amended by Amdt. 147-5, 57 FR

28960, June 29, 1992]

Appendix B to Part 147--General Curriculum Subjects

This appendix lists the subjects required in at least 400 hours in general

curriculum subjects.

The number in parentheses before each item listed under each subject

heading indicates the level of proficiency at which that item must be taught.

Teaching

level

a. basic electricity

(2)  1. Calculate and measure capacitance and inductance.

(2)  2. Calculate and measure electrical power.

(3)  3. Measure voltage, current, resistance, and continuity.

(3)  4. Determine the relationship of voltage, current, and resistance

in electrical circuits.

(3)  5. Read and interpret aircraft electrical circuit diagrams,

including solid state devices and logic functions.

(3)  6. Inspect and service batteries.

b. aircraft drawings

(2)  7. Use aircraft drawings, symbols, and system schematics.

(3)  8. Draw sketches of repairs and alterations.

(3)  9. Use blueprint information.

(3)  10. Use graphs and charts.

c. weight and balance

(2)  11. Weigh aircraft.

(3)  12. Perform complete weight-and-balance check and record data.

d. fluid lines and fittings

(3)  13. Fabricate and install rigid and flexible fluid lines and

fittings.

e. materials and processes

(1)  14. Identify and select appropriate nondestructive testing methods.

(2)  15. Perform dye penetrant, eddy current, ultrasonic, and magnetic

particle inspections.

(1)  16. Perform basic heat-treating processes.

(3)  17. Identify and select aircraft hardware and materials.

(3)  18. Inspect and check welds.

(3)  19. Perform precision measurements.

f. ground operation and servicing

(2)  20. Start, ground operate, move, service, and secure aircraft and

identify typical ground operation hazards.

(2)  21. Identify and select fuels.

g. cleaning and corrosion control

(3)  22. Identify and select cleaning materials.

(3)  23. Inspect, identify, remove, and treat aircraft corrosion and

perform aircraft cleaning.

h. mathematics

(3)  24. Extract roots and raise numbers to a given power.

(3)  25. Determine areas and volumes of various geometrical shapes.

(3)  26. Solve ratio, proportion, and percentage problems.

(3)  27. Perform algebraic operations involving addition, subtraction,

multiplication, and division of positive and negative numbers.

i. maintenance forms and records

(3)  28. Write descriptions of work performed including aircraft

discrepancies and corrective actions using typical aircraft

maintenance records.

(3)  29. Complete required maintenance forms, records, and inspection

reports.

j. basic physics

(2)  30. Use and understand the principles of simple machines; sound,

fluid, and heat dynamics; basic aerodynamics; aircraft

structures; and theory of flight.

k. maintenance publications

(3)  31. Demonstrate ability to read, comprehend, and apply information

contained in FAA and manufacturers' aircraft maintenance

specifications, data sheets, manuals, publications, and related

Federal Aviation Regulations, Airworthiness Directives, and

Advisory material.

(3)  32. Read technical data.

l. mechanic privileges and limitations

(3)  33. Exercise mechanic privileges within the limitations prescribed

by Part 65 of this chapter.

[Amdt. 147-2, 35 FR 5534, Apr. 3, 1970, as amended by Amdt. 147-5, 57 FR

28960, June 29, 1992]

Appendix C to Part 147--Airframe Curriculum Subjects

This appendix lists the subjects required in at least 750 hours of each

airframe curriculum, in addition to at least 400 hours in general curriculum

subjects.

The number in parentheses before each item listed under each subject

heading indicates the level of proficiency at which that item must be taught.

I. Airframe Structures

Teaching

level

a. wood structures

(1)  1. Service and repair wood structures.

(1)  2. Identify wood defects.

(1)  3. Inspect wood structures.

b. aircraft covering

(1)  4. Select and apply fabric and fiberglass covering materials.

(1)  5. Inspect, test, and repair fabric and fiberglass.

c. aircraft finishes

(1)  6. Apply trim, letters, and touchup paint.

(2)  7. Identify and select aircraft finishing materials.

(2)  8. Apply finishing materials.

(2)  9. Inspect finishes and identify defects.

d. sheet metal and non-metallic structures

(2)  10. Select, install and remove special fasteners for metallic,

bonded, and composite structures.

(2)  11. Inspect bonded structures.

(2)  12. Inspect, test and repair fiberglass, plastics, honeycomb,

composite, and laminated primary and secondary structures.

(2)  13. Inspect, check, service, and repair windows, doors, and

interior furnishings.

(3)  14. Inspect and repair sheet-metal structures.

(3)  15. Install conventional rivets.

(3)  16. Form, lay out, and bend sheet metal.

e. welding

(1)  17. Weld magnesium and titanium.

(1)  18. Solder stainless steel.

(1)  19. Fabricate tubular structures.

(2)  20. Solder, braze, gas-weld, and arc-weld steel.

(1)  21. Weld aluminum and stainless steel.

f. assembly and rigging

(1)  22. Rig rotary-wing aircraft.

(2)  23. Rig fixed-wing aircraft.

(2)  24. Check alignment of structures.

(3)  25. Assemble aircraft components, including flight control

surfaces.

(3)  26. Balance, rig, and inspect movable primary and secondary flight

control surfaces.

(3)  27. Jack aircraft.

g. airframe inspection

(3)  28. Perform airframe conformity and airworthiness inspections.

II. Airframe Systems and Components

Teaching

level

a. aircraft landing gear systems

(3)  29. Inspect, check, service, and repair landing gear, retraction

systems, shock struts, brakes, wheels, tires, and steering

systems.

b. hydraulic and pneumatic power systems

(2)  30. Repair hydraulic and pneumatic power systems components.

(3)  31. Identify and select hydraulic fluids.

(3)  32. Inspect, check, service, troubleshoot, and repair hydraulic and

pneumatic power systems.

c. cabin atmosphere control systems

(1)  33. Inspect, check, troubleshoot, service, and repair heating,

cooling, air conditioning, pressurization systems, and air cycle

machines.

(1)  34. Inspect, check, troubleshoot, service, and repair heating,

cooling, air-conditioning, and pressurization systems.

(2)  35. Inspect, check, troubleshoot, service and repair oxygen

systems.

d. aircraft instrument systems

(1)  36. Inspect, check, service, troubleshoot and repair electronic

flight instrument systems and both mechanical and electrical

heading, speed, altitude, temperature, pressure, and position

indicating systems to include the use of built-in test equipment.

(2)  37. Install instruments and perform a static pressure system leak

test.

e. communication and navigation systems

(1)  38. Inspect, check, and troubleshoot autopilot, servos and approach

coupling systems.

(1)  39. Inspect, check, and service aircraft electronic communication

and navigation systems, including VHF passenger address

interphones and static discharge devices, aircraft VOR, ILS,

LORAN, Radar beacon transponders, flight management computers, and

GPWS.

(2)  40. Inspect and repair antenna and electronic equipment

installations.

f. aircraft fuel systems

(1)  41. Check and service fuel dump systems.

(1)  42. Perform fuel management transfer, and defueling.

(1)  43. Inspect, check, and repair pressure fueling systems.

(2)  44. Repair aircraft fuel system components.

(2)  45. Inspect and repair fluid quantity indicating systems.

(2)  46. Troubleshoot, service, and repair fluid pressure and

temperature warning systems.

(3)  47. Inspect, check, service, troubleshoot, and repair aircraft fuel

systems.

g. aircraft electrical systems

(2)  48. Repair and inspect aircraft electrical system components; crimp

and splice wiring to manufacturers' specifications; and repair

pins and sockets of aircraft connectors.

(3)  49. Install, check, and service airframe electrical wiring,

controls, switches, indicators, and protective devices.

(3)  50.a. Inspect, check, troubleshoot, service, and repair alternating

and direct current electrical systems.

(1)  50.b. Inspect, check, and troubleshoot constant speed and

integrated speed drive generators.

h. position and warning systems

(2)  51. Inspect, check, and service speed and configuration warning

systems, electrical brake controls, and anti-skid systems.

(3)  52. Inspect, check, troubleshoot, and service landing gear position

indicating and warning systems.

i. ice and rain control systems

(2)  53. Inspect, check, troubleshoot, service, and repair airframe ice

and rain control systems.

j. fire protection systems

(1)  54. Inspect, check, and service smoke and carbon monoxide detection

systems.

(3)  55. Inspect, check, service, troubleshoot, and repair aircraft fire

detection and extinguishing systems.

[Amdt. 147-2, 35 FR 5535, Apr. 3, 1970, as amended by Amdt. 147-5, 57 FR

28960, June 29, 1992]

Appendix D to Part 147--Powerplant Curriculum Subjects

This appendix lists the subjects required in at least 750 hours of each

powerplant curriculum, in addition to at least 400 hours in general

curriculum subjects.

The number in parentheses before each item listed under each subject

heading indicates the level of proficiency at which that item must be taught.

I. Powerplant Theory and Maintenance

Teaching

level

a. reciprocating engines

(1)  1. Inspect and repair a radial engine.

(2)  2. Overhaul reciprocating engine.

(3)  3. Inspect, check, service, and repair reciprocating engines and

engine installations.

(3)  4. Install, troubleshoot, and remove reciprocating engines.

b. turbine engines

(2)  5. Overhaul turbine engine.

(3)  6. Inspect, check, service, and repair turbine engines and turbine

engine installations.

(3)  7. Install, troubleshoot, and remove turbine engines.

c. engine inspection

(3)  8. Perform powerplant conformity and air worthiness inspections.

II. Powerplant Systems and Components

Teaching

level

a. engine instrument systems

(2)  9. Troubleshoot, service, and repair electrical and mechanical

fluid rate-of-flow indicating systems.

(3)  10. Inspect, check, service, troubleshoot, and repair electrical

and mechanical engine temperature, pressure, and r.p.m. indicating

systems.

b. engine fire protection systems

(3)  11. Inspect, check, service, troubleshoot, and repair engine fire

detection and extinguishing systems.

c. engine electrical systems

(2)  12. Repair engine electrical system components.

(3)  13. Install, check, and service engine electrical wiring, controls,

switches, indicators, and protective devices.

d. lubrication systems

(2)  14. Identify and select lubricants.

(2)  15. Repair engine lubrication system components.

(3)  16. Inspect, check, service, troubleshoot, and repair engine

lubrication systems.

e. ignition and starting systems

(2)  17. Overhaul magneto and ignition harness.

(2)  18. Inspect, service, troubleshoot, and repair reciprocating and

turbine engine ignition systems and components.

(3)  19.a. Inspect, service, troubleshoot, and repair turbine engine

electrical starting systems.

(1)  19.b. Inspect, service, and troubleshoot turbine engine pneumatic

starting systems.

f. fuel metering systems

(1)  20. Troubleshoot and adjust turbine engine fuel metering systems

and electronic engine fuel controls.

(2)  21. Overhaul carburetor.

(2)  22. Repair engine fuel metering system components.

(3)  23. Inspect, check, service, troubleshoot, and repair reciprocating

and turbine engine fuel metering systems.

g. engine fuel systems

(2)  24. Repair engine fuel system components.

(3)  25. Inspect, check, service, troubleshoot, and repair engine fuel

systems.

h. induction and engine airflow systems

(2)  26. Inspect, check, troubleshoot, service, and repair engine ice

and rain control systems.

(1)  27. Inspect, check, service, troubleshoot and repair heat

exchangers, superchargers, and turbine engine airflow and

temperature control systems.

(3)  28. Inspect, check, service, and repair carburetor air intake and

induction manifolds.

i. engine cooling systems

(2)  29. Repair engine cooling system components.

(3)  30. Inspect, check, troubleshoot, service, and repair engine

cooling systems.

j. engine exhaust and reserver systems

(2)  31. Repair engine exhaust system components.

(3)  32.a. Inspect, check, troubleshoot, service, and repair engine

exhaust systems.

(1)  32.b. Troubleshoot and repair engine thrust reserver systems and

related components.

k. propellers

(1)  33. Inspect, check, service, and repair propeller synchronizing and

ice control systems.

(2)  34. Identify and select propeller lubricants.

(1)  35. Balance propellers.

(2)  36. Repair propeller control system components.

(3)  37. Inspect, check, service, and repair fixed-pitch, constant-

speed, and feathering propellers, and propeller governing systems.

(3)  38. Install, troubleshoot, and remove propellers.

(3)  39. Repair aluminum alloy propeller blades.

l. unducted fans

(1)  40. Inspect and troubleshoot unducted fan systems and components.

m. auxiliary power units

(1)  41. Inspect, check, service, and troubleshoot turbine-driven

auxiliary power units.

(Sec. 6(c), Dept. of Transportation Act; 49 U.S.C. 1655(c))

[Amdt. 147-2, 35 FR 5535, Apr. 3, 1970, as amended by Amdt. 147-5, 57 FR

28961, June 29, 1992]