PART 33--AIRWORTHINESS STANDARDS: AIRCRAFT ENGINES

Subpart A--General

Sec.

33.1  Applicability.

33.3  General.

33.4  Instructions for Continued Airworthiness.

33.5  Instruction manual for installing and operating the engine.

33.7  Engine ratings and operating limitations.

33.8  Selection of engine power and thrust ratings.

Subpart B--Design and Construction; General

33.11  Applicability.

33.13  [Reserved]

33.14  Start-stop cyclic stress (low-cycle fatigue).

33.15  Materials.

33.17  Fire prevention.

33.19  Durability.

33.21  Engine cooling.

33.23  Engine mounting attachments and structure.

33.25  Accessory attachments.

33.27  Turbine, compressor, fan, and turbosupercharger rotors.

33.28  Electrical and electronic engine control systems.

33.29  Instrument connection.

Subpart C--Design and Construction; Reciprocating Aircraft Engines

33.31  Applicability.

33.33  Vibration.

33.35  Fuel and induction system.

33.37  Ignition system.

33.39  Lubrication system.

Subpart D--Block Tests; Reciprocating Aircraft Engines

33.41  Applicability.

33.42  General.

33.43  Vibration test.

33.45  Calibration tests.

33.47  Detonation test.

33.49  Endurance test.

33.51  Operation test.

33.53  Engine component tests.

33.55  Teardown inspection.

33.57  General conduct of block tests.

Subpart E--Design and Construction; Turbine Aircraft Engines

33.61  Applicability.

33.62  Stress analysis.

33.63  Vibration.

33.65  Surge and stall characteristics.

33.66  Bleed air system.

33.67  Fuel system.

33.68  Induction system icing.

33.69  Ignitions system.

33.71  Lubrication system.

33.72  Hydraulic actuating systems.

33.73  Power or thrust response.

33.75  Safety analysis.

33.77  Foreign object ingestion.

33.79  Fuel burning thrust augmentor.

Subpart F--Block Tests; Turbine Aircraft Engines

33.81  Applicability.

33.82  General.

33.83  Vibration test.

33.85  Calibration tests.

33.87  Endurance test.

33.88  Engine overtemperature test.

33.89  Operation test.

33.90  Initial maintenance inspection.

33.91  Engine component tests.

33.92  Windmilling tests.

33.93  Teardown inspection.

33.94  Blade containment and rotor unbalance tests.

33.95  Engine-propeller systems tests.

33.96  Engine tests in auxiliary power unit (APU) mode.

33.97  Thrust reversers.

33.99  General conduct of block tests.

Appendix A--Instructions for Continued Airworthiness

Authority: 49 U.S.C. 1344, 1354(a), 1355, 1421, 1423, 1424, 1425; 49 U.S.C.

106(g) (Revised Pub. L. 97-449, January 12, 1983).

Source: Docket No. 3025, 29 FR 7453, June 10, 1964, unless otherwise noted.

Note: For miscellaneous amendments to cross references in this Part 33, see

Amdt. 33-2, 31 FR 9211, July 6, 1966.

Subpart A--General

Sec. 33.1  Applicability.

(a) This part prescribes airworthiness standards for the issue of type

certificates and changes to those certificates, for aircraft engines.

(b) Each person who applies under part 21 for such a certificate or change

must show compliance with the applicable requirements of this part and the

applicable requirements of part 34 of this chapter.

[Amdt. 33-7, 41 FR 55474, Dec. 20, 1976, as amended by Amdt. 33-14, 55 FR

32861, Aug. 10, 1990]

Sec. 33.3  General.

Each applicant must show that the aircraft engine concerned meets the

applicable requirements of this part.

Sec. 33.4  Instructions for Continued Airworthiness.

The applicant must prepare Instructions for Continued Airworthiness in

accordance with Appendix A to this part that are acceptable to the

Administrator. The instructions may be incomplete at type certification if a

program exists to ensure their completion prior to delivery of the first

aircraft with the engine installed, or upon issuance of a standard

certificate of airworthiness for the aircraft with the engine installed,

whichever occurs later.

[Amdt. 33-9, 45 FR 60181, Sept. 11, 1980]

Sec. 33.5  Instruction manual for installing and operating the engine.

Each applicant must prepare and make available to the Administrator prior

to the issuance of the type certificate, and to the owner at the time of

delivery of the engine, approved instructions for installing and operating

the engine. The instructions must include at least the following:

(a) Installation instructions. (1) The location of engine mounting

attachments, the method of attaching the engine to the aircraft, and the

maximum allowable load for the mounting attachments and related structure.

(2) The location and description of engine connections to be attached to

accessories, pipes, wires, cables, ducts, and cowling.

(3) An outline drawing of the engine including overall dimensions.

(b) Operation instructions. (1) The operating limitations established by

the Administrator.

(2) The power or thrust ratings and procedures for correcting for

nonstandard atmosphere.

(3) The recommended procedures, under normal and extreme ambient conditions

for--

(i) Starting;

(ii) Operating on the ground; and

(iii) Operating during flight.

[Amdt. 33-6, 39 FR 35463, Oct. 1, 1974, as amended by Amdt. 33-9, 45 FR

60181, Sept. 11, 1980]

Sec. 33.7  Engine ratings and operating limitations.

(a) Engine ratings and operating limitations are established by the

Administrator and included in the engine certificate data sheet specified in

Sec. 21.41 of this chapter, including ratings and limitations based on the

operating conditions and information specified in this section, as

applicable, and any other information found necessary for safe operation of

the engine.

(b) For reciprocating engines, ratings and operating limitations are

established relating to the following:

(1) Horsepower or torque, r.p.m., manifold pressure, and time at critical

pressure altitude and sea level pressure altitude for--

(i) Rated maximum continuous power (relating to unsupercharged operation or

to operation in each supercharger mode as applicable); and

(ii) Rated takeoff power (relating to unsupercharged operation or to

operation in each supercharger mode as applicable).

(2) Fuel grade or specification.

(3) Oil grade or specification.

(4) Temperature of the--

(i) Cylinder;

(ii) Oil at the oil inlet; and

(iii) Turbosupercharger turbine wheel inlet gas.

(5) Pressure of--

(i) Fuel at the fuel inlet; and

(ii) Oil at the main oil gallery.

(6) Accessory drive torque and overhang moment.

(7) Component life.

(8) Turbosupercharger turbine wheel r.p.m.

(c) For turbine engines, ratings and operating limitations are established

relating to the following:

(1) Horsepower, torque, or thrust, r.p.m., gas temperature, and time for--

(i) Rated maximum continuous power or thrust (augmented);

(ii) Rated maximum continuous power or thrust (unaugmented);

(iii) Rated takeoff power or thrust (augmented);

(iv) Rated takeoff power or thrust (unaugmented);

(v) Rated 30-minute OEI power;

(vi) Rated 2 1/2 -minute OEI power;

(vii) Rated continuous OEI power; and

(viii) Auxiliary power unit (APU) mode of operation.

(2) Fuel designation or specification.

(3) Oil grade or specification.

(4) Hydraulic fluid specification.

(5) Temperature of--

(i) Oil at a location specified by the applicant;

(ii) Induction air at the inlet face of a supersonic engine, including

steady state operation and transient over-temperature and time allowed;

(iii) Hydraulic fluid of a supersonic engine;

(iv) Fuel at a location specified by the applicant; and

(v) External surfaces of the engine, if specified by the applicant.

(6) Pressure of--

(i) Fuel at the fuel inlet;

(ii) Oil at a location specified by the applicant;

(iii) Induction air at the inlet face of a supersonic engine, including

steady state operation and transient overpressure and time allowed; and

(iv) Hydraulic fluid.

(7) Accessory drive torque and overhang moment.

(8) Component life.

(9) Fuel filtration.

(10) Oil filtration.

(11) Bleed air.

(12) The number of start-stop stress cycles approved for each rotor disc

and spacer.

(13) Inlet air distortion at the engine inlet.

(14) Transient rotor shaft overspeed r.p.m., and number of overspeed

occurrences.

(15) Transient gas overtemperature, and number of overtemperature

occurrences.

(16) For engines to be used in supersonic aircraft, engine rotor

windmilling rotational r.p.m.

[Amdt. 33-6, 39 FR 35463, Oct. 1, 1974, as amended by Amdt. 33-10, 49 FR

6850, Feb. 23, 1984; Amdt. 33-11, 51 FR 10346, Mar. 25, 1986; Amdt. 33-12, 53

FR 34220, Sept. 2, 1988]

Sec. 33.8  Selection of engine power and thrust ratings.

(a) Requested engine power and thrust ratings must be selected by the

applicant.

(b) Each selected rating must be for the lowest power or thrust that all

engines of the same type may be expected to produce under the conditions used

to determine that rating.

[Amdt. 33-3, 32 FR 3736, Mar. 4, 1967]

Subpart B--Design and Construction; General

Sec. 33.11  Applicability.

This subpart prescribes the general design and construction requirements

for reciprocating and turbine aircraft engines.

Sec. 33.13  [Reserved]

Sec. 33.14  Start-stop cyclic stress (low-cycle fatigue).

By a procedure approved by the FAA, operating limitations must be

established which specify the maximum allowable number of start-stop stress

cycles for each rotor structural part (such as discs, spacers, hubs, and

shafts of the compressors and turbines), the failure of which could produce a

hazard to the aircraft. A start-stop stress cycle consists of a flight cycle

profile or an equivalent representation of engine usage. It includes starting

the engine, accelerating to maximum rated power or thrust, decelerating, and

stopping. For each cycle, the rotor structural parts must reach stabilized

temperature during engine operation at a maximum rate power or thrust and

after engine shutdown, unless it is shown that the parts undergo the same

stress range without temperature stabilization.

[Amdt. 33-10, 49 FR 6850, Feb. 23, 1984]

Sec. 33.15  Materials.

The suitability and durability of materials used in the engine must--

(a) Be established on the basis of experience or tests; and

(b) Conform to approved specifications (such as industry or military

specifications) that ensure their having the strength and other properties

assumed in the design data.

(Secs. 313(a), 601, and 603, 72 Stat. 759, 775, 49 U.S.C. 1354(a), 1421, and

1423; sec. 6(c), 49 U.S.C. 1655(c))

[Amdt. 33-8, 42 FR 15047, Mar. 17, 1977, as amended by Amdt. 33-10, 49 FR

6850, Feb. 23, 1984]

Sec. 33.17  Fire prevention.

(a) The design and construction of the engine and the materials used must

minimize the probability of the occurrence and spread of fire. In addition,

the design and construction of turbine engines must minimize the probability

of the occurrence of an internal fire that could result in structural

failure, overheating, or other hazardous conditions.

(b) Except as provided in paragraphs (c), (d), and (e) of this section,

each external line, fitting, and other component, which contains or conveys

flammable fluid must be fire resistant. Components must be shielded or

located to safeguard against the ignition of leaking flammable fluid.

(c) Flammable fluid tanks and supports which are part of and attached to

the engine must be fireproof or be enclosed by a fireproof shield unless

damage by fire to any non-fireproof part will not cause leakage or spillage

of flammable fluid. For a reciprocating engine having an integral oil sump of

less than 25-quart capacity, the oil sump need not be fireproof nor be

enclosed by fireproof shield.

(d) For turbine engines type certificated for use in supersonic aircraft,

each external component which conveys or contains flammable fluid must be

fireproof.

(e) Unwanted accumulation of flammable fluid and vapor must be prevented by

draining and venting.

(Secs. 313(a), 601, and 603, 72 Stat. 759, 775, 49 U.S.C. 1354(a), 1421, and

1423; sec. 6(c), 49 U.S.C. 1655(c))

[Amdt. 33-6, 39 FR 35464, Oct. 1, 1974, as amended by Amdt. 33-8, 42 FR

15047, Mar. 17, 1977; Amdt. 33-10, 49 FR 6850, Feb. 23, 1984]

Sec. 33.19  Durability.

(a) Engine design and construction must minimize the development of an

unsafe condition of the engine between overhaul periods. The design of the

compressor and turbine rotor cases must provide for the containment of damage

from rotor blade failure. Energy levels and trajectories of fragments

resulting from rotor blade failure that lie outside the compressor and

turbine rotor cases must be defined.

(b) Each component of the propeller blade pitch control system which is a

part of the engine type design must meet the requirements of Sec. 35.42 of

this chapter.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-9, 45 FR

60181, Sept. 11, 1980; Amdt. 33-10, 49 FR 6851, Feb. 23, 1984]

Sec. 33.21  Engine cooling.

Engine design and construction must provide the necessary cooling under

conditions in which the airplane is expected to operate.

Sec. 33.23  Engine mounting attachments and structure.

(a) The maximum allowable limit and ultimate loads for engine mounting

attachments and related engine structure must be specified.

(b) The engine mounting attachments and related engine structure must be

able to withstand--

(1) The specified limit loads without permanent deformation; and

(2) The specified ultimate loads without failure, but may exhibit permanent

deformation.

[Amdt. 33-10, 49 FR 6851, Feb. 23, 1984]

Sec. 33.25  Accessory attachments.

The engine must operate properly with the accessory drive and mounting

attachments loaded. Each engine accessory drive and mounting attachment must

include provisions for sealing to prevent contamination of, or unacceptable

leakage from, the engine interior. A drive and mounting attachment requiring

lubrication for external drive splines, or coupling by engine oil, must

include provisions for sealing to prevent unacceptable loss of oil and to

prevent contamination from sources outside the chamber enclosing the drive

connection. The design of the engine must allow for the examination,

adjustment, or removal of each accessory required for engine operation.

[Amdt. 33-10, 49 FR 6851, Feb. 23, 1984]

Sec. 33.27  Turbine, compressor, fan, and turbosupercharger rotors.

(a) Turbine, compressor, fan, and turbosupercharger rotors must have

sufficient strength to withstand the test conditions specified in paragraph

(c) of this section.

(b) The design and functioning of engine control devices, systems, and

instruments must give reasonable assurance that those engine operating

limitations that affect turbine, compressor, fan, and turbosupercharger rotor

structural integrity will not be exceeded in service.

(c) The most critically stressed rotor component (except blades) of each

turbine, compressor, and fan, including integral drum rotors and centrifugal

compressors in an engine or turbosupercharger, as determined by analysis or

other acceptable means, must be tested for a period of 5 minutes--

(1) At its maximum operating temperature, except as provided in paragraph

(c)(2)(iv) of this section; and

(2) At the highest speed of the following, as applicable:

(i) 120 percent of its maximum permissible r.p.m. if tested on a rig and

equipped with blades or blade weights.

(ii) 115 percent of its maximum permissible r.p.m. if tested on an engine.

(iii) 115 percent of its maximum permissible r.p.m. if tested on

turbosupercharger driven by a hot gas supply from a special burner rig.

(iv) 120 percent of the r.p.m. at which, while cold spinning, it is subject

to operating stresses that are equivalent to those induced at the maximum

operating temperature and maximum permissible r.p.m.

(v) 105 percent of the highest speed that would result from failure of the

most critical component or system in a representative installation of the

engine.

(vi) The highest speed that would result from the failure of any component

or system in a representative installation of the engine, in combination with

any failure of a component or system that would not normally be detected

during a routine preflight check or during normal flight operation.

Following the test, each rotor must be within approved dimensional limits for

an overspeed condition and may not be cracked.

[Amdt. 33-10, 49 FR 6851, Feb. 23, 1984]

Sec. 33.28  Electrical and electronic engine control systems.

Each control system which relies on electrical and electronic means for

normal operation must:

(a) Have the control system description, the percent of available power or

trust controlled in both normal operation and failure conditions, and the

range of control of other controlled functions, specified in the instruction

manual required by Sec. 33.5 for the engine;

(b) Be designed and constructed so that any failure of aircraft-supplied

power or data will not result in an unacceptable change in power or thrust,

or prevent continued safe operation of the engine;

(c) Be designed and constructed so that no single failure or malfunction,

or probable combination of failures of electrical or electronic components of

the control system, results in an unsafe condition;

(d) Have environmental limits, including transients caused by lightning

strikes, specified in the instruction manual; and

(e) Have all associated software designed and implemented to prevent errors

that would result in an unacceptable loss of power or thrust, or other unsafe

condition, and have the method used to design and implement the software

approved by the Administrator.

[Amdt. 33-15, 58 FR 29095, May 18, 1993]

Sec. 33.29  Instrument connection.

(a) Unless it is constructed to prevent its connection to an incorrect

instrument, each connection provided for powerplant instruments required by

aircraft airworthiness regulations or necessary to insure operation of the

engine in compliance with any engine limitation must be marked to identify it

with its corresponding instrument.

(b) A connection must be provided on each turbojet engine for an indicator

system to indicate rotor system unbalance.

[Amdt. 33-5, 39 FR 1831, Jan. 15, 1974, as amended by Amdt. 33-6, 39 FR

35465, Oct. 1, 1974]

Subpart C--Design and Construction; Reciprocating Aircraft Engines

Sec. 33.31  Applicability.

This subpart prescribes additional design and construction requirements for

reciprocating aircraft engines.

Sec. 33.33  Vibration.

The engine must be designed and constructed to function throughout its

normal operating range of crankshaft rotational speeds and engine powers

without inducing excessive stress in any of the engine parts because of

vibration and without imparting excessive vibration forces to the aircraft

structure.

Sec. 33.35  Fuel and induction system.

(a) The fuel system of the engine must be designed and constructed to

supply an appropriate mixture of fuel to the cylinders throughout the

complete operating range of the engine under all flight and atmospheric

conditions.

(b) The intake passages of the engine through which air or fuel in

combination with air passes for combustion purposes must be designed and

constructed to minimize the danger of ice accretion in those passages. The

engine must be designed and constructed to permit the use of a means for ice

prevention.

(c) The type and degree of fuel filtering necessary for protection of the

engine fuel system against foreign particles in the fuel must be specified.

The applicant must show that foreign particles passing through the prescribed

filtering means will not critically impair engine fuel system functioning.

(d) Each passage in the induction system that conducts a mixture of fuel

and air must be self-draining, to prevent a liquid lock in the cylinders, in

all attitudes that the applicant establishes as those the engine can have

when the aircraft in which it is installed is in the static ground attitude.

(e) If provided as part of the engine, the applicant must show for each

fluid injection (other than fuel) system and its controls that the flow of

the injected fluid is adequately controlled.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-10, 49 FR

6851, Feb. 23, 1984]

Sec. 33.37  Ignition system.

Each spark ignition engine must have a dual ignition system with at least

two spark plugs for each cylinder and two separate electric circuits with

separate sources of electrical energy, or have an ignition system of

equivalent in-flight reliability.

Sec. 33.39  Lubrication system.

(a) The lubrication system of the engine must be designed and constructed

so that it will function properly in all flight attitudes and atmospheric

conditions in which the airplane is expected to operate. In wet sump engines,

this requirement must be met when only one-half of the maximum lubricant

supply is in the engine.

(b) The lubrication system of the engine must be designed and constructed

to allow installing a means of cooling the lubricant.

(c) The crankcase must be vented to the atmosphere to preclude leakage of

oil from excessive pressure in the crankcase.

Subpart D--Block Tests; Reciprocating Aircraft Engines

Sec. 33.41  Applicability.

This subpart prescribes the block tests and inspections for reciprocating

aircraft engines.

Sec. 33.42  General.

Before each endurance test required by this subpart, the adjustment setting

and functioning characteristic of each component having an adjustment setting

and a functioning characteristic that can be established independent of

installation on the engine must be established and recorded.

[Amdt. 33-6, 39 FR 35465, Oct. 1, 1974]

Sec. 33.43  Vibration test.

(a) Each engine must undergo a vibration survey to establish the torsional

and bending vibration characteristics of the crankshaft and the propeller

shaft or other output shaft, over the range of crankshaft speed and engine

power, under steady state and transient conditions, from idling speed to

either 110 percent of the desired maximum continuous speed rating or 103

percent of the maximum desired takeoff speed rating, whichever is higher. The

survey must be conducted using, for airplane engines, the same configuration

of the propeller type which is used for the endurance test, and using, for

other engines, the same configuration of the loading device type which is

used for the endurance test.

(b) The torsional and bending vibration stresses of the crankshaft and the

propeller shaft or other output shaft may not exceed the endurance limit

stress of the material from which the shaft is made. If the maximum stress in

the shaft cannot be shown to be below the endurance limit by measurement, the

vibration frequency and amplitude must be measured. The peak amplitude must

be shown to produce a stress below the endurance limit; if not, the engine

must be run at the condition producing the peak amplitude until, for steel

shafts, 10 million stress reversals have been sustained without fatigue

failure and, for other shafts, until it is shown that fatigue will not occur

within the endurance limit stress of the material.

(c) Each accessory drive and mounting attachment must be loaded, with the

loads imposed by each accessory used only for an aircraft service being the

limit load specified by the applicant for the drive or attachment point.

(d) The vibration survey described in paragraph (a) of this section must be

repeated with that cylinder not firing which has the most adverse vibration

effect, in order to establish the conditions under which the engine can be

operated safely in that abnormal state. However, for this vibration survey,

the engine speed range need only extend from idle to the maximum desired

takeoff speed, and compliance with paragraph (b) of this section need not be

shown.

[Amdt. 33-6, 39 FR 35465, Oct. 1, 1974, as amended by Amdt. 33-10, 49 FR

6851, Feb. 23, 1984]

Sec. 33.45  Calibration tests.

(a) Each engine must be subjected to the calibration tests necessary to

establish its power characteristics and the conditions for the endurance test

specified in Sec. 33.49. The results of the power characteristics calibration

tests form the basis for establishing the characteristics of the engine over

its entire operating range of crankshaft rotational speeds, manifold

pressures, fuel/air mixture settings, and altitudes. Power ratings are based

upon standard atmospheric conditions with only those accessories installed

which are essential for engine functioning.

(b) A power check at sea level conditions must be accomplished on the

endurance test engine after the endurance test. Any change in power

characteristics which occurs during the endurance test must be determined.

Measurements taken during the final portion of the endurance test may be used

in showing compliance with the requirements of this paragraph.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-6, 39 FR

35465, Oct. 1, 1974]

Sec. 33.47  Detonation test.

Each engine must be tested to establish that the engine can function

without detonation throughout its range of intended conditions of operation.

Sec. 33.49  Endurance test.

(a) General. Each engine must be subjected to an endurance test that

includes a total of 150 hours of operation (except as provided in paragraph

(e)(1)(iii) of this section) and, depending upon the type and contemplated

use of the engine, consists of one of the series of runs specified in

paragraphs (b) through (e) of this section, as applicable. The runs must be

made in the order found appropriate by the Administrator for the particular

engine being tested. During the endurance test the engine power and the

crankshaft rotational speed must be kept within +/-3 percent of the rated

values. During the runs at rated takeoff power and for at least 35 hours at

rated maximum continuous power, one cylinder must be operated at not less

than the limiting temperature, the other cylinders must be operated at a

temperature not lower than 50 degrees F. below the limiting temperature, and

the oil inlet temperature must be maintained within +/-10 degrees F. of the

limiting temperature. An engine that is equipped with a propeller shaft must

be fitted for the endurance test with a propeller that thrust-loads the

engine to the maximum thrust which the engine is designed to resist at each

applicable operating condition specified in this section. Each accessory

drive and mounting attachment must be loaded. During operation at rated

takeoff power and rated maximum continuous power, the load imposed by each

accessory used only for an aircraft service must be the limit load specified

by the applicant for the engine drive or attachment point.

(b) Unsupercharged engines and engines incorporating a gear-driven single-

speed supercharger. For engines not incorporating a supercharger and for

engines incorporating a gear-driven single-speed supercharger the applicant

must conduct the following runs:

(1) A 30-hour run consisting of alternate periods of 5 minutes at rated

takeoff power with takeoff speed, and 5 minutes at maximum best economy

cruising power or maximum recommended cruising power.

(2) A 20-hour run consisting of alternate periods of 1 1/2  hours at rated

maximum continuous power with maximum continuous speed, and 1/2  hour at 75

percent rated maximum continuous power and 91 percent maximum continuous

speed.

(3) A 20-hour run consisting of alternate periods of 1 1/2  hours at rated

maximum continuous power with maximum continuous speed, and 1/2  hour at 70

percent rated maximum continuous power and 89 percent maximum continuous

speed.

(4) A 20-hour run consisting of alternate periods of 1 1/2  hours at rated

maximum continuous power with maximum continuous speed, and 1/2  hour at 65

percent rated maximum continuous power and 87 percent maximum continuous

speed.

(5) A 20-hour run consisting of alternate periods of 1 1/2  hours at rated

maximum continuous power with maximum continuous speed, and 1/2  hour at 60

percent rated maximum continuous power and 84.5 percent maximum continuous

speed.

(6) A 20-hour run consisting of alternate periods of 1 1/2  hours at rated

maximum continuous power with maximum continuous speed, and 1/2  hour at 50

percent rated maximum continuous power and 79.5 percent maximum continuous

speed.

(7) A 20-hour run consisting of alternate periods of 2 1/2  hours at rated

maximum continuous power with maximum continuous speed, and 2 1/2  hours at

maximum best economy cruising power or at maximum recommended cruising power.

(c) Engines incorporating a gear-driven two-speed supercharger. For engines

incorporating a gear-driven two-speed supercharger the applicant must conduct

the following runs:

(1) A 30-hour run consisting of alternate periods in the lower gear ratio

of 5 minutes at rated takeoff power with takeoff speed, and 5 minutes at

maximum best economy cruising power or at maximum recommended cruising power.

If a takeoff power rating is desired in the higher gear ratio, 15 hours of

the 30-hour run must be made in the higher gear ratio in alternate periods of

5 minutes at the observed horsepower obtainable with the takeoff critical

altitude manifold pressure and takeoff speed, and 5 minutes at 70 percent

high ratio rated maximum continuous power and 89 percent high ratio maximum

continuous speed.

(2) A 15-hour run consisting of alternate periods in the lower gear ratio

of 1 hour at rated maximum continuous power with maximum continuous speed,

and 1/2  hour at 75 percent rated maximum continuous power and 91 percent

maximum continuous speed.

(3) A 15-hour run consisting of alternate periods in the lower gear ratio

of 1 hour at rated maximum continuous power with maximum continuous speed,

and 1/2  hour at 70 percent rated maximum continuous power and 89 percent

maximum continuous speed.

(4) A 30-hour run in the higher gear ratio at rated maximum continuous

power with maximum continuous speed.

(5) A 5-hour run consisting of alternate periods of 5 minutes in each of

the supercharger gear ratios. The first 5 minutes of the test must be made at

maximum continuous speed in the higher gear ratio and the observed horsepower

obtainable with 90 percent of maximum continuous manifold pressure in the

higher gear ratio under sea level conditions. The condition for operation for

the alternate 5 minutes in the lower gear ratio must be that obtained by

shifting to the lower gear ratio at constant speed.

(6) A 10-hour run consisting of alternate periods in the lower gear ratio

of 1 hour at rated maximum continuous power with maximum continuous speed,

and 1 hour at 65 percent rated maximum continuous power and 87 percent

maximum continuous speed.

(7) A 10-hour run consisting of alternate periods in the lower gear ratio

of 1 hour at rated maximum continuous power with maximum continuous speed,

and 1 hour at 60 percent rated maximum continuous power and 84.5 percent

maximum continuous speed.

(8) A 10-hour run consisting of alternate periods in the lower gear ratio

of 1 hour at rated maximum continuous power with maximum continuous speed,

and 1 hour at 50 percent rated maximum continuous power and 79.5 percent

maximum continuous speed.

(9) A 20-hour run consisting of alternate periods in the lower gear ratio

of 2 hours at rated maximum continuous power with maximum continuous speed,

and 2 hours at maximum best economy cruising power and speed or at maximum

recommended cruising power.

(10) A 5-hour run in the lower gear ratio at maximum best economy cruising

power and speed or at maximum recommended cruising power and speed.

Where simulated altitude test equipment is not available when operating in

the higher gear ratio, the runs may be made at the observed horsepower

obtained with the critical altitude manifold pressure or specified

percentages thereof, and the fuel-air mixtures may be adjusted to be rich

enough to suppress detonation.

(d) Helicopter engines. To be eligible for use on a helicopter each engine

must either comply with paragraphs (a) through (j) of Sec. 29.923 of this

chapter, or must undergo the following series of runs:

(1) A 35-hour run consisting of alternate periods of 30 minutes each at

rated takeoff power with takeoff speed, and at rated maximum continuous power

with maximum continuous speed.

(2) A 25-hour run consisting of alternate periods of 2 1/2  hours each at

rated maximum continuous power with maximum continuous speed, and at 70

percent rated maximum continuous power with maximum continuous speed.

(3) A 25-hour run consisting of alternate periods of 2 1/2  hours each at

rated maximum continuous power with maximum continuous speed, and at 70

percent rated maximum continuous power with 80 to 90 percent maximum

continuous speed.

(4) A 25-hour run consisting of alternate periods of 2 1/2  hours each at

30 percent rated maximum continuous power with takeoff speed, and at 30

percent rated maximum continuous power with 80 to 90 percent maximum

continuous speed.

(5) A 25-hour run consisting of alternate periods of 2 1/2  hours each at

80 percent rated maximum continuous power with takeoff speed, and at either

rated maximum continuous power with 110 percent maximum continuous speed or

at rated takeoff power with 103 percent takeoff speed, whichever results in

the greater speed.

(6) A 15-hour run at 105 percent rated maximum continuous power with 105

percent maximum continuous speed or at full throttle and corresponding speed

at standard sea level carburetor entrance pressure, if 105 percent of the

rated maximum continuous power is not exceeded.

(e) Turbosupercharged engines. For engines incorporating a

turbosupercharger the following apply except that altitude testing may be

simulated provided the applicant shows that the engine and supercharger are

being subjected to mechanical loads and operating temperatures no less severe

than if run at actual altitude conditions:

(1) For engines used in airplanes the applicant must conduct the runs

specified in paragraph (b) of this section, except--

(i) The entire run specified in paragraph (b)(1) of this section must be

made at sea level altitude pressure;

(ii) The portions of the runs specified in paragraphs (b) (2) through (7)

of this section at rated maximum continuous power must be made at critical

altitude pressure, and the portions of the runs at other power must be made

at 8,000 feet altitude pressure; and

(iii) The turbosupercharger used during the 150-hour endurance test must be

run on the bench for an additional 50 hours at the limiting turbine wheel

inlet gas temperature and rotational speed for rated maximum continuous power

operation unless the limiting temperature and speed are maintained during 50

hours of the rated maximum continuous power operation.

(2) For engines used in helicopters the applicant must conduct the runs

specified in paragraph (d) of this section, except--

(i) The entire run specified in paragraph (d)(1) of this section must be

made at critical altitude pressure;

(ii) The portions of the runs specified in paragraphs (d) (2) and (3) of

this section at rated maximum continuous power must be made at critical

altitude pressure and the portions of the runs at other power must be made at

8,000 feet altitude pressure;

(iii) The entire run specified in paragraph (d)(4) of this section must be

made at 8,000 feet altitude pressure;

(iv) The portion of the runs specified in paragraph (d)(5) of this section

at 80 percent of rated maximum continuous power must be made at 8,000 feet

altitude pressure and the portions of the runs at other power must be made at

critical altitude pressure;

(v) The entire run specified in paragraph (d)(6) of this section must be

made at critical altitude pressure; and

(vi) The turbosupercharger used during the endurance test must be run on

the bench for 50 hours at the limiting turbine wheel inlet gas temperature

and rotational speed for rated maximum continuous power operation unless the

limiting temperature and speed are maintained during 50 hours of the rated

maximum continuous power operation.

[Amdt. 33-3, 32 FR 3736, Mar. 4, 1967, as amended by Amdt. 33-6, 39 FR 35465,

Oct. 1, 1974; Amdt. 33-10, 49 FR 6851, Feb. 23, 1984]

Sec. 33.51  Operation test.

The operation test must include the testing found necessary by the

Administrator to demonstrate backfire characteristics, starting, idling,

acceleration, overspeeding, functioning of propeller and ignition, and any

other operational characteristic of the engine. If the engine incorporates a

multispeed supercharger drive, the design and construction must allow the

supercharger to be shifted from operation at the lower speed ratio to the

higher and the power appropriate to the manifold pressure and speed settings

for rated maximum continuous power at the higher supercharger speed ratio

must be obtainable within five seconds.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-3, 32 FR

3737, Mar. 4, 1967]

Sec. 33.53  Engine component tests.

(a) For each engine that cannot be adequately substantiated by endurance

testing in accordance with Sec. 33.49, the applicant must conduct additional

tests to establish that components are able to function reliably in all

normally anticipated flight and atmospheric conditions.

(b) Temperature limits must be established for each component that requires

temperature controlling provisions in the aircraft installation to assure

satisfactory functioning, reliability, and durability.

Sec. 33.55  Teardown inspection.

After completing the endurance test--

(a) Each engine must be completely disassembled;

(b) Each component having an adjustment setting and a functioning

characteristic that can be established independent of installation on the

engine must retain each setting and functioning characteristic within the

limits that were established and recorded at the beginning of the test; and

(c) Each engine component must conform to the type design and be eligible

for incorporation into an engine for continued operation, in accordance with

information submitted in compliance with Sec. 33.4.

[Amdt. 33-6, 39 FR 35466, Oct. 1, 1974, as amended by Amdt. 33-9, 45 FR

60181, Sept. 11, 1980]

Sec. 33.57  General conduct of block tests.

(a) The applicant may, in conducting the block tests, use separate engines

of identical design and construction in the vibration, calibration,

detonation, endurance, and operation tests, except that, if a separate engine

is used for the endurance test it must be subjected to a calibration check

before starting the endurance test.

(b) The applicant may service and make minor repairs to the engine during

the block tests in accordance with the service and maintenance instructions

submitted in compliance with Sec. 33.4. If the frequency of the service is

excessive, or the number of stops due to engine malfunction is excessive, or

a major repair, or replacement of a part is found necessary during the block

tests or as the result of findings from the teardown inspection, the engine

or its parts may be subjected to any additional test the Administrator finds

necessary.

(c) Each applicant must furnish all testing facilities, including equipment

and competent personnel, to conduct the block tests.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-6, 39 FR

35466, Oct. 1, 1974; Amdt. 33-9, 45 FR 60181, Sept. 11, 1980]

Subpart E--Design and Construction; Turbine Aircraft Engines

Sec. 33.61  Applicability.

This subpart prescribes additional design and construction requirements for

turbine aircraft engines.

Sec. 33.62  Stress analysis.

A stress analysis must be performed on each turbine engine showing the

design safety margin of each turbine engine rotor, spacer, and rotor shaft.

[Amdt. 33-6, 39 FR 35466, Oct. 1, 1974]

Sec. 33.63  Vibration.

Each engine must be designed and constructed to function throughout its

operating range of rotational speeds and engine power without inducing

excessive stress in any engine part because of vibration and without

imparting excessive vibration forces to the aircraft structure.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-10, 49 FR

6851, Feb. 23, 1984]

Sec. 33.65  Surge and stall characteristics.

When the engine is operated in accordance with operating instructions

required by Sec. 33.5(b), starting, a change of power or thrust, power or

thrust augmentation, limiting inlet air distortion, or inlet air temperature

may not cause surge or stall to the extent that flameout, structural failure,

overtemperature, or failure of the engine to recover power or thrust will

occur at any point in the operating envelope.

[Amdt. 33-6, 39 FR 35466, Oct. 1, 1974]

Sec. 33.66   Bleed air system.

The engine must supply bleed air without adverse effect on the engine,

excluding reduced thrust or power output, at all conditions up to the

discharge flow conditions established as a limitation under Sec. 33.7(c)(11).

If bleed air used for engine anti-icing can be controlled, provision must be

made for a means to indicate the functioning of the engine ice protection

system.

[Amdt. 33-10, 49 FR 6851, Feb. 23, 1984]

Sec. 33.67  Fuel system.

(a) With fuel supplied to the engine at the flow and pressure specified by

the applicant, the engine must function properly under each operating

condition required by this part. Each fuel control adjusting means that may

not be manipulated while the fuel control device is mounted on the engine

must be secured by a locking device and sealed, or otherwise be inaccessible.

All other fuel control adjusting means must be accessible and marked to

indicate the function of the adjustment unless the function is obvious.

(b) There must be a fuel strainer or filter between the engine fuel inlet

opening and the inlet of either the fuel metering device or the engine-driven

positive displacement pump whichever is nearer the engine fuel inlet. In

addition, the following provisions apply to each strainer or filter required

by this paragraph (b):

(1) It must be accessible for draining and cleaning and must incorporate a

screen or element that is easily removable.

(2) It must have a sediment trap and drain except that it need not have a

drain if the strainer or filter is easily removable for drain purposes.

(3) It must be mounted so that its weight is not supported by the

connecting lines or by the inlet or outlet connections of the strainer or

filter, unless adequate strength margins under all loading conditions are

provided in the lines and connections.

(4) It must have the type and degree of fuel filtering specified as

necessary for protection of the engine fuel system against foreign particles

in the fuel. The applicant must show:

(i) That foreign particles passing through the specified filtering means do

not impair the engine fuel system functioning; and

(ii) That the fuel system is capable of sustained operation throughout its

flow and pressure range with the fuel initially saturated with water at 80

deg. F (27 deg. C) and having 0.025 fluid ounces per gallon (0.20 milliliters

per liter) of free water added and cooled to the most critical condition for

icing likely to be encountered in operation. However, this requirement may be

met by demonstrating the effectiveness of specified approved fuel anti-icing

additives, or that the fuel system incorporates a fuel heater which maintains

the fuel temperature at the fuel strainer or fuel inlet above 32 deg. F (0

deg. C) under the most critical conditions.

(5) The applicant must demonstrate that the filtering means has the

capacity (with respect to engine operating limitations) to ensure that the

engine will continue to operate within approved limits, with fuel

contaminated to the maximum degree of particle size and density likely to be

encountered in service. Operation under these conditions must be demonstrated

for a period acceptable to the Administrator, beginning when indication of

impending filter blockage is first given by either:

(i) Existing engine instrumentation; or

(ii) Additional means incorporated into the engine fuel system.

(6) Any strainer or filter bypass must be designed and constructed so that

the release of collected contaminants is minimized by appropriate location of

the bypass to ensure that collected contaminants are not in the bypass flow

path.

(c) If provided as part of the engine, the applicant must show for each

fluid injection (other than fuel) system and its controls that the flow of

the injected fluid is adequately controlled.

[Amdt. 33-6, 39 FR 35466, Oct. 1, 1974, as amended by Amdt. 33-10, 49 FR

6851, Feb. 23, 1984]

Sec. 33.68  Induction system icing.

Each engine, with all icing protection systems operating, must--

(a) Operate throughout its flight power range (including idling) without

the accumulation of ice on the engine components that adversely affects

engine operation or that causes a serious loss of power or thrust in

continuous maximum and intermittent maximum icing conditions as defined in

Appendix C of Part 25 of this chapter; and

(b) Idle for 30 minutes on the ground, with the available air bleed for

icing protection at its critical condition, without adverse effect, in an

atmosphere that is at a temperature between 15 deg. and 30 deg. F (between -9

deg. and -1 deg. C) and has a liquid water content not less than 0.3 grams

per cubic meter in the form of drops having a mean effective diameter not

less than 20 microns, followed by a momentary operation at takeoff power or

thrust. During the 30 minutes of idle operation the engine may be run up

periodically to a moderate power or thrust setting in a manner acceptable to

the Administrator.

[Amdt. 33-6, 39 FR 35466, Oct. 1, 1974, as amended by Amdt. 33-10, 49 FR

6852, Feb. 23, 1984]

Sec. 33.69  Ignitions system.

Each engine must be equipped with an ignition system for starting the

engine on the ground and in flight. An electric ignition system must have at

least two igniters and two separate secondary electric circuits, except that

only one igniter is required for fuel burning augmentation systems.

[Amdt. 33-6, 39 FR 35466, Oct. 1, 1974]

Sec. 33.71  Lubrication system.

(a) General. Each lubrication system must function properly in the flight

attitudes and atmospheric conditions in which an aircraft is expected to

operate.

(b) Oil strainer or filter. There must be an oil strainer or filter through

which all of the engine oil flows. In addition:

(1) Each strainer or filter required by this paragraph that has a bypass

must be constructed and installed so that oil will flow at the normal rate

through the rest of the system with the strainer or filter element completely

blocked.

(2) The type and degree of filtering necessary for protection of the engine

oil system against foreign particles in the oil must be specified. The

applicant must demonstrate that foreign particles passing through the

specified filtering means do not impair engine oil system functioning.

(3) Each strainer or filter required by this paragraph must have the

capacity (with respect to operating limitations established for the engine)

to ensure that engine oil system functioning is not impaired with the oil

contaminated to a degree (with respect to particle size and density) that is

greater than that established for the engine in paragraph (b)(2) of this

section.

(4) For each strainer or filter required by this paragraph, except the

strainer or filter at the oil tank outlet, there must be means to indicate

contamination before it reaches the capacity established in accordance with

paragraph (b)(3) of this section.

(5) Any filter bypass must be designed and constructed so that the release

of collected contaminants is minimized by appropriate location of the bypass

to ensure that the collected contaminants are not in the bypass flow path.

(6) Each strainer or filter required by this paragraph that has no bypass,

except the strainer or filter at an oil tank outlet or for a scavenge pump,

must have provisions for connection with a warning means to warn the pilot of

the occurence of contamination of the screen before it reaches the capacity

established in accordance with paragraph (b)(3) of this section.

(7) Each strainer or filter required by this paragraph must be accessible

for draining and cleaning.

(c) Oil tanks. (1) Each oil tank must have an expansion space of not less

than 10 percent of the tank capacity.

(2) It must be impossible to inadvertently fill the oil tank expansion

space.

(3) Each recessed oil tank filler connection that can retain any

appreciable quantity of oil must have provision for fitting a drain.

(4) Each oil tank cap must provide an oil-tight seal.

(5) Each oil tank filler must be marked with the word "oil."

(6) Each oil tank must be vented from the top part of the expansion space,

with the vent so arranged that condensed water vapor that might freeze and

obstruct the line cannot accumulate at any point.

(7) There must be means to prevent entrance into the oil tank or into any

oil tank outlet, of any object that might obstruct the flow of oil through

the system.

(8) There must be a shutoff valve at the outlet of each oil tank, unless

the external portion of the oil system (including oil tank supports) is

fireproof.

(9) Each unpressurized oil tank may not leak when subjected to a maximum

operating temperature and an internal pressure of 5 p.s.i., and each

pressurized oil tank may not leak when subjected to maximum operating

temperature and an internal pressure that is not less than 5 p.s.i. plus the

maximum operating pressure of the tank.

(10) Leaked or spilled oil may not accumulate between the tank and the

remainder of the engine.

(11) Each oil tank must have an oil quantity indicator or provisions for

one.

(12) If the propeller feathering system depends on engine oil--

(i) There must be means to trap an amount of oil in the tank if the supply

becomes depleted due to failure of any part of the lubricating system other

than the tank itself;

(ii) The amount of trapped oil must be enough to accomplish the feathering

opeation and must be available only to the feathering pump; and

(iii) Provision must be made to prevent sludge or other foreign matter from

affecting the safe operation of the propeller feathering system.

(d) Oil drains. A drain (or drains) must be provided to allow safe drainage

of the oil system. Each drain must--

(1) Be accessible; and

(2) Have manual or automatic means for positive locking in the closed

position.

(e) Oil radiators. Each oil radiator must withstand, without failure, any

vibration, inertia, and oil pressure load to which it is subjected during the

block tests.

[Amdt. 33-6, 39 FR 35466, Oct. 1, 1974, as amended by Amdt. 33-10, 49 FR

6852, Feb. 23, 1984]

Sec. 33.72  Hydraulic actuating systems.

Each hydraulic actuating system must function properly under all conditions

in which the engine is expected to operate. Each filter or screen must be

accessible for servicing and each tank must meet the design criteria of Sec.

33.71.

[Amdt. 33-6, 39 FR 35467, Oct. 1, 1974]

Sec. 33.73  Power or thrust response.

The design and construction of the engine must enable an increase--

(a) From minimum to rated takeoff power or thrust with the maximum bleed

air and power extraction to be permitted in an aircraft, without

overtemperature, surge, stall, or other detrimental factors occurring to the

engine whenever the power control lever is moved from the minimum to the

maximum position in not more than 1 second, except that the Administrator may

allow additional time increments for different regimes of control operation

requiring control scheduling; and

(b) From the fixed minimum flight idle power lever position when provided,

or if not provided, from not more than 15 percent of the rated takeoff power

or thrust available to 95 percent rated takeoff power or thrust in not over 5

seconds. The 5-second power or thrust response must occur from a stabilized

static condition using only the bleed air and accessories loads necessary to

run the engine. This takeoff rating is specified by the applicant and need

not include thrust augmentation.

[Amdt. 33-1, 36 FR 5493, Mar. 24, 1971]

Sec. 33.75  Safety analysis.

It must be shown by analysis that any probable malfunction or any probable

single or multiple failure, or any probable improper operation of the engine

will not cause the engine to--

(a) Catch fire;

(b) Burst (release hazardous fragments through the engine case);

(c) Generate loads greater than those ultimate loads specified in Sec.

33.23(a); or

(d) Lose the capability of being shut down.

[Amdt. 33-6, 39 FR 35467, Oct. 1, 1974, as amended by Amdt. 33-10, 49 FR

6852, Feb. 23, 1984]

Sec. 33.77  Foreign object ingestion.

(a) Ingestion of a 4-pound bird, under the conditions prescribed in

paragraph (e) of this section, may not cause the engine to--

(1) Catch fire;

(2) Burst (release hazardous fragments through the engine case);

(3) Generate loads greater than those ultimate loads specified in Sec.

33.23(a); or

(4) Lose the capability of being shut down.

(b) Ingestion of 3-ounce birds or 1 1/2 -pound birds, under the conditions

prescribed in paragraph (e) of this section, may not--

(1) Cause more than a sustained 25 percent power or thrust loss;

(2) Require the engine to be shut down within 5 minutes from the time of

ingestion; or

(3) Result in a potentially hazardous condition.

(c) Ingestion of water, ice, or hail, under the conditions prescribed in

paragraph (e) of this section, may not cause a sustained power or thrust loss

or require the engine to be shut down. It must be demonstrated that the

engine can accelerate and decelerate safely while inducting a mixture of at

least 4 percent water by weight of engine airflow following stabilized

operation at both flight idle and takeoff power settings with at least a 4

percent water-to-air ratio.

(d) For an engine that incorporates a protection device, compliance with

this section need not be demonstrated with respect to foreign objects to be

ingested under the conditions prescribed in paragraph (e) of this section if

it is shown that--

(1) Such foreign objects are of a size that will not pass through the

protective device;

(2) The protective device will withstand the impact of the foreign objects;

and

(3) The foreign object, or objects, stopped by the protective device will

not obstruct the flow of induction air into the engine with a resultant

sustained reduction in power or thrust greater than those values required by

paragraphs (b) and (c) of this section.

(e) Compliance with paragraphs (a), (b), and (c) of this section must be

shown by engine test under the following ingestion conditions:

Foreign                        Speed of          Engine

object      Test quantity   foreign object     operation       Ingestion

Birds:

3-ounce size  One for each    Liftoff speed    Takeoff         In rapid

50 square       of typical                       sequence to

inches of       aircraft                         simulate a

inlet area or                                    flock

fraction                                         encounter

thereof up to                                    and aimed at

a maximum of                                     selected

16 birds.                                        critical

Three-ounce                                      areas.

bird

ingestion not

required if a

1 1/2 -pound

bird will

pass the

inlet guide

vanes into

the rotor

blades

1 1/2 -pound  One for the     Initial climb    Takeoff         In rapid

size          first 300       speed of                         sequence to

square inches   typical                          simulate a

of inlet        aircraft                         flock

area, if it                                      encounter

can enter the                                    and aimed at

inlet, plus                                      selected

one for each                                     critical

additional                                       areas.

600 square

inches of

inlet area or

fraction

thereof up to

a maximum of

8 birds

4-pound size  One, if it can  Maximum climb    Maximum cruise  Aimed at

enter the       speed of                         critical

inlet           typical                          area.

aircraft if

the engine has

inlet guide

vanes

Liftoff speed    Takeoff         Aimed at

of typical                       critical

aircraft, if                     area.

the engine

does not have

inlet guide

vanes

Ice            Maximum         Sucked in        Maximum cruise  To simulate a

accumulation                                     continuous

on a typical                                     maximum

inlet cowl                                       icing

and engine                                       encounter at

face                                             25 deg.F.

resulting

from a 2-

minute delay

in actuating

anti-icing

system, or a

slab of ice

which is

comparable in

weight or

thickness for

that size

engine

Hail (0.8 to   For all         Rough air        Maximum cruise  In a volley

0.9 specific   engines: With   flight speed     at 15,000       to simulate

gravity)       inlet area of   of typical       feet altitude   a hailstone

not more than   aircraft                         encounter.

100 square                                       One-half the

inches: one                                      number of

1-inch                                           hailstones

hailstone.                                       aimed at

With inlet                                       random area

area of more                                     over the

than 100                                         face of the

square                                           inlet and

inches: one                                      the other

1-inch and                                       half aimed

one 2-inch                                       at the

hailstone for                                    critical

each 150                                         face area.

square inches

of inlet area

or fraction

thereof

For supersonic  Supersonic       Maximum cruise  Aimed at

engines (in     cruise                           critical

addition): 3    velocity.                        engine face

hailstones      Alternatively,                   area.

each having a   use subsonic

diameter        velocities

equal to that   with larger

in a straight   hailstones to

line            give

variation       equivalent

from 1 inch     kinetic energy

at 35,000

feet to 1/4

inch at

60,000 feet

using

diameter

corresponding

to the lowest

supersonic

cruise

altitude

expected

Water          At least 4      Sucked in        Flight idle,    For 3 minutes

percent of                       acceleration,   each at idle

engine                           takeoff,        and takeoff,

airflow by                       deceleration    and during

weight                                           acceleration

and

deceleration

in spray to

simulate

rain.

Note.--The term "inlet area" as used in this section means the engine inlet

projected area at the front face of the engine. It includes the projected

area of any spinner or bullet nose that is provided.

[Amdt. 33-10, 49 FR 6852, Feb. 23, 1984]

Sec. 33.79  Fuel burning thrust augmentor.

Each fuel burning thrust augmentor, including the nozzle, must--

(a) Provide cutoff of the fuel burning thrust augmentor;

(b) Permit on-off cycling;

(c) Be controllable within the intended range of operation;

(d) Upon a failure or malfunction of augmentor combustion, not cause the

engine to lose thrust other than that provided by the augmentor; and

(e) Have controls that function compatibly with the other engine controls

and automatically shut off augmentor fuel flow if the engine rotor speed

drops below the minimum rotational speed at which the augmentor is intended

to function.

[Amdt. 33-6, 39 FR 35468, Oct. 1, 1974]

Subpart F--Block Tests; Turbine Aircraft Engines

Sec. 33.81  Applicability.

This subpart prescribes the block tests and inspections for turbine

engines.

[Doc. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-6, 39 FR 35468,

Oct. 1, 1974]

Sec. 33.82  General.

Before each endurance test required by this subpart, the adjustment setting

and functioning characteristic of each component having an adjustment setting

and a functioning characteristic that can be established independent of

installation on the engine must be established and recorded.

[Amdt. 36-6, 39 FR 35468, Oct. 1, 1974]

Sec. 33.83  Vibration test.

(a) Each engine must undergo a vibration survey to establish the vibration

characteristics of the rotor discs, rotor blades, rotor shafts, stator

blades, and any other components that are subject to vibratory exciting

forces which could induce failure at the maximum inlet distortion limit. The

survey is to cover the range of rotor speeds and engine power or thrust,

under steady state and transient conditions, from idling speed to 103 percent

of the maximum permissible speed. The survey must be conducted using the same

configuration of the loading device which is used for the endurance test,

except that the Administrator may allow the use of a modified configuration

if that loading device type is incompatible with the necessary vibration

instrumentation.

(b) The vibration stresses (or strains) of rotor and stator components

determined under paragraph (a) of this section must be less, by a margin

acceptable to the Administrator, than the endurance limit of the material

from which these parts are made, adjusted for the most severe operating

conditions.

(c) Each accessory drive and mounting attachment must be loaded, with the

load imposed by each accessory used only for an aircraft service being the

limit load specified by the applicant for the engine drive or attachment

point.

[Amdt. 33-6, 39 FR 35468, Oct. 1, 1974, as amended by Amdt. 33-10, 49 FR

6853, Feb. 23, 1984]

Sec. 33.85  Calibration tests.

(a) Each engine must be subjected to those calibration tests necessary to

establish its power characteristics and the conditions for the endurance test

specified Sec. 33.87. The results of the power characteristics calibration

tests form the basis for establishing the characteristics of the engine over

its entire operating range of speeds, pressures, temperatures, and altitudes.

Power ratings are based upon standard atmospheric conditions with no airbleed

for aircraft services and with only those accessories installed which are

essential for engine functioning.

(b) A power check at sea level conditions must be accomplished on the

endurance test engine after the endurance test and any change in power

characteristics which occurs during the endurance test must be determined.

Measurements taken during the final portion of the endurance test may be used

in showing compliance with the requirements of this paragraph.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-6, 39 FR

35468, Oct. 1, 1974]

Sec. 33.87   Endurance test.

(a) General. Each engine must be subjected to an endurance test that

includes a total of 150 hours of operation and, depending upon the type and

contemplated use of the engine, consists of one of the series of runs

specified in paragraphs (b) through (f) of this section, as applicable. For

engines tested under paragraph (b), (c), (d), or (e) of this section, the

prescribed 6-hour test sequence must be conducted 25 times to complete the

required 150 hours of operation. The following test requirements apply:

(1) The runs must be made in the order found appropriate by the

Administrator for the particular engine being tested.

(2) Any automatic engine control that is part of the engine must control

the engine during the endurance test except for operations where automatic

control is normally overridden by manual control or where manual control is

otherwise specified for a particular test run.

(3) Except as provided in paragraph (a)(5) of this section, power or

thrust, gas temperature, rotor shaft rotational speed, and, if limited,

temperature of external surfaces of the engine must be at least 100 percent

of the value associated with the particular engine operation being tested.

More than one test may be run if all parameters cannot be held at the 100

percent level simultaneously.

(4) The runs must be made using fuel, lubricants and hydraulic fluid which

conform to the specifications specified in complying with Sec. 33.7(c).

(5) Maximum air bleed for engine and aircraft services must be used during

at least one-fifth of the runs. However, for these runs, the power or thrust

or the rotor shaft rotational speed may be less than 100 percent of the value

associated with the particular operation being tested if the Administrator

finds that the validity of the endurance test is not compromised.

(6) Each accessory drive and mounting attachment must be loaded. The load

imposed by each accessory used only for aircraft service must be the limit

load specified by the applicant for the engine drive and attachment point

during rated maximum continuous power or thrust and higher output. The

endurance test of any accessory drive and mounting attachment under load may

be accomplished on a separate rig if the validity of the test is confirmed by

an approved analysis.

(7) During the runs at any rated power or thrust the gas temperature and

the oil inlet temperature must be maintained at the limiting temperature

except where the test periods are not longer than 5 minutes and do not allow

stabilization. At least one run must be made with fuel, oil, and hydraulic

fluid at the minimum pressure limit and at least one run must be made with

fuel, oil, and hydraulic fluid at the maximum pressure limit with fluid

temperature reduced as necessary to allow maximum pressure to be attained.

(8) If the number of occurrences of either transient rotor shaft overspeed

or transient gas overtemperature is limited, that number of the accelerations

required by paragraphs (b), (c), (d), and (e) of this section must be made at

the limiting overspeed or overtemperature. If the number of occurrences is

not limited, half the required accelerations must be made at the limiting

overspeed or overtemperature.

(9) For each engine type certificated for use on supersonic aircraft the

following additional test requirements apply:

(i) To change the thrust setting, the power control lever must be moved

from the initial position to the final position in not more than one second

except for movements into the fuel burning thrust augmentor augmentation

position if additional time to confirm ignition is necessary.

(ii) During the runs at any rated augmented thrust the hydraulic fluid

temperature must be maintained at the limiting temperature except where the

test periods are not long enough to allow stabilization.

(iii) During the simulated supersonic runs the fuel temperature and

induction air temperature may not be less than the limiting temperature.

(iv) The endurance test must be conducted with the fuel burning thrust

augmentor installed, with the primary and secondary exhaust nozzles

installed, and with the variable area exhaust nozzles operated during each

run according to the methods specified in complying with Sec. 33.5(b).

(v) During the runs at thrust settings for maximum continuous thrust and

percentages thereof, the engine must be operated with the inlet air

distortion at the limit for those thrust settings.

(b) Engines other than certain rotorcraft engines. For each engine except a

rotorcraft engine for which a rating is desired under paragraph (c), (d), or

(e) of this section, the applicant must conduct the following runs:

(1) Takeoff and idling. One hour of alternate five-minute periods at rated

takeoff power and thrust and at idling power and thrust. The developed powers

and thrusts at takeoff and idling conditions and their corresponding rotor

speed and gas temperature conditions must be as established by the power

control in accordance with the schedule established by the manufacturer. The

applicant may, during any one period, manually control the rotor speed,

power, and thrust while taking data to check performance. For engines with

augmented takeoff power ratings that involve increases in turbine inlet

temperature, rotor speed, or shaft power, this period of running at takeoff

must be at the augmented rating. For engines with augmented takeoff power

ratings that do not materially increase operating severity, the amount of

running conducted at the augmented rating is determined by the Administrator.

In changing the power setting after each period, the power-control lever must

be moved in the manner prescribed in paragraph (b)(5) of this section.

(2) Rated maximum continuous and takeoff power and thrust. Thirty minutes

at--

(i) Rated maximum continuous power and thrust during fifteen of the twenty-

five 6-hour endurance test cycles; and

(ii) Rated takeoff power and thrust during ten of the twenty-five 6-hour

endurance test cycles.

(3) Rated maximum continuous power and thrust. One hour and 30 minutes at

rated maximum continuous power and thrust.

(4) Incremental cruise power and thrust. Two hours and 30 minutes at the

successive power lever positions corresponding to at least 15 approximately

equal speed and time increments between maximum continuous engine rotational

speed and ground or minimum idle rotational speed. For engines operating at

constant speed, the thrust and power may be varied in place of speed. If

there is significant peak vibration anywhere between ground idle and maximum

continuous conditions, the number of increments chosen may be changed to

increase the amount of running made while subject to the peak vibrations up

to not more than 50 percent of the total time spent in incremental running.

(5) Acceleration and deceleration runs. 30 minutes of accelerations and

decelerations, consisting of six cycles from idling power and thrust to rated

takeoff power and thrust and maintained at the takeoff power lever position

for 30 seconds and at the idling power lever position for approximately four

and one-half minutes. In complying with this paragraph, the power-control

lever must be moved from one extreme poition to the other in not more than

one second, except that, if different regimes of control operations are

incorporated necessitating scheduling of the power-control lever motion in

going from one extreme position to the other, a longer period of time is

acceptable, but not more than two seconds.

(6) Starts. One hundred starts must be made, of which 25 starts must be

preceded by at least a two-hour engine shutdown. There must be at least 10

false engine starts, pausing for the applicant's specified minimum fuel

drainage time, before attempting a normal start. There must be at least 10

normal restarts with not longer than 15 minutes since engine shutdown. The

remaining starts may be made after completing the 150 hours of endurance

testing.

(c) Rotorcraft engines for which a 30-minute OEI power rating is desired.

For each rotorcraft engine for which a 30-minute OEI power rating is desired,

the applicant must conduct the following series of tests:

(1) Takeoff and idling. One hour of alternate 5-minute periods at rated

takeoff power and at idling power. The developed powers at takeoff and idling

conditions and their corresponding rotor speed and gas temperature conditions

must be as established by the power control in accordance with the schedule

established by the manufacturer. During any one period, the rotor speed and

power may be controlled manually while taking data to check performance. For

engines with augmented takeoff power ratings that involve increases in

turbine inlet temperature, rotor speed, or shaft power, this period of

running at rated takeoff power must be at the augmented power rating. In

changing the power setting after each period, the power control lever must be

moved in the manner prescribed in paragraph (c)(5) of this section.

(2) Rated 30-minute OEI power. Thirty minutes at rated 30-minute OEI power.

(3) Rated maximum continuous power. Two hours at rated maximum continuous

power.

(4) Incremental cruise power. Two hours at the successive power lever

positions corresponding with not less than 12 approximately equal speed and

time increments between maximum continuous engine rotational speed and ground

or minimum idle rotational speed. For engines operating at constant speed,

power may be varied in place of speed. If there are significant peak

vibrations anywhere between ground idle and maximum continuous conditions,

the number of increments chosen must be changed to increase the amount of

running conducted while being subjected to the peak vibrations up to not more

than 50 percent of the total time spent in incremental running.

(5) Acceleration and deceleration runs. Thirty minutes of accelerations and

decelerations, consisting of six cycles from idling power to rated takeoff

power and maintained at the takeoff power lever position for 30 seconds and

at the idling power lever position for approximately 4 1/2  minutes. In

complying with this paragraph, the power control lever must be moved from one

extreme position to the other in not more than 1 second, except that if

different regimes of control operations are incorporated necessitating

scheduling of the power control lever motion in going from one extreme

position to the other, a longer period of time is acceptable, but not more

than 2 seconds.

(6) Starts. One hundred starts, of which 25 starts must be preceded by at

least a two-hour engine shutdown. There must be at least 10 false engine

starts, pausing for the applicant's specified minimum fuel drainage time,

before attempting a normal start. There must be at least 10 normal restarts

with not longer than 15 minutes since engine shutdown. The remaining starts

may be made after completing the 150 hours of endurance testing.

(d) Rotorcraft engines for which a continuous OEI rating is desired. For

each rotorcraft engine for which a continuous OEI power rating is desired,

the applicant must conduct the following series of tests:

(1) Takeoff and idling. One hour of alternate 5-minute periods at rated

takeoff power and at idling power. The developed powers at takeoff and idling

conditions and their corresponding rotor speed and gas temperature conditions

must be as established by the power control in accordance with the schedule

established by the manufacturer. During any one period the rotor speed and

power may be controlled manually while taking data to check performance. For

engines with augmented takeoff power ratings that involve increases in

turbine inlet temperature, rotor speed, or shaft power, this period of

running at rated takeoff power must be at the augmented power rating. In

changing the power setting after each period, the power control lever must be

moved in the manner prescribed in paragraph (c)(5) of this section.

(2) Rated maximum continuous and takeoff power. Thirty minutes at--

(i) Rated maximum continuous power during fifteen of the twenty-five 6-hour

endurance test cycles; and

(ii) Rated takeoff power during ten of the twenty-five 6-hour endurance

test cycles.

(3) Rated continuous OEI power. One hour at rated continuous OEI power.

(4) Rated maximum continuous power. One hour at rated maximum continuous

power.

(5) Incremental cruise power. Two hours at the successive power lever

positions corresponding with not less than 12 approximately equal speed and

time increments between maximum continuous engine rotational speed and ground

or minimum idle rotational speed. For engines operating at constant speed,

power may be varied in place of speed. If there are significant peak

vibrations anywhere between ground idle and maximum continuous conditions,

the number of increments chosen must be changed to increase the amount of

running conducted while being subjected to the peak vibrations up to not more

than 50 percent of the total time spent in incremental running.

(6) Acceleration and deceleration runs. Thirty minutes of accelerations and

decelerations, consisting of six cycles from idling power to rated takeoff

power and maintained at the takeoff power lever position for 30 seconds and

at the idling power lever position for approximately 4 1/2  minutes. In

complying with this paragraph, the power control lever must be moved from one

extreme position to the other in not more than 1 second, except that if

different regimes of control operations are incorporated necessitating

scheduling of the power control lever motion in going from one extreme

position to the other, a longer period of time is acceptable, but not more

than 2 seconds.

(7) Starts. One hundred starts, of which 25 starts must be preceded by at

least a 2-hour engine shutdown. There must be at least 10 false engine

starts, pausing for the applicant's specified minimum fuel drainage time,

before attempting a normal start. There must be at least 10 normal restarts

with not longer than 15 minutes since engine shutdown. The remaining starts

may be made after completing the 150 hours of endurance testing.

(e) Rotorcraft engines for which a 2 1/2 -minute OEI power rating is

desired. For each rotorcraft engine for which a 2 1/2 -minute OEI power

rating is desired, the applicant must conduct the following series of tests:

(1) Takeoff, 2 1/2 -minute OEI, and idling. One hour of alternate 5-minute

periods at rated takeoff power and at idling power except that, during the

third and sixth takeoff power periods, only 2 1/2  minutes need be conducted

at rated takeoff power, and the remaining 2 1/2  minutes must be conducted at

rated 2 1/2 -minute OEI power. The developed powers at takeoff, 2 1/2 -minute

OEI, and idling conditions and their corresponding rotor speed and gas

temperature conditions must be as established by the power control in

accordance with the schedule established by the manufacturer. The applicant

may, during any one period, control manually the rotor speed and power while

taking data to check performance. For engines with augmented takeoff power

ratings that involve increases in turbine inlet temperature, rotor speed, or

shaft power, this period of running at rated takeoff power must be at the

augmented rating. In changing the power setting after or during each period,

the power control lever must be moved in the manner prescribed in paragraph

(d)(6) of this section.

(2) The tests required in paragraphs (b)(2) through (b)(6), or (c)(2)

through (c)(6), or (d)(2) through (d)(7) of this section, as applicable,

except that in one of the 6-hour test sequences, the last 5 minutes of the 30

minutes at takeoff power test period of paragraph (b)(2) of this section, or

of the 30 minutes at 30-minute OEI power test period of paragraph (c)(2) of

this section, or of the l hour at continuous OEI power test period of

paragraph (d)(3) of this section, must be run at 2 1/2 -minute OEI power.

(f) Supersonic aircraft engines. For each engine type certificated for use

on supersonic aircraft the applicant must conduct the following:

(1) Subsonic test under sea level ambient atmospheric conditions. Thirty

runs of one hour each must be made, consisting of--

(i) Two periods of 5 minutes at rated takeoff augmented thrust each

followed by 5 minutes at idle thrust;

(ii) One period of 5 minutes at rated takeoff thrust followed by 5 minutes

at not more than 15 percent of rated takeoff thrust;

(iii) One period of 10 minutes at rated takeoff augmented thrust followed

by 2 minutes at idle thrust, except that if rated maximum continuous

augmented thrust is lower than rated takeoff augmented thrust, 5 of the 10-

minute periods must be at rated maximum continuous augmented thrust; and

(iv) Six periods of 1 minute at rated takeoff augmented thrust each

followed by 2 minutes, including acceleration and deceleration time, at idle

thrust.

(2) Simulated supersonic test. Each run of the simulated supersonic test

must be preceded by changing the inlet air temperature and pressure from that

attained at subsonic condition to the temperature and pressure attained at

supersonic velocity, and must be followed by a return to the temperature

attained at subsonic condition. Thirty runs of 4 hours each must be made,

consisting of--

(i) One period of 30 minutes at the thrust obtained with the power control

lever set at the position for rated maximum continuous augmented thrust

followed by 10 minutes at the thrust obtained with the power control lever

set at the position for 90 percent of rated maximum continuous augmented

thrust. The end of this period in the first five runs must be made with the

induction air temperature at the limiting condition of transient

overtemperature, but need not be repeated during the periods specified in

paragraphs (e)(2) (ii) through (iv) of this section;

(ii) One period repeating the run specified in paragraph (e)(2)(i) of this

section, except that it must be followed by 10 minutes at the thrust obtained

with the power control lever set at the position for 80 percent of rated

maximum continuous augmented thrust;

(iii) One period repeating the run specified in paragraph (e)(2)(i) of this

section, except that it must be followed by 10 minutes at the thrust obtained

with the power control lever set at the position for 60 percent of rated

maximum continuous augmented thrust and then 10 minutes at not more than 15

percent of rated takeoff thrust;

(iv) One period repeating the runs specified in paragraphs (e)(2) (i) and

(ii) of this section; and

(v) One period of 30 minutes with 25 of the runs made at the thrust

obtained with the power control lever set at the position for rated maximum

continuous augmented thrust, each followed by idle thrust and with the

remaining 5 runs at the thrust obtained with the power control lever set at

the position for rated maximum continuous augmented thrust for 25 minutes

each, followed by subsonic operation at not more than 15 percent or rated

takeoff thrust and accelerated to rated takeoff thrust for 5 minutes using

hot fuel.

(3) Starts. One hundred starts must be made, of which 25 starts must be

preceded by an engine shutdown of at least 2 hours. There must be at least 10

false engine starts, pausing for the applicant's specified minimum fuel

drainage time before attempting a normal start. At least 10 starts must be

normal restarts, each made no later than 15 minutes after engine shutdown.

The starts may be made at any time, including the period of endurance

testing.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-3, 32 FR

3737, Mar. 4, 1967; Amdt. 33-6, 39 FR 35468, Oct. 1, 1974; Amdt. 33-10, 49 FR

6853, Feb. 23, 1984; Amdt. 33-12, 53 FR 34220, Sept. 2, 1988]

Sec. 33.88  Engine overtemperature test.

Each engine must be run for 5 minutes at maximum permissible r.p.m with the

gas temperature at least 75 deg.F (42 deg.C) higher than the maximum

operating limit. Following this run, the turbine assembly must be within

serviceable limits.

[Amdt. 33-10, 49 FR 6853, Feb. 23, 1984]

Sec. 33.89  Operation test.

(a) The operation test must include testing found necessary by the

Administrator to demonstrate--

(1) Starting, idling, acceleration, overspeeding, ignition, functioning of

the propeller (if the engine is designated to operate with a propeller);

(2) Compliance with the engine response requirements of Sec. 33.73; and

(3) The minimum power or thrust response time to 95 percent rated takeoff

power or thrust, from power lever positions representative of minimum idle

and of minimum flight idle, starting from stabilized idle operation, under

the following engine load conditions:

(i) No bleed air and power extraction for aircraft use.

(ii) Maximum allowable bleed air and power extraction for aircraft use.

(iii) An intermediate value for bleed air and power extraction

representative of that which might be used as a maximum for aircraft during

approach to a landing.

(4) If testing facilities are not available, the determination of power

extraction required in paragraph (a)(3) (ii) and (iii) of this section may be

accomplished through appropriate analytical means.

(b) The operation test must include all testing found necessary by the

Administrator to demonstrate that the engine has safe operating

characteristics throughout its specified operating envelope.

[Amdt. 33-4, 36 FR 5493, Mar. 24, 1971, as amended by Amdt. 33-6, 39 FR

35469, Oct. 1, 1974; Amdt. 33-10, 49 FR 6853, Feb. 23, 1984]

Sec. 33.90  Initial maintenance inspection.

Each engine, except engines being type certificated through amendment of an

existing type certificate or through supplemental type certification

procedures, must undergo an approved test run that simulates the conditions

in which the engine is expected to operate in service, including typical

start-stop cycles, to establish when the initial maintenance inspection is

required. The test run must be accomplished on an engine which substantially

conforms to the final type design.

[Amdt. 33-10, 49 FR 6854, Feb. 23, 1984]

Sec. 33.91  Engine component tests.

(a) For those systems that cannot be adequately substantiated by endurance

testing in accordance with the provisions of Sec. 33.87, additional tests

must be made to establish that components are able to function reliably in

all normally anticipated flight and atmospheric conditions.

(b) Temperature limits must be established for those components that

require temperature controlling provisions in the aircraft installation to

assure satisfactory functioning, reliability, and durability.

(c) Each unpressurized hydraulic fluid tank may not fail or leak when

subjected to maximum operating temperature and an internal pressure of 5

p.s.i., and each pressurized hydraulic fluid tank may not fail or leak when

subjected to maximum operating temperature and an internal pressure not less

than 5 p.s.i. plus the maximum operating pressure of the tank.

(d) For an engine type certificated for use in supersonic aircraft, the

systems, safety devices, and external components that may fail because of

operation at maximum and minimum operating temperatures must be identified

and tested at maximum and minimum operating temperatures and while

temperature and other operating conditions are cycled between maximum and

minimum operating values.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-6, 39 FR

35469, Oct. 1, 1974]

Sec. 33.92  Windmilling tests.

(a) For engines to be used in supersonic aircraft, unless means are

incorporated in the engine to stop rotation of the engine rotors when the

engine is shut down in flight, each engine rotor must either seize or be

capable of rotation for 3 hours at the limiting windmilling rotational r.p.m.

with no oil in the engine system, without the engine--

(1) Catching fire;

(2) Bursting (releasing hazardous uncontained fragments); or

(3) Generating loads greater than those ultimate loads specified in Sec.

33.23(a).

(b) A turbojet or turbofan engine incorporating means to stop rotation of

the engine rotors when the engine is shut down in flight must be subjected to

25 operations under the following conditions:

(1) Each engine must be shut down while operating at rated maximum

continuous thrust.

(2) For engines certificated for use on supersonic aircraft, the

temperature of the induction air and the external surfaces of the engine must

be held at the maximum limit during the tests required by this paragraph.

[Amdt. 33-6, 39 FR 35470, Oct. 1, 1974, as amended by Amdt. 33-10, 49 FR

6854, Feb. 23, 1984]

Sec. 33.93  Teardown inspection.

After completing the endurance test each engine must be completely

disassembled, and--

(a) Each component having an adjustment setting and a functioning

characteristic that can be established independent of installation on the

engine must retain each setting and functioning characteristic within the

limits that were established and recorded at the beginning of the test; and

(b) Each engine part must conform to the type design and be eligible for

incorporation into an engine for continued operation, in accordance with

information submitted in compliance with Sec. 33.4.

[Amdt. 33-6, 39 FR 35470, Oct. 1, 1974, as amended by Amdt. 33-9, 45 FR

60181, Sept. 11, 1980; Amdt. 33-10, 49 FR 6854, Feb. 23, 1984]

Sec. 33.94   Blade containment and rotor unbalance tests.

(a) Except as provided in paragraph (b) of this section, it must be

demonstrated by engine tests that the engine is capable of containing damage

without catching fire and without failure of its mounting attachments when

operated for at least 15 seconds, unless the resulting engine damage induces

a self shutdown, after each of the following events:

(1) Failure of the most critical compressor or fan blade while operating at

maximum permissible r.p.m. The blade failure must occur at the outermost

retention groove or, for integrally-bladed rotor discs, at least 80 percent

of the blade must fail.

(2) Failure of the most critical turbine blade while operating at maximum

permissible r.p.m. The blade failure must occur at the outermost retention

groove or, for integrally-bladed rotor discs, at least 80 percent of the

blade must fail. The most critical turbine blade must be determined by

considering turbine blade weight and the strength of the adjacent turbine

case at case temperatures and pressures associated with operation at maximum

permissible r.p.m.

(b) Analysis based on rig testing, component testing, or service experience

may be substitute for one of the engine tests prescribed in paragraphs (a)(1)

and (a)(2) of this section if--

(1) That test, of the two prescribed, produces the least rotor unbalance;

and

(2) The analysis is shown to be equivalent to the test.

(Secs. 313(a), 601, and 603, Federal Aviation Act of 1958 (49 U.S.C. 1354(a),

1421, and 1423); and 49 U.S.C. 106(g) Revised, Pub. L. 97-449, Jan. 12, 1983)

[Amdt. 33-10, 49 FR 6854, Feb. 23, 1984]

Sec. 33.95  Engine-propeller systems tests.

If the engine is designed to operate with a propeller, the following tests

must be made with a representative propeller installed by either including

the tests in the endurance run or otherwise performing them in a manner

acceptable to the Administrator:

(a) Feathering operation: 25 cycles.

(b) Negative torque and thrust system operation: 25 cycles from rated

maximum continuous power.

(c) Automatic decoupler operation: 25 cycles from rated maximum continuous

power (if repeated decoupling and recoupling in service is the intended

function of the device).

(d) Reverse thrust operation: 175 cycles from the flight-idle position to

full reverse and 25 cycles at rated maximum continuous power from full

forward to full reverse thrust. At the end of each cycle the propeller must

be operated in reverse pitch for a period of 30 seconds at the maximum

rotational speed and power specified by the applicant for reverse pitch

operation.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-3, 32 FR

3737, Mar. 4, 1967]

Sec. 33.96  Engine tests in auxiliary power unit (APU) mode.

If the engine is designed with a propeller brake which will allow the

propeller to be brought to a stop while the gas generator portion of the

engine remains in operation, and remain stopped during operation of the

engine as an auxiliary power unit ("APU mode"), in addition to the

requirements of Sec. 33.87, the applicant must conduct the following tests:

(a) Ground locking: A total of 45 hours with the propeller brake engaged in

a manner which clearly demonstrates its ability to function without adverse

effects on the complete engine while the engine is operating in the APU mode

under the maximum conditions of engine speed, torque, temperature, air bleed,

and power extraction as specified by the applicant.

(b) Dynamic braking: A total of 400 application-release cycles of brake

engagements must be made in a manner which clearly demonstrates its ability

to function without adverse effects on the complete engine under the maximum

conditions of engine acceleration/deceleration rate, speed, torque, and

temperature as specified by the applicant. The propeller must be stopped

prior to brake release.

(c) One hundred engine starts and stops with the propeller brake engaged.

(d) The tests required by paragraphs (a), (b), and (c) of this section must

be performed on the same engine, but this engine need not be the same engine

used for the tests required by Sec. 33.87.

(e) The tests required by paragraphs (a), (b), and (c) of this section must

be followed by engine disassembly to the extent necessary to show compliance

with the requirements of Sec. 33.93(a) and Sec. 33.93(b).

[Amdt. 33-11, 51 FR 10346, Mar. 25, 1986]

Sec. 33.97  Thrust reversers.

(a) If the engine incorporates a reverser, the endurance calibration,

operation, and vibration tests prescribed in this subpart must be run with

the reverser installed. In complying with this section, the power control

lever must be moved from one extreme position to the other in not more than

one second except, if regimes of control operations are incorporated

necessitating scheduling of the power-control lever motion in going from one

extreme position to the other, a longer period of time is acceptable but not

more than three seconds. In addition, the test prescribed in paragraph (b) of

this section must be made. This test may be scheduled as part of the

endurance run.

(b) 175 reversals must be made from flight-idle forward thrust to maximum

reverse thrust and 25 reversals must be made from rated takeoff thrust to

maximum reverse thrust. After each reversal the reverser must be operated at

full reverse thrust for a period of one minute, except that, in the case of a

reverser intended for use only as a braking means on the ground, the reverser

need only be operated at full reverse thrust for 30 seconds.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-3, 32 FR

3737, Mar. 4, 1967]

Sec. 33.99  General conduct of block tests.

(a) Each applicant may, in making a block test, use separate engines of

identical design and construction in the vibration, calibration, endurance,

and operation tests, except that, if a separate engine is used for the

endurance test it must be subjected to a calibration check before starting

the endurance test.

(b) Each applicant may service and make minor repairs to the engine during

the block tests in accordance with the service and maintenance instructions

submitted in compliance with Sec. 33.4. If the frequency of the service is

excessive, or the number of stops due to engine malfunction is excessive, or

a major repair, or replacement of a part is found necessary during the block

tests or as the result of findings from the teardown inspection, the engine

or its parts must be subjected to any additional tests the Administrator

finds necessary.

(c) Each applicant must furnish all testing facilities, including equipment

and competent personnel, to conduct the block tests.

[Doc. No. 3025, 29 FR 7453, June 10, 1964, as amended by Amdt. 33-6, 39 FR

35470, Oct. 1, 1974; Amdt. 33-9, 45 FR 60181, Sept. 11, 1980]

Appendix A--Instructions for Continued Airworthiness

A33.1  GENERAL

(a) This appendix specifies requirements for the preparation of

Instructions for Continued Airworthiness as required by Sec. 33.4.

(b) The Instructions for Continued Airworthiness for each engine must

include the Instructions for Continued Airworthiness for all engine parts. If

Instructions for Continued Airworthiness are not supplied by the engine part

manufacturer for an engine part, the Instructions for Continued Airworthiness

for the engine must include the information essential to the continued

airworthiness of the engine.

(c) The applicant must submit to the FAA a program to show how changes to

the Instructions for Continued Airworthiness made by the applicant or by the

manufacturers of engine parts will be distributed.

A33.2  FORMAT

(a) The Instructions for Continued Airworthiness must be in the form of a

manual or manuals as appropriate for the quantity of data to be provided.

(b) The format of the manual or manuals must provide for a practical

arrangement.

A33.3  CONTENT

The contents of the manual or manuals must be prepared in the English

language. The Instructions for Continued Airworthiness must contain the

following manuals or sections, as appropriate, and information:

(a) Engine Maintenance Manual or Section. (1) Introduction information that

includes an explanation of the engine's features and data to the extent

necessary for maintenance or preventive maintenance.

(2) A detailed description of the engine and its components, systems, and

installations.

(3) Installation instructions, including proper procedures for uncrating,

deinhibiting, acceptance checking, lifting, and attaching accessories, with

any necessary checks.

(4) Basic control and operating information describing how the engine

components, systems, and installations operate, and information describing

the methods of starting, running, testing, and stopping the engine and its

parts including any special procedures and limitations that apply.

(5) Servicing information that covers details regarding servicing points,

capacities of tanks, reservoirs, types of fluids to be used, pressures

applicable to the various systems, locations of lubrication points,

lubricants to be used, and equipment required for servicing.

(6) Scheduling information for each part of the engine that provides the

recommended periods at which it should be cleaned, inspected, adjusted,

tested, and lubricated, and the degree of inspection the applicable wear

tolerances, and work recommended at these periods. However, the applicant may

refer to an accessory, instrument, or equipment manufacturer as the source of

this information if the applicant shows that the item has an exceptionally

high degree of complexity requiring specialized maintenance techniques, test

equipment, or expertise. The recommended overhaul periods and necessary cross

references to the Airworthiness Limitations section of the manual must also

be included. In addition, the applicant must include an inspection program

that includes the frequency and extent of the inspections necessary to

provide for the continued airworthiness of the engine.

(7) Troubleshooting information describing probable malfunctions, how to

recognize those malfunctions, and the remedial action for those malfunctions.

(8) Information describing the order and method of removing the engine and

its parts and replacing parts, with any necessary precautions to be taken.

Instructions for proper ground handling, crating, and shipping must also be

included.

(9) A list of the tools and equipment necessary for maintenance and

directions as to their method of use.

(b) Engine Overhaul Manual or Section. (1) Disassembly information

including the order and method of disassembly for overhaul.

(2) Cleaning and inspection instructions that cover the materials and

apparatus to be used and methods and precautions to be taken during overhaul.

Methods of overhaul inspection must also be included.

(3) Details of all fits and clearances relevant to overhaul.

(4) Details of repair methods for worn or otherwise substandard parts and

components along with the information necessary to determine when replacement

is necessary.

(5) The order and method of assembly at overhaul.

(6) Instructions for testing after overhaul.

(7) Instructions for storage preparation, including any storage limits.

(8) A list of tools needed for overhaul.

A33.4  AIRWORTHINESS LIMITATIONS SECTION

The Instructions for Continued Airworthiness must contain a section titled

Airworthiness Limitations that is segregated and clearly distinguishable from

the rest of the document. This section must set forth each mandatory

replacement time, inspection interval, and related procedure required for

type certification. If the Instructions for Continued Airworthiness consist

of multiple documents, the section required by this paragraph must be

included in the principal manual. This section must contain a legible

statement in a prominent location that reads: "The Airworthiness Limitations

section is FAA approved and specifies maintenance required under Secs. 43.16

and 91.403 of the Federal Aviation Regulations unless an alternative program

has been FAA approved."

[Amdt. 33-9, 45 FR 60181, Sept. 11, 1980, as amended by Amdt. 33-13, 54 FR

34330, Aug. 18, 1989]

Effective Date Note At 54 FR 34330, Aug. 18, 1989, Sec. A33.4 in Appendix

A, Part 33 was amended by changing the cross reference "Sec. 91.163" to "Sec.

91.403", effective August 18, 1990.